Keywords

Hydrodynamics, infragravity waves, run-up, overtopping, hydrostatic, surfbeat, non-hydrostatic, morphodynamics, dune erosion, overwash, breaching, coral reefs, vegetation, ships

Organizations

Deltares, UNESCO-IHE Institute of Water Education and Delft University of Technology

Authors

Dano Roelvink, Ap van Dongeren, Robert McCall, Bas Hoonhout, Arnold van Rooijen, Pieter van Geer, Lodewijk de Vet, Kees Nederhoff and many others

State final

Contents

1	Introduction	4

1.1	Readers guide	5

1.2	Manual version and revisions	5

2	Processes and model formulation	6

2.1	Domain and definitions	6

2.1.1	Coordinate system	6

2.1.2	Grid set-up	6

2.2	Hydrodynamics options	7

2.2.1	Stationary mode	8

2.2.2	Surf beat mode (instationary)	9

2.2.3	Non-hydrostatic mode (wave resolving)	11

2.3	Short wave action	13

2.3.1	Short wave action balance	13

2.3.2	Dissipation	14

2.3.3	Radiation stresses	18

2.3.4	Wave shape	18

2.3.5	Turbulence	20

2.3.6	Roller energy balance	21

2.4	Shallow water equations	23

2.4.1	Horizontal viscosity	23

2.4.2	Bed shear stress	24

2.4.3	Damping by vegetation	25

2.4.4	Wind	26

2.5	Non-hydrostatic pressure correction	26

2.6	Groundwater flow	27

2.6.1	Continuity	27

2.6.2	Equation of motions	28

2.7	Sediment transport	32

2.7.1	Advection-diffusion equation	32

2.7.2	General parameters	32

2.7.3	Transport formulations	33

2.7.4	Effects of wave nonlinearity	36

2.7.5	Hindered erosion by dilatancy	36

2.7.6	Bed slope effect	37

2.8	Bottom updating	38

2.8.1	Due to sediment fluxes	38

2.8.2	Avalanching	39

2.8.3	Bed composition	39

2.9	Ship-induced wave motions	41

3	Boundary conditions	43

3.1	Waves	43

3.1.1	Spectral conditions	43

3.1.2	Non-spectral conditions	43

3.1.3	Special conditions	44

3.1.4	Lateral boundary conditions	44

3.2	Shallow water equations	45

3.2.1	Offshore boundary	45

3.2.2	Lateral boundaries	46

3.2.3	Time varying water level	47

3.2.4	River and point discharge	47

3.3	Sediment transport	47

4	Input description	49

4.1	General	49

4.2	Physical processes	50

4.3	Grid and bathymetry	51

4.4	Waves input	54

4.4.1	Spectral wave boundary conditions	57

4.4.2	Non-spectral wave boundary conditions	64

4.4.3	Special types of wave boundary conditions	66

4.4.4	Temporally and/or spatially varying wave boundary conditions	66

4.5	Flow, tide and surge input	67

4.5.1	Flow boundary conditions	67

4.5.2	Time-varying water level	69

4.6	Water level (dam break)	70

4.7	Wind input	71

4.8	Sediment input	72

4.9	Vegetation input	74

4.10	Discharge input	75

4.11	Drifters input	77

4.12	Ship-induced wave motions	77

4.13	Output selection	79

4.13.1	Output types	81

4.13.2	Output times	89

4.13.3	Output format	90

4.14	Time parameters	92

5	Bibliography	93

Appendices

A	Hands on exercises	99

A.1	Dune erosion at Delfland, Netherlands (1D)	99

A.2	Nourishment scenarios near Kijkduin, Holland (1D)	100

A.3	Overwash at Santa Rosa Island , USA (2DH)	101

A.4	Yanchep perched beach and natural breakwater (2DH)	101

B	Advanced model coefficients	103

B.1	Wave numerics	103

B.2	Wave dissipation	103

B.3	Rollers	105

B.4	Wave-current interaction	105

B.5	Bed friction and viscosity	105

B.6	Flow numerics	106

B.7	Sediment transport	107

B.8	Sediment transport numerics	111

B.9	Quasi-3D sediment transport	111

B.10	Morphology	112

B.11	Bed update	113

B.12	Groundwater flow	115

B.13	Non-hydrostatic correction	117

B.14	Physical constants	118

B.15	Coriolis force	118

B.16	MPI	119

B.17	Output projection	119

C	Numerical implementation	121

C.1	Grid set-up		121

C.2	Wave action balance	121

C.2.1	Surfbeat solver	121

C.2.2	Stationary solver	122

C.3	Shallow water equations	123

C.3.1	Mass balance equation	123

C.3.2	Momentum balance equation	123

C.3.3	Time integration scheme	125

C.3.4	Groundwater flow	125

C.4	Sediment transport	130

C.5	Bottom updating schemes	131

C.5.1	Avalanching	132

C.5.2	Bed composition	132

C.6	Boundary conditions	134

C.7	Non-hydrostatic	135

C.7.1	Global continuity equation	135

C.7.2	Local continuity equation	136

C.7.3	Horizontal Momentum	136

C.7.4	Vertical momentum equations	139

# Introduction

XBeach is an open-source numerical model which is originally developed to simulate hydrodynamic and morphodynamic processes and impacts on sandy coasts with a domain size of kilometers and on the time scale of storms. Since then, the model has been applied to other types of coasts and purposes.

The model includes the hydrodynamic processes of short wave transformation (refraction, shoaling and breaking), long wave (infragravity wave) transformation (generation, propagation and dissipation), wave-induced setup and unsteady currents, as well as overwash and inundation. The morphodynamic processes include bed load and suspended sediment transport, dune face avalanching, bed update and breaching. Effects of vegetation and of hard structures have been included. The model has been validated with a series of analytical, laboratory and field test cases using a standard set of parameter settings.

XBeach has two modes: a hydrostatic and a non-hydrostatic mode. In the hydrostatic mode, the short wave amplitude variation is solved separately from the long waves, currents and morphological change. This saves considerable computational time, with the expense that the phase of the short waves is not simulated. A more complete model is the non-hydrostatic model which solves all processes including short wave motions, but with more computational demand.

The original application (surfbeat mode), funded by the U.S. Corps of Engineers in the framework of the Morphos project and the U.S. Geological Survey, was to be able to assess hurricane impacts on sandy beaches. Since then with funding from the Dutch Public Works Department, the model has been extended, applied and validated for storm impacts on dune and urbanized coasts for the purpose of dune safety assessments. With support from the European Commission XBeach has been validated on a number of dissipative and reflective beaches bordering all regional seas in the EU.

Beyond sandy coasts, the model has been applied to coral fringing and atoll reefs, in cooperation with and with funding by the University of Western Australia, the USGS and the Asian Development Bank. The model now also includes vegetative damping effects, with support of the U.S. Office of Naval Research.

The non-hydrostatic model has been developed initially by the TU Delft (as a prototype version of the SWASH (Zijlema et al. 2011) model). For the purpose of simulating the morphodynamic processes on gravel beaches, the model was extended and validated with support from the University of Plymouth. In this mode, ship-induced waves can be simulated as well, demonstrating the flight that the model has taken since its first inception.

This development of XBeach could not have been possible without all of the above mentioned funding agencies and partners. It would also not have been possible without the enthusiastic, critical and constructive approach of all consultants, researchers, M.Sc. and Ph.D. students who have taken up XBeach, and made it into the tool that it is today.

This manual serves as an introduction to the model and a reference guide to its many functionalities, options and parameters. We sincerely hope that this document will help existing and new researchers apply the model for their purposes and advance our knowledge of coastal hydro- and morphodynamics.

## Readers guide

To make this manual more accessible we will briefly describe the contents of each chapter and the appendix. If this is your first time to start working with XBeach we suggest you to read Section 4.1 which gives an overview which settings are needed to run an XBeach simulation and to practice with the hands-on exercises in Appendix A.

Chapter 2: Processes and model formulations. This Chapter describes the theoretical physics modeled in XBeach. This includes short wave propagation, flow, sediment transport and bed level change.

Chapter 3: Boundary Conditions. Several possibilities to impose information at the offshore and lateral boundary are implemented in XBeach (eg. wave energy, water levels).

Chapter 4: Input description. XBeach works by reading a configuration file called params.txt. In this Chapter all the standard keywords for this configuration file are treated. This includes keyword to switch on and off specific processes.

Chapter 5: References. Provides a list of publications and related material of XBeach.

Appendix A: Hands-on exercises. Several exercises for XBeach to get familiar with the numerical program based on basic course given during the Delft Software Days 2014.

Appendix B: Advanced model coefficients. In Chapter 4 the main input parameters and files required by XBeach to start a simulation are explained. In this chapter many more parameters to fine-tune the simulation of different processes are elaborated.

Appendix C: Numerical implementation. Discusses the numerical implementation of the processes and model formulation of Chapter 2.

## Manual version and revisions

A manual applies to a certain release of the related numerical program. This manual applies to XBeach version 1.22 (revision 4567) also known as the ‘King’s ‘day release.

# Processes and model formulation

## Domain and definitions

### Coordinate system

XBeach uses a coordinate system where the computational x-axis is always oriented towards the coast, approximately perpendicular to the coastline, and the y-axis is alongshore, see Figure A.1 and Figure A.1. This coordinate system is defined in world coordinates. The grid size in x- and y-direction may be variable but the grid must be curvilinear. Alternatively, in case of a rectangular grid (a special case of a curvilinear grid) the user can provide coordinates in a local coordinate system that is oriented with respect to world coordinates (xw, yw) through an origin (xori, yori) and an orientation (alfa) as depicted in Figure A.1. The orientation is defined counter-clockwise w.r.t. the xw-axis (East).

Figure 2.1	Rectangular coordinate system of XBeach

### Grid set-up

The grid applied is a staggered grid, where the bed levels, water levels, water depths and concentrations are defined in cell centers, and velocities and sediment transports are defined in u- and v-points, viz. at the cell interfaces. In the wave energy balance, the energy, roller energy and radiation stress are defined at the cell centers, whereas the radiation stress gradients are defined at u- and v-points.

Velocities at the u- and v-points are denoted by the output variables uu and vv respectively; velocities u and v at the cell centers are obtained by interpolation and are for output purpose only. The water level, zs, and the bed level, zb, are both defined positive upward. uv and vu are the u-velocity at the v-grid point and the v-velocity at the u-grid point respectively. These are obtained by interpolation of the values of the velocities at the four surrounding grid points.

The model solves coupled 2D horizontal equations for wave propagation, flow, sediment transport and bottom changes, for varying (spectral) wave and flow boundary conditions.

Figure 2.2	Curvilinear coordinate system of XBeach

## Hydrodynamics options

XBeach was originally developed as a short-wave averaged but wave-group resolving model, allowing resolving the short wave variations on the wave group scale and the long waves associated with them. Since the original paper by Roelvink et al. (2009) a number of additional model options have been implemented, thereby allowing users to choose which time-scales to resolve:

- Stationary wave model (keyword: wavemodel = stationary), efficiently solving wave-averaged equations but neglecting infragravity waves;

- Surfbeat mode (instationary) (keyword: wavemodel = surfbeat), where the short wave variations on the wave group scale (short wave envelope) and the long waves associated with them are resolved;

- Non-hydrostatic mode (wave-resolving) (keyword: wavemodel = nonh), where a combination of the non-linear shallow water equations with a pressure correction term is applied, allowing to model the propagation and decay of individual waves.

In the following these options are discussed in more detail. Important to note that all times in XBeach are prescribed on input in morphological time. If you apply a morphological acceleration factor (keyword: morfac) all input time series and other time parameters are divided internally by morfac. This way, you can specify the time series as real times, and vary the morfac without changing the rest of the input files (keyword: morfacopt = 1).

Figure 2.3	Principle sketch of the relevant wave processes

### Stationary mode

In stationary mode the wave-group variations and thereby all infragravity motions are neglected. This is useful for conditions where the incident waves are relatively small and/or short, and these motions would be small anyway. The model equations are similar to HISWA (Holthuijsen et al., 1989) but do not include wave growth or wave period variations. Processes that are resolved are wave propagation, directional spreading, shoaling, refraction, bottom dissipation and wave breaking, and a roller model is included; these processes are usually dominant in nearshore areas of limited extent. For the breaking dissipation we use the Baldock et al. (1998) model, which is valid for wave-averaged modeling. The radiation stress gradients from the wave and roller model force the shallow water equations, drive currents and lead to wave setdown and setup. Additionally, wind and tidal forcing can be applied.

The mean return flow due to mass flux and roller is included in the model and affects the sediment transport, leading to an offshore contribution. To balance this, effects of wave asymmetry and skewness are included as well. Bed slope effects can further modify the cross-shore behavior. A limited number of model coefficients allow the user to calibrate the profile shape resulting from these interactions.

A typical application would be to model morphological changes during moderate wave conditions, often in combination with tides. The wave boundary conditions can be specified as constant (keyword: wbctype = stat) or as a time-series of wave conditions (keyword: wbctype = stat_table). Typical examples of such model applications are given below for tombolo formation behind an offshore breakwater (left panel) and development of an ebb delta at a tidal inlet (right panel). A big advantage of the stationary XBeach wave model over other models is that the lateral boundaries are entirely without disturbance if the coast is longshore uniform near these boundaries.

Figure 2.4 Root-mean square wave height (left panels) and final bathymetry (right panels) for an offshore breakwater case (upper panels) and a tidal inlet with waves from 330 degrees (lower panels).

### Surf beat mode (instationary)

The short-wave motion is solved using the wave action equation which is a time-dependent forcing of the HISWA equations (Holthuijsen et al., 1989). This equation solves the variation of short-waves envelope (wave height) on the scale of wave groups. It employs a dissipation model for use with wave groups (Roelvink, 1993a; Daly et al., 2012) and a roller model (Svendsen, 1984; Nairn et al., 1990; Stive and de Vriend, 1994) to represent momentum stored at the surface after breaking. These variations, through radiation stress gradients (Longuet-Higgins and Stewart, 1962, 1964) exert a force on the water column and drive longer period waves (infragravity waves) and unsteady currents, which are solved by the nonlinear shallow water equations (e.g. Phillips, 1977). Thus, wave-driven currents (longshore current, rip currents and undertow), and wind-driven currents (stationary and uniform) for local wind set-up, long (infragravity) waves, and runup and rundown of long waves (swash) are included.

Using the surfbeat mode is necessary when the focus is on swash zone processes rather than time-averaged currents and setup. It is fully valid on dissipative beaches, where the short waves are mostly dissipated by the time they are near the shoreline. On intermediate beaches and during extreme events the swash motions are still predominantly in the infragravity band and so is the wave runup.

Under this surfbeat mode, several options are available, depending on the circumstances:

- 1D cross-shore; in this case the longshore gradients are ignored and the domain reduces to a single gridline (keyword: ny = 0). Within this mode the following options are available:

  - Retaining directional spreading (keyword: dtheta < thetamax – thetamin); this has a limited effect on the wave heights because of refraction, but can also allow obliquely incident waves and the resulting longshore currents;

  - Using a single directional bin (keyword: dtheta = thetamax – thetamin); this leads to perpendicular waves always and ignores refraction. If the keyword snells = 1 is applied, the mean wave direction is determined based on Snell's law. In this case also longshore currents are generated.

- 2DH area; the model is solved on a curvilinear staggered grid (rectilinear is a special case). The incoming short wave energy will vary along the seaward boundary and in time, depending on the wave boundary conditions. This variation is propagated into the model domain. Within this mode the following options are available:

  - Resolving the wave refraction 'on the fly' using the propagation in wave directional space. For large directional spreading or long distances this can lead to some smoothing of groupiness since the waves from different directions do not interfere but their energy is summed up. This option is possible for arbitrary bathymetry and any wave direction. The user must specify the width of the directional bins for the surfbeat mode (keyword: dtheta)

  - Solving the wave direction at regular intervals using the stationary solver, and then propagating the wave energy along the mean wave direction. This preserves the groupiness of the waves therefore leads to more forcing of the infragravity waves (keyword: single_dir = 1). The user must now specify a single directional bin for the instationary mode (dtheta = thetamax - thetaminn) and a smaller bin size for the stationary solver (keyword: dtheta_s).

  - For schematic, longshore uniform cases the mean wave direction can also be computed using Snell's law (keyword: snells = 1). This will then give comparable results to the single_dir option.

In the figures below some typical applications of 1D and 2D models are shown; a reproduction of a large-scale flume test, showing the ability of XBeach to model both short-wave (HF) and long-wave (LF) wave heights and velocities; and a recent 2DH simulation (Nederhoff et al., 2015) of the impact of hurricane Sandy on Camp Osborne, Brick, NJ.

Figure 2.5 Computed and observed hydrodynamic parameters for test 2E of the LIP11D experiment. Top left: bed level and mean water level. Top right: measured (dots) and computed

Figure 2.6 Pre (left) and post-Sandy (right) in a three dimensional plot with both bed and water levels as simulated by XBeach (Nederhoff et al. 2015)

### Non-hydrostatic mode (wave resolving)

For non-hydrostatic XBeach calculations (keyword: wavemodel = nonh) depth-averaged flow due to waves and currents are computed using the non-linear shallow water equations, including a non-hydrostatic pressure. The depth-averaged normalized dynamic pressure (q) is derived in a method similar to a one-layer version of the SWASH model (Zijlema et al. 2011). The depth averaged dynamic pressure is computed from the mean of the dynamic pressure at the surface and at the bed by assuming the dynamic pressure at the surface to be zero and a linear change over depth.

Under these formulations dispersive behavior is added to the long wave equations and the model can be used as a short-wave resolving model. Wave breaking is implemented by disabling the non-hydrostatic pressure term when waves exceed a certain steepness, after which the bore-like breaking implicit in the momentum-conserving shallow water equations takes over.

In case the non-hydrostatic mode is used, the short wave action balance is no longer required. However, in the wave-resolving mode we need much higher spatial resolution and associated smaller time steps, making this mode much more computationally expensive than the surfbeat mode.

The main advantages of the non-hydrostatic mode are that the incident-band (short wave) runup and overwashing are included, which is especially important on steep slopes such as gravel beaches. Another advantage is that the wave asymmetry and skewness are resolved by the model and no approximate local model or empirical formulation is required for these terms. Finally, in cases where diffraction is a dominant process, wave-resolving modeling is needed as it is neglected in the short wave averaged mode.

An application of the non-hydrostatic mode is XBeach-G, which is a branch of the main XBeach source code but is specifically developed to simulate storm impacts on gravel beaches (McCall et al, 2014). The formulations for gravel beaches are developed for the non-hydrostatic mode and although sandy morphology can be simulated using the wave-resolving mode, it has not been extensively validated and it is likely that changes in the sediment transport formulations will be implemented in the near future.

An interesting recent application that has been validated for a number of cases concerns the modeling of primary waves generated by large ships, see Section 2.9.

Figure 2.7 Measured (black) and modeled (red) time series of overtopping during BARDEX experiment, from McCall et al, 2014.

## Short wave action

### Short wave action balance

The wave forcing in the shallow water momentum equation is obtained from a time dependent version of the wave action balance equation. Similar to Delft University’s (stationary) HISWA model (Holthuijsen et al., 1989) the directional distribution of the action density is taken into account, whereas the frequency spectrum is represented by a frequency, best represented by the spectral parameter fm-1,0. The wave action balance (keyword: swave) is then given by:

(2.1)

In which the wave action A is calculated as:

(2.2)

where θ represents the angle of incidence with respect to the x-axis, Sw represents the wave energy density in each directional bin and σ the intrinsic wave frequency. The intrinsic frequency σ and group velocity cg is obtained from the linear dispersion relation. Dw, Df and Dv are dissipation terms for respectively waves, bottom friction and vegetation. The intrinsic frequency is for example obtained with:

(2.3)

The wave action propagation speeds in x, y and directional space are given by:

(2.4)

where h represents the local water depth and k the wave number. The intrinsic wave frequency σ is determined without wave current interaction (keyword: wci=1, see Section 2.3.1.1), which means it is equal to the absolute radial frequency ω.

#### Wave current interaction (wci)

Wave-current interaction is the interaction between waves and the mean flow. The interaction implies an exchange of energy, so after the start of the interaction both the waves and the mean flow are affected by each other. This feature is especially of importance in gullies and rip-currents (Reniers et al., 2007).

In XBeach this is taken into account by correcting the wave number k with the use of Eikonal equations, which will have impact on the group and wave propagation speed (x, y, and directional space). The cross-shore and alongshore wave numbers, kx and ky, are defined according to (2.5). In these formulations the subscripts refer to the direction of the wave vector components.

(2.5)

Where subscript n-1 refers the wave number of the previous time step,  and  are the wave number corrections and kx and ky are the corrected wave numbers that take into account the presence of a current. The correction terms are determined with a second set of equations, the Eikonal equations:

(2.6)

The wave number is then given by:

(2.7)

The absolute radial frequency ω is calculated with:

(2.8)

where uL and vL are the cross-shore and alongshore depth-averaged Lagrangian velocities respectively. The wave action propagation speed (in x and y direction) is given by:

(2.9)

The propagation speed in directional (θ) space, where bottom refraction (first term) and current refraction (last two terms) are taken into account, is obtained from:

(2.10)

### Dissipation

In XBeach there are three short wave dissipation processes that can be accounted for: wave breaking (Dw), bottom friction (Df) and vegetation (Dv). The three processes are explained in more detail in the following subsections.

#### Wave breaking

Five different wave breaking formulations are implemented in XBeach. The formulations can be selected using the keyword break (Table 2.1).

Table A.1	Different wave breaking formulations implemented

| Wave breaking formula | Type of waves | keyword |
| --- | --- | --- |
| Roelvink (1993a) | Instationary | roelvink1 |
| Roelvink (1993a) extended | Instationary | roelvink2 |
| Daly et al. (2010) | Instationary | roelvink_daly |
| Baldock et al. (1998) | Stationary | baldock |
| Janssen & Battjes (2007) | Stationary | janssen |

For the surf beat approach the total wave energy dissipation, i.e. directionally integrated, due to wave breaking can be modeled according to Roelvink (1993a, keyword: break=roelvink1). In the formulation of the dissipation due to wave breaking the idea is to calculate the dissipation with a fraction of breaking waves (Qb) multiplied by the dissipation per breaking event. In this formulation α is applied as wave dissipation coefficient of O(1) (keyword: alpha), Trep is the representative wave period and Ew is the energy of the wave. The fraction of wave breaking is determined with the root-mean-square wave height (Hrms) and the maximum wave height (Hmax). The maximum wave height is calculated as ratio of the water depth (h) plus a fraction of the wave height (δHrms, keyword: delta) using a breaker index γ (keyword: gamma). In the formulation for Hrms the ρ represents the water density and g the gravitational constant. The total wave energy Ew is calculated by integrating over the wave directional bins.

(2.11)

In variation of (2.11), one could also use another wave breaking formulation, presented in (2.12). This formulation is somewhat different than the formulation of Roelvink (1993a) and selected using keyword break=roelvink2. The main difference with the original formulation is that wave dissipation with break=roelvink2 is proportional to H3/h instead of H2.

(2.12)

Alternatively the formulation of Daly et al. (2010) states that waves are fully breaking if the wave height exceeds a threshold (γ) and stop breaking if the wave height fall below another threshold (γ2). This formulation is selected by break=roelvink_daly and the second threshold, γ2, can be set using keyword: gamma2.

(2.13)

In case of stationary waves Baldock et al. (1998) is applied (keyword: break=baldock), which is presented in (2.14). In this breaking formulation the fraction breaking waves Qb and breaking wave height Hb are calculated differently compared to the breaking formulations used for the non-stationary situation. In (2.14) α is applied as wave dissipation coefficient, frep represents a representative intrinsic frequency and y is a calibration factor.

(2.14)

Finally, it is possible to use the Janssen & Battjes (2007) formulation for wave breaking of stationary waves (keyword: break=janssen). This formulation is a revision of Baldock’s formulation.

(2.15)

In both the instationary or stationary case the total wave dissipation is distributed proportionally over the wave directions with the formulation in (2.16).

(2.16)

#### Bottom friction

The moment equations used to compute the mean currents, orbital velocities and surface elevations contain a friction term following (Ruessink et al, 2001; see Section 2.4.2) and are thus considered separately from the bottom friction that has an impact on the wave action balance. The short wave dissipation by bottom friction is especially of importance for coral reeds (eg. Van Dongeren et al, 2012, Quataert et al, 2015) and is modeled as

(2.17)

In (2.17) the fw (keyword: fw) is the short-wave friction coefficient and Tm01 is the mean period defined by the first and zeroth moments of the wave spectrum. This value only affects the wave action equation and is unrelated to bed friction in the flow equation. Studies conducted on reefs (e.g. Lowe et al., 2007) indicate that fw should be an order of magnitude (or more) larger than the friction coefficient for flow (cf) due to the dependency of wave frictional dissipation rates on the frequency of the motion.

The derivation of the short wave dissipation term is based time-averaged instantaneous bottom dissipation using the Johnson friction factor fw of the bed shear stress:

(2.18)

The evaluation of the term , the so-called third even velocity moment, depends on the situation. First we need expressions for the orbital velocity amplitude, which is expressed as:

(2.19)

In this formulation Tp is the peak wave period, Hrms is the root-mean-square wave height, k is the wave number and h is the local water depth.

If we consider the slowly-varying dissipation in wave groups, we need only to average over a single wave period and we can use a monochromatic (regular wave) expression. If we want to have the time-average dissipation over a full spectrum we get the best approximation from considering a linear Gaussian distribution. Guza and Thornton (1985) give pragmatic expressions for both cases.

For the monochromatic case:

(2.20)

For the linear Gaussian approximation:

(2.21)

Combining (2.18) and (2.12) we get:

(2.22)

In XBeach the orbital velocity amplitude is computed as in (2.19) end the dissipation according to (2.12) which is correct for the case of instationary simulations on wave-group scale.

For the stationary case formulations (2.19) and (2.21) are similarly combined into:

(2.23)

#### Vegetation

The presence of aquatic vegetation (keyword: vegetation) within the area of wave propagation or wave breaking results in an additional dissipation mechanism for short waves. This is modeled using the approach of Mendez & Losada (2004), which was adjusted by Suzuki et al., (2011) to take into account vertically heterogeneous vegetation, see Van Rooijen et al. (2015). The short wave dissipation due to vegetation is calculated as function of the local wave height and several vegetation parameters (keyword: veggiefile) at a specific number of locations (keyword: veggiemapfile). The vegetation can be schematized in a number of vertical elements with each specific property. In this way the wave damping effect of vegetation such as mangrove trees, with a relatively dense root system but sparse stem area, can be modeled. The dissipation term is then computed as the sum of the dissipation per vegetation layer (Suzuki et al, 2011):

(2.24)

where Dv,i is the dissipation by vegetation in vegetation layer i and nv is the number of vegetation layers. The dissipation per layer is given by:

(2.25)

where is a (bulk) drag coefficient, bv,i is the vegetation stem diameter, Nv,i is the vegetation density, and αi is the relative vegetation height (= hv / h) for layer i. In case only one vegetation layer is specified, the plants are assumed to be vertically uniform, which would for example typically apply in case of modeling sea grass.

### Radiation stresses

Given the spatial distribution of the wave action (and therefore wave energy) the radiation stresses can be evaluated by using linear wave theory as described by:

(2.26)

### Wave shape

The morphodynamic model considered is (short) wave averaged and resolves hydrodynamics associated with the wave group time scale. As a result the short wave shape is not solved for. However, as waves propagate from deep water onto beaches, their surface form and orbital water motion become increasingly non-linear because of the amplification of the higher harmonics.

There are two wave forms implemented to take this non-linearity into account:

- A formulation of Ruessink et al. (2012) based on a parameterization with the Ursell number. (keyword: waveform = ruessink_vanrijn)

- A formulation of Van Thiel de Vries (2009) based on the parameterized wave shape model of Rienecker and Fenton (1981) (keyword: waveform = vanthiel)

The formulation of Ruessink et al. (2012) relies on parameterizations for the non-linearity parameter r and phase Φ. The parameterizations are based on a data set of 30.000+ field observations of the orbital skewness Sk and asymmetry As, collected under non-breaking and breaking wave conditions. The only variable parameter is the Ursell number, since according to Ruessink et al. (2012) the Ursell that includes Hs, T and h, describes the variability in Sk and As well. The Ursell number is calculated with the equation below.

(2.27)

The value for the skewness and asymmetry is calculated with the use of a Boltzmann sigmoid. The skewness and asymmetry are a function of Ψ. In the formulation of Ruessink et al. (2012) the p1:6 are used as parameterized factors on the data set of field observations.

(2.28)

Alternatively, Van Thiel de Vries (2009) utilized and extended the wave shape model of Rienecker and Fenton (1981). In this model the short wave shape is described by the weighted sum of eight sine and cosine functions

(2.29)

where ubed is the near-bed short wave flow velocity, i refers to the ith harmonic, ω is the angular wave frequency, Ai is the amplitude of a specific harmonic and w is a weighting function affecting the wave shape. The amplitudes A1:8 are computed from stream function theory and vary with the dimensionless wave height and dimensionless wave period.

The wave skewness of near bed flow velocities is computed according to (2.30). The wave asymmetries (As) can be computed with the same expression replacing ubed by its Hilbert transform.

(2.30)

For w equals one a skewed (Stokes) wave is obtained with high peaks and flat troughs whereas w equals zero results in an asymmetric (saw tooth) wave with steep wave fronts. It is hypothesized that the weighting w can be expressed as a function of wave skewness and asymmetry. The relation between the phase and the weighting is studied in more detail by Van Thiel de Vries (2009) by varying w between zero and one in small steps and computing the amplitudes A1:8 with Rienecker and Fenton for a range of wave heights, wave periods and water depths. It is found that a unique relation between w and Φ exists for any combination of wave height, wave period and water depth that is described by:

(2.31)

As explained in the next section, short-wave turbulence can be computed averaged over the bore interval (Tbore). The bore interval is directly related to the wave shape and hence requires the weighting function w is determined. For the formulation of Ruessink et al. (2012) no exact wave shape is determined and therefore no bore interval can be calculated. Therefore this approach cannot be combined with bore averaged short-wave turbulence.

### Turbulence

Wave breaking induced turbulence at the water surface has to be transported towards the bed in order to affect the up-stirring of sediment. Roelvink and Stive (1989) used an exponential decay model with the mixing length proportional to Hrms to estimate the time averaged turbulence energy at the bed from turbulence at the water surface:

(2.32)

where kb is turbulence variance at the bed and k is the time averaged turbulence variance at the water surface.

There are three possibilities for the turbulence variance at the bed (kb) implemented into XBeach:

- Wave averaged near-bed turbulence energy (keyword: turb = wave_averaged):

(2.33)

- Bore-averaged near-bed turbulence energy (keyword: turb = bore_averaged)

(2.34)

- Not taking into account the turbulence variance at the bed (keyword: turb = none)

Both formulations make use of the wave-averaged turbulence energy (ks) and a mixing length (Lmix). The wave averaged turbulence energy at the surface is computed from the roller energy dissipation and following Battjes (1975) in which Dr is roller dissipation:

(2.35)

The mixing length (Lmix) is expressed as thickness of the surface roller near the water surface and depends on the roller volume Ar (Svendsen, 1984):

(2.36)

### Roller energy balance

While the short wave action balance adequately describes the propagation and decay of organized wave energy, it has often been found that there is a delay between the point where the waves start to break (which is where you would expect the strongest radiation stress gradients to occur) and the point where the wave set-up and longshore current start to build. This transition zone effect is generally attributed to the temporary storage of shoreward momentum in the surface rollers. Several authors have analyzed the typical dimensions of such rollers and their effect on the radiation stress (e.g. Longuet-Higgins and Turner, 1974, Svendsen, 1984, Roelvink and Stive, 1989, Nairn et al., 1990, Deigaard 1993, Stive and De Vriend, 1994).

The rollers can be represented as a blob of water with cross-sectional area A that slides down the front slope of a breaking wave. The roller exerts a shear stress on the water beneath it equal to:

(2.37)

where  is the slope of the breaking wave front, R is the roller area and L is the wave length. The roller has a kinetic energy equal to:

(2.38)

and a contribution to the radiation stress equal to:

(2.39)

We can now formulate an energy balance for the roller as follows:

(2.40)

where S is the loss of organized wave motion due to breaking and D is the dissipation.  The latter is equal to the work done by the shear stress between the roller and the wave:

(2.41)

Given the complex motion in the breaking waves, we can only give approximate estimates of the order of magnitude of the parameters in the (2.37) till (2.41). Various authors have suggested that the velocity in the roller can be approximated as purely horizontal and equal to the wave celerity cg. In that case we get (for waves travelling in x-direction):

(2.42)

However, this must be seen as a (unrealistic) upper limit on the radiation stress contribution as this can only be valid for wroller=0. Nairn et al. (1990) showed that the conceptual model of Roelvink and Stive (1989) would lead to a factor 0.22 instead of 2. However, a ratio in the order of 1 seems more realistic. Stive and De Vriend (1994) found a discrepancy between the roller shear stress derived from an energy balance and that derived from the momentum balance, in the order of a factor two. They explained this by a complicated analysis of the effect of water entering and leaving the roller, which led to a modification of the propagation term in the roller energy balance by a factor two. As this leads to the unphysical result that rollers would propagate at twice the wave celerity, we believe that the discrepancy must be sought in the ratio between roller energy and radiation stress contribution. Therefore we stick to the roller energy balance suggested by Nairn et al. (1990) in (2.43) and the roller contribution to the radiation stress:

(2.43)

This leads to an elegant and consistent distribution of the wave-induced forcing through the surfzone. To close the roller energy balance we need to express the dissipation of the roller as a function of Er. This can be done by introducing:

(2.44)

Combining this with (2.38) and (2.44) we then find:

(2.45)

The coefficients  and  are usually lumped together into a single coefficient. This coefficient β is in the O(0.1) (keyword: beta), which may vary through the surf zone. The forcing of the longshore current by the radiation stress gradient can be derived from the wave and roller energy balances (2.1) and (2.40):

(2.46)

In a longshore uniform situation, according to Snell’s law, the first term on the right-hand side equals zero; the second term exactly equals the sum of the wave energy dissipation and the roller energy input and dissipation terms, so the forcing term reduces to:

(2.47)

## Shallow water equations

For the low-frequency waves and mean flows we use the shallow water equations. To account for the wave induced mass-flux and the subsequent (return) flow these are cast into a depth-averaged Generalized Lagrangian Mean (GLM) formulation (Andrews and McIntyre, 1978, Walstra et al, 2000). In such a framework, the momentum and continuity equations are formulated in terms of the Lagrangian velocity uL which is defined as the distance a water particle travels in one wave period, divided by that period. This velocity is related to the Eulerian velocity (the short-wave-averaged velocity observed at a fixed point) by:

(2.48)

where uS and vS represent the Stokes drift in x- and y-direction respectively (Phillips, 1977). The Strokes drift is calculated with (2.49) in which the wave-group varying short wave energy Ew and direction are obtained from the wave-action balance.

(2.49)

The resulting GLM-momentum equations are given by:

(2.50)

where τsx and τsy are the wind shear stresses, τbx and τby are the bed shear stresses, η is the water level, Fx and Fy are the wave-induced stresses, Fv,x, and Fv,y are the stresses induced by vegetation, νh is the horizontal viscosity and f is the Coriolis coefficient. Note that the shear stress terms are calculated with the Eulerian velocities as experienced by the bed and not with the GLM velocities, as can be seen in (2.50).

### Horizontal viscosity

The horizontal viscosity (vh) is by default computed using the Smagorinsky (1963) model to account for the exchange of horizontal momentum at spatial scales smaller than the computational grid size, which is given as:

(2.51)

In (2.51) cS is the Smagorinsky constant (keyword: nuh), set at 0.1 in all model simulations. It is also possible to use a user-defined value for the horizontal viscosity by turning off the Smagorinsky model (keyword: smag = 0) and specifying the value directly (also keyword: nuh).

### Bed shear stress

The bed friction associated with mean currents and long waves is included via the formulation of the bed shear stress (τb). Using the approach of Ruessink et al. (2001) the bed shear stress is calculated with:

(2.52)

There are five different formulations in order to determine the dimensionless bed friction coefficient cf (keyword: bedfriction) implemented in XBeach (Table A.1).

Table A.1	Different bed friction formulations implemented

| Bed friction formulation | Relevant coefficient | keyword |
| --- | --- | --- |
| Dimensionless friction coefficient | cf | cf |
| Chézy | C | chezy |
| Manning | n | manning |
| White-Colebrook | ks | white-colebrook |
| White-Colebrook grain size | D90 | white-colebrook-grainsize |

The dimensionless friction coefficient can be calculated from the Chézy value with (2.53). A typical Chézy value for sandy coasts is in the order of 55 m1/2/s.

(2.53)

In the Manning formulation the Manning coefficient (n) must be specified. The dimensionless friction coefficient is calculated from (2.54). Manning can be seen as a depth-dependent Chézy value and a typical Manning value for sandy coasts would be in the order of 0.02 s/m1/3.

(2.54)

In the White-Colebrook formulation the geometrical roughness of Nikuradse (ks) must be specified. The dimensionless friction coefficient is calculated from (2.55) The White-Colebrook formulation has al log relation with the water depth and a typical ks value for sandy coasts would be in the order of 0.01 - 0.15 m.

(2.55)

The option of White-Colebrook based on the grain size is somewhat different than the other four formulations. This formulation is based on the relation between the D90 of the top bed layer and the geometrical roughness of Nikuradse according to (2.56).The user doesn’t have to specify a value for the bed friction coefficient.

(2.56)

Values of the drag coefficient for different seabed sediment grain sizes (flat beds) and similarly for bed form scenarios have been empirically derived from field and laboratory data in previous studies for different bed friction coefficients. The value of the friction coefficient (C, cf, n or ks) can be defined with one single value (keyword: bedfriccoef) or for a separate value per grid cell (keyword: bedfricfile)

### Damping by vegetation

The presence of aquatic vegetation within the area of wave propagation or wave breaking may not only result in short wave dissipation (Section 2.3.2.3), but also in damping of infragravity waves and/or mean flow. Since both long waves and mean flow are fully resolved with the nonlinear shallow water equations, the effect of vegetation can be modeled using a drag force (e.g. Dalrymple et al., 1984), which can be directly added to the momentum equations (Van Rooijen et al., 2015 or see (2.50)):

(2.57)

Where CD is a drag coefficient, bv is the vegetation stem diameter, N is the vegetation density and u is the wave or current related velocity. To take into account the velocity due to mean flow and infragravity waves, we use the Lagrangian velocity (uL) here. The vegetation-induced time varying drag force is then calculated as the sum of the vegetation-induced drag force per vegetation layer:

(2.58)

where is a (bulk) drag coefficient, bv,i is the vegetation stem diameter, Nv,i is the vegetation density, and hv,i is the vegetation height for layer i.

### Wind

The first term on the right hand side of the momentum equations [(2.50)] represents the forcing due to the wind stress. These forcing terms due to the wind are formulated as:

(2.59)										(

where τw is wind stress, ρa is density of air, Cd is the wind drag the coefficient, W is the wind velocity. The wind stress is turned off by default, and can be turned on specifying a constant wind velocity (keyword: windv = value) or by specifying a time varying wind file.

## Non-hydrostatic pressure correction

For non-hydrostatic XBeach calculations (keyword: waveform = nonh) depth-averaged flow due to waves and currents are computed using the non-linear shallow water equations, including a non-hydrostatic pressure. The non-hydrostatic model accounts for all wave motions (including short waves) within the shallow water equations, so the wave action balance should be turned off (keyword: swave = 0). The depth-averaged normalized dynamic pressure (q) is derived in a method similar to a one-layer version of the SWASH model (Zijlema et al. 2011). The depth averaged dynamic pressure is computed from the mean of the dynamic pressure at the surface and at the bed by assuming the dynamic pressure at the surface to be zero and a linear change over depth. In order to compute the normalized dynamic pressure at the bed, the contributions of advective and diffusive terms to the vertical momentum balance are assumed to be negligible.

(2.60)

In (2.60) w is the vertical velocity and z is the vertical coordinate. The vertical velocity at the bed is set by the kinematic boundary condition:

(2.61)

Combining the Keller-box method (Lam and Simpson 1976), as applied by Stelling and Zijlema (2003) for the description of the pressure gradient in the vertical, the dynamic pressure at the bed can be described by:

(2.62)

Substituting (2.61) in (2.60) allows the vertical momentum balance at the surface to be described by:

(2.63)

In (2.63) the subscript s refers to the location at the surface. The dynamic pressure at the bed is subsequently solved by combining (2.62) and the local continuity equation:

(2.64)

In order to improve the computed location and magnitude of wave breaking, the hydrostatic front approximation (HFA) of Smit et al. (2013) is applied, in which the pressure distribution under breaking bores is assumed to be hydrostatic. Following the recommendations of Smit et al. (2013), we consider hydrostatic bores if  and reform if . The values can respectively be changed with the keywords maxbrsteep and secbrsteep.

Figure 2.8	During wave breaking and wave runup the wave is modelled as a bore (Smit et al, 2010)

Although this method greatly oversimplifies the complex hydrodynamics of plunging waves, McCall et al. (2014) shows that the application of this method provides sufficient skill to describe dominant characteristics of the flow, without requiring computationally expensive high-resolution discretization of the vertical and surface tracking of overturning waves.

## Groundwater flow

The groundwater module (keyword: gwflow = 1) in XBeach utilizes the principle of Darcy flow for laminar flow conditions and a parameterization of the Forchheimer equations for turbulent groundwater flow. The module includes a vertical interaction flow between the surface water and groundwater. This flow is assumed to be a magnitude smaller than the horizontal flow and is not incorporated in the momentum balance.

### Continuity

In order to solve mass continuity in the groundwater model, the groundwater is assumed to be incompressible. Continuity is achieved by imposing a non-divergent flow field:

(2.65)

where U is the total specific discharge velocity vector, with components in the horizontal (ugw, vgw) and vertical (wgw) direction:

(2.66)

### Equation of motions

Laminar flow of an incompressible fluid through a homogeneous medium can be described using the well-known Law of Darcy (1856), valid for laminar flow conditions (keyword: gwscheme = laminar)

(2.67)

in which K is the hydraulic conductivity of the medium (keyword: kx, ky, kz, for each horizontal and vertical direction) and H is the hydraulic head.

In situations in which flow is not laminar, turbulent and inertial terms may become important. In these cases the user can specify XBeach to use a method (keyword: gwscheme = turbulent) that is comparable with the USGS MODFLOW-2005 groundwater model (Harbaugh 2005), in which the turbulent hydraulic conductivity is estimated based on the laminar hydraulic conductivity (Klam) and the Reynolds number at the start of turbulence (Recrit) (Halford 2000):

(2.68)

In (2.68) the Reynolds number (Re) is calculated using the median grain size (D50), the kinematic viscosity of water (ν) and the groundwater velocity in the pores (U/np), where np is the porosity. Similar expressions exist for the other two components of the groundwater flow.

The critical Reynolds number for the start of turbulence (Recrit) is specified by the user, based on in-situ or laboratory measurements, or expert judgment (keyword: gwReturb). Since the hydraulic conductivity in the turbulent regime is dependent on the local velocity, an iterative approach is taken to find the correct hydraulic conductivity and velocity.

#### Determination of the groundwater head

The XBeach groundwater model allows two methods to determine the groundwater head: a hydrostatic approach (keyword: gwnonh = 0) and a non-hydrostatic approach (keyword: gwnonh = 1).

Hydrostatic approach

In the hydrostatic approach, the groundwater head is computed as follows:

- In cells where there is no surface water the groundwater head is set equal to the groundwater surface level ηgw.

- In cells where there is surface water, but the groundwater surface level ηgw is more than dwetlayer (keyword: dwetlayer) below the surface of the bed, the groundwater head is set equal to the groundwater surface level.

- In cells where there is surface water and the groundwater surface level ηgw is equal to the surface of the bed, the groundwater head is set equal to the surface water level.

In cells where there is surface water and the groundwater surface level ηgw is equal to or less than dwetlayer below the surface of the bed, the groundwater head is linearly weighted between that of the surface water level and the groundwater level, according to the distance from the groundwater surface to the surface of the bed.

It should be noted that the numerical parameter dwetlayer is required to ensure numerical stability of the hydrostatic groundwater model. Larger values of dwetlayer will increase numerical stability, at the expense of numerical accuracy.

Non-hydrostatic approach

Groundwater flow in the swash and surf zone has been shown to be non-hydrostatic (e.g., Li and Barry 2000; Lee et al. 2007). In order to capture this, it may be necessary in certain cases to reject the Dupuit–Forchheimer assumption of hydrostatic groundwater pressure.

In the non-hydrostatic approach, the groundwater head is not assumed to be constant in the vertical. Since XBeach is depth-averaged, the model cannot compute true vertical profiles of the groundwater head and velocity. In order to estimate of the groundwater head variation over the vertical, a quasi-3D modeling approach is applied, which is set by two boundary conditions and one non-hydrostatic shape assumption:

- There is no exchange of groundwater between the aquifer and the impermeable layer below the aquifer.

- The groundwater head at the upper surface of the groundwater is continuous with the head applied at the groundwater surface.

- The shape of the non-hydrostatic head profile is parabolic (keyword: gwheadmodel = parabolic), implying that the vertical velocity increases or decreases linearly from the bottom of the aquifer to the upper surface of the groundwater, or the non-hydrostatic head profile is hyperbolic (keyword: gwheadmodel = exponential), cf., Raubenheimer et al. (1998).

The vertical groundwater head approximation can be solved for the three imposed conditions by a vertical head function, shown here for the parabolic head assumption. The depth-average value of the groundwater head is used to calculate the horizontal groundwater flux and is found by integrating the groundwater head approximation over the vertical:

(2.69)

In (2.69) the mean vertical ground water head (H) is calculated using the groundwater head imposed at the groundwater surface (Hbc), the groundwater head parabolic curvature coefficient (β) and the height of the groundwater level above the bottom of the aquifer (hgw).

The unknown curvature coefficient (β) in the vertical groundwater head approximation (2.69) is solved using the coupled equations for continuity and motion [Equations (2.65)  and (2.67)], thereby producing the depth-average horizontal groundwater head gradients and vertical head gradients at the groundwater surface.

Although the requirement for non-hydrostatic pressure has the benefit of being a more accurate representation of reality, and does not require the numerical smoothing parameter dwetlayer, resolving the non-hydrostatic pressure field can be computationally expensive, particularly in 2DH applications.

#### Exchange with surface water

In the groundwater model there are three mechanisms for the vertical exchange of groundwater and surface water: 1) submarine exchange, 2) infiltration and 3) exfiltration. The rate of exchange between the groundwater and surface water (S) is given in terms of surface water volume, and is defined positive when water is exchanged from the surface water to the groundwater.

Infiltration and exfiltration can only occur in locations where the groundwater and surface water are not connected. Infiltration takes place when surface water covers an area in which the groundwater level is lower than the bed level. The flux of surface water into the bed is related to the pressure gradient across the wetting front.

(2.70)

In (2.70) the surface water-groundwater exchange flow of infiltration (Sinf) is calculated using the effective hydraulic conductivity (K), the surface water pressure at the bed () and the thickness of the wetting front ().

Since the groundwater model is depth-averaged and cannot track multiple layers of groundwater infiltrating into the bed, the wetting front thickness is reset to zero when there is no available surface water, the groundwater exceeds the surface of the bed, or the groundwater and the surface water become connected. In addition, all infiltrating surface water is instantaneously added to the groundwater volume, independent of the distance from the bed to the groundwater table. Since the groundwater model neglects the time lag between infiltration at the beach surface and connection with the groundwater table a phase error may occur in the groundwater response to swash dynamics

Exfiltration (Sexf) occurs where the groundwater and surface water are not connected and the groundwater level exceeds the bed level. The rate of exfiltration is related to the rate of the groundwater level exceeding the bed level.

(2.71)

Submarine exchange (Ssub) represents the high and low frequency infiltration and exfiltration through the bed due pressure gradients across the saturated bed. This process only takes place where the groundwater and surface water are connected. In the case of the non-hydrostatic groundwater model, the rate of submarine exchange is determined by the vertical specific discharge velocity at the interface between the groundwater and surface water. The value of this velocity can be found using the vertical derivative of the approximated groundwater head at the groundwater-surface water interface (shown for the parabolic head approximation).

(2.72)

In the case of the hydrostatic groundwater model, the difference between the surface water head and the groundwater head is used to drive submarine discharge when the groundwater level is less than dwetlayer from the bed surface.

While most beach systems can acceptably described through vertical exchange of surface water and groundwater, in cases of very steep permeable slopes (e.g., porous breakwaters), it is necessary to include the horizontal exchange of groundwater and surface water between neighboring cells (keyword: gwhorinfil = 1). In this case the horizontal head gradient between the surface water and groundwater across vertical interface between the cells is used to determine the horizontal exchange flux:

(2.73)

where δHs is the head gradient between the surface water and groundwater in neighboring cell, δs is the gradient distance, defined as the numerical grid size, and A is the surface area through which the exchange takes place, defined as the difference in bed level between the neighboring cells.

#### Calculation of groundwater and surface water levels

Groundwater levels are updated through the continuity relation:

(2.74)

In these same areas the surface water level is modified to account for exchange fluxes:

(2.75)

#### Boundary conditions

Since the groundwater dynamics are described by a parabolic equation, the system of equations requires boundary conditions at all horizontal and vertical boundaries, as well as an initial condition:

A zero flux condition is imposed at the horizontal boundaries and bottom of the aquifer.

The initial condition for the solution is specified by the model user in terms of the initial groundwater head (keyword: gw0, or gw0file).

## Sediment transport

### Advection-diffusion equation

Sediment concentrations in the water column are modeled using a depth-averaged advection-diffusion scheme with a source-sink term based on equilibrium sediment concentrations (Galappatti and Vreugdenhil, 1985):

(2.76)

In (2.76) C represents the depth-averaged sediment concentration which varies on the wave-group time scale and Dh is the sediment diffusion coefficient. The entrainment of the sediment is represented by an adaptation time Ts, given by a simple approximation based on the local water depth h and sediment fall velocity ws. A small value of Ts corresponds to nearly instantaneous sediment response (keyword: Tsmin). The factor fTs is a correction and calibration factor to take into account the fact that ws is determined on depth-averaged data (keyword: tsfac).

(2.77)

The entrainment or deposition of sediment is determined by the mismatch between the actual sediment concentration C and the equilibrium concentration Ceq thus representing the source term in the sediment transport equation.

### General parameters

In the sediment transport formulations, the equilibrium sediment concentration Ceq (for both the bed load and the suspended load) is related to the velocity magnitude (vmg), the orbital velocity (urms) and the fall velocity (ws). This section elaborates how these are calculated.   Important to note: XBeach calculates the equilibrium concentration for the bed and suspended load separately.

First of all the Eulerian velocity magnitude, if long wave stirring is turned on (keyword: lws = 1), the velocity magnitude vmg is equal to the magnitude of the Eulerian velocity, as can be seen in Equation (2.78).

(2.78)

If wave stirring is turned off (keyword: lws = 0), the velocity magnitude will be determined by two terms: first of all a factor of the velocity magnitude of the previous time step (vmgn-1) and secondly a current-averaged part. Averaging will be carried out based on a certain factor fcats (keyword: cats) of the representative wave period Trep.

(2.79)

Secondly, the root-mean-squared velocity, the urms is obtained from the wave group varying wave energy using linear wave theory. In this formulation Trep is the representative wave period and the Hrms is the root-mean-square wave height. In this equation the water depth is enhanced with a certain factor of the wave height (keyword: delta).

(2.80)

To account for wave breaking induced turbulence due to short waves, the orbital velocity is adjusted (van Thiel de Vries, 2009). In this formulation kb is the wave breaking induced turbulence due to short waves. The turbulence is approximated with an empirical formulation in XBeach.

(2.81)

Thirdly, the fall velocity, the ws is calculated using the formulations of

(2000) which are derived based on a relationship suggested by Hallermeier (1981):

(2.82)

(2.83)

(2.84)

For high sediment concentrations, the fall velocity is reduced (keyword: fallvelred = 1) using the expression of Richardson and Zaki (1954):

(2.85)

The exponent α is estimated using the equation of Rowe (1987), which depends purely on the Reynolds particle number R:

(2.86)

(2.87)

### Transport formulations

In the present version of XBeach, two sediment transport formulations are available. The formulae of the two formulations are presented in the following sections. For both methods the total equilibrium sediment concentration is calculated with (2.88). In this equation the minimum value of the equilibrium sediment concentration (for both bed load en suspended load) compared to the maximum allowed sediment concentration (keyword: cmax).

(2.88)

The transport formulations implemented into XBeach distinguishes bed load and suspended load transport. It is possible to in- and exclude these transports components (keywords: bed & sus, with bed = 1 one will include bed load transport). There is also a possibility to compute the total bulk transport rather than bed and suspended load separately (keyword: bulk = 1). The bed load will be calculated if it is suspended transport. On top of that this switch will have impact on how the bed slope effect (see Section 2.7.6) will be calculated

#### Soulsby-Van Rijn

The first possible sediment transport formulation are the Soulsby-Van Rijn equations (keyword: form = soulsby_vanrijn) (Soulsby, 1997; van Rijn, 1984). The equilibrium sediment concentrations are calculated according to:

(2.89)

For which the bed load and suspended load coefficients are calculated with:

(2.90)

In which the dimensionless sediment diameter (D*) can be calculated with the following formulation. The v is the kinematic viscosity based on the expression of Van Rijn and is a function of the water temperature. XBeach assumes a constant temperature of 20 degrees Celsius, this result in a constant kinematic viscosity of 10-6 m2/s.

(2.91)

The critical velocity (Ucr) defines at which depth averaged velocity sediment motion is initiated:

(2.92)

Finally the drag coefficient (Cd) is calculated with Equation (2.93). A drag coefficient is used to determine the equilibrium sediment concentrations. On top of that Souslby (1997) gives a relation between the bed shear stress of the depth-averaged current speed.

(2.93)

In this equation z0 is used for the bed roughness length and is used as zero flow velocity level in the formulation of the sediment concentration. In XBeach this is a fixed value (keyword: z0), but Soulsby (1997) argues there is a relation between the Nikuradse and kinematic viscosity.

#### Van Thiel-Van Rijn

The second possible sediment transport formulation are the Van Thiel-Van Rijn transport equations (keyword: form = vanthiel_vanrijn) (van Rijn, 2007; van Thiel de Vries, 2009). The major difference between the Soulsby – Van Rijn equations is twofold. First of all, there is no drag coefficient calculated anymore and secondly the critical velocity is determined by calculating separately the critical velocity for currents (Ucrc) according to Shields (1936) and for waves (Ucrw) according to Komen and Miller (1975).

The equilibrium sediment concentrations are calculated according to

(2.94)

For which the bed-load and suspended load coefficient are calculated with:

(2.95)

The critical velocity is computed as weighted summation of the separate contributions by currents and waves (Van Rijn, 2007):

(2.96)

The critical velocity for currents is based on Shields (1936):

(2.97)

The critical velocity for waves is based on Komer and Miller (1975):

(2.98)

### Effects of wave nonlinearity

Effects of wave skewness and asymmetry are accounted for in the advection-diffusion equation, repeated here:

(2.99)

XBeach considers the wave energy of short waves as averaged over their length, and hence does not simulate the wave shape. A discretization of the wave skewness and asymmetry was introduced by Van Thiel de Vries (2009), to affect the sediment advection velocity. In this equation ua is calculated as function of wave skewness (Sk), wave asymmetry parameter (As), root-mean square velocity urms and two calibration factor fSk and fAs (keyword: facSk & facAs), see (2.100). To set both values one can use the keyword: facua. The method to determine the skewness and asymmetry is described in Section 2.3.4. A higher value for ua will simulate a stronger onshore sediment transport component.

(2.100)

### Hindered erosion by dilatancy

Under overwash and breaching conditions (high flow velocities and large bed level variations in time), dilatancy might hinder the erosion rates (De Vet, 2014). To account for this effect, the theory of Van Rhee (2010) could be applied (keyword: dilatancy = 1), reducing the critical Shields parameter at high flow velocities:

(2.101)

In this equation, ve refers to the erosion velocity, kl is the permeability, n0 is the porosity prior, nl is the porosity in the sheared zone (keyword pormax) and the parameter A (keyword rheeA) is equal to 3/4 for single particles and approximately 1.7 for a continuum.

The larger the permeability of the bed, the smaller the dilatancy effect. Van Rhee (2010) suggests using the equation proposed by Den Adel (1987):

(2.102)

Finally, the erosion velocity ve, is the velocity at which the bottom level decreases:

(2.103)

### Bed slope effect

The bed slope affects the sediment transport in various ways (Walstra, 2007):

- The bed slope influences the local near-bed flow velocity;

- The bed slope may change the transport rate once the sediment is in motion;

- The bed slope may change the transport direction once the sediment is in motion;

- The bed slope will change the threshold conditions for initiation of motion.

The influence of the bed slope on the local hydrodynamics is not considered in XBeach.

Two possible expressions are implemented to change the magnitude of the sediment transport. The first method is the default one in XBeach:

(2.104)

This method could be applied on either the total sediment transport (keyword: bdslpeffmag = roelvink_total) or only on the bed load transport (keyword: bdslpeffmag = roelvink_bed). The second method is based on the engineering formula of Soulsby (1997):

(2.105)

Also this method could be applied on the total transport (keyword: bdslpeffmag = soulsby_total) or on the bed load transport only (keyword: bdslpeffmag = soulsby_total). To change the direction of the bed load transport, the expressions of Van Bendegom (1947) and Talmon et al. (1995) could be used (keyword: bdslpeffdir = talmon):

(2.106)

(2.107)

(2.108)

Finally, it is possible to adjust the initiation of motion criteria for the total transport (keyword bdslpeffini = total) or the bed load transport only (keyword bdslpeffini = bed) through (Soulsby, 1997):

(2.109)

In this equation is ψ the difference in angle between the flow direction and the on-slope directed vector, β the bed slope and ϕi the angle of repose.

De Vet (2014) provides a detailed overview on how the bed slope and flow direction are calculated and how the bed slope effect is combined with the dilatancy concept if the adjustment to the initiation of motion is considered.

## Bottom updating

### Due to sediment fluxes

Based on the gradients in the sediment transport the bed level changes according to:

(2.110)

In (2.110) ρ is the porosity, fmor (keyword: morfac) is a morphological acceleration factor of O(1-10) (Reniers et al., 2004) and qx and qy represent the sediment transport rates in x- and y-direction respectively. Sediment transport can be activated with the keyword: sedtrans.

The morphological acceleration factor speeds up the morphological time scale relative to the hydrodynamic timescale. It means that if you have a simulation of 10 minutes with a morfac of 6 you effectively simulate the morphological evolution over one hour. There are now two ways in which you can input the time-varying parameters in combination with morfac:

- All times are prescribed on input in morphological time. If you apply a morfac all input time series and other time parameters are divided internally by morfac. This is determined with keyword morfacopt = 1. If you now specify a morfac of 6, the model just runs for 10 (hydrodynamic) minutes each hour, during which the bottom changes per step are multiplied by a factor 6. This of course saves a factor of 6 in computation time.

- This method is appropriate for short-term simulations with extreme events. This approach is only valid as long as the water level changes that are now accelerated by morfac do not modify the hydrodynamics too much. This is the case if the tide is perpendicular to the coast and the vertical variations do not lead to significant currents. If you have an alongshore tidal current, as is the case in shallow seas, you cannot apply this method because you would affect the inertia terms and thus modify the tidal currents.

Alternatively you run the model over, say, over a tidal cycle, and apply the morfac without modifying the time parameters. This means you leave all the hydrodynamic parameters unchanged and just exaggerate what happens within a tidal cycle. As long as the evolution over a single tidal cycle is limited, the mean evolution over a tidal cycle using a morfac is very similar to running morfac tidal cycles without morfac. See Roelvink (2006) for a more detailed description of this approach. This option is enabled with keyword: morfacopt = 0.

- This method is more appropriate for longer-term simulations with not too extreme events.

### Avalanching

To account for the slumping of sandy material from the dune face to the foreshore during storm-induced dune erosion avalanching (keyword: avalanching) is introduced to update the bed evolution. Avalanching is introduced via the use of a critical bed slope for both the dry and wet area (keyword: wetslp and dryslp). It is considered that inundated areas are much more prone to slumping and therefore two separate critical slopes for dry and wet points are used. The default values are 1.0 and 0.3 respectively. When this critical slope is exceeded, material is exchanged between the adjacent cells to the amount needed to bring the slope back to the critical slope.

(2.110)

To prevent the generation of large shockwaves due to sudden changes of the bottom level, bottom updating due to avalanching has been limited to a maximum speed of vav,max (keyword: dzmax). Equation (2.110) shows the resulting bed level change within one time step.

(2.110)

### Bed composition

If the effect of different sediment fractions, sorting and armoring are of importance, a bed composition constituting multiple sediment fractions can be defined. Each sediment fraction is characterized by a median grain size (D50) and possible a D15 and D90 as well. When using multiple sediment fractions, multiple bed layers are needed as well to describe the vertical distribution of the sediment fractions in the bed.

By specifying multiple bed layers, XBeach can keep track of the different sediment fractions both in the horizontal and in the vertical. Coarse sediments may be deposited on top of fine sediment after which erosion of the coarse sediment is needed to expose the fine sediment again, effectively armoring the bed. Three types of bed layers are distinguished: 1) the top layers 2) the variable or “breathing” layer and 3) the bottom layers. The top layer is the only layer that interacts with the water column and can be eroded, but preserves its thickness. The bottom layers are layers of constant thickness that move with the top layer. A single variable or “breathing” layer is defined that adapts its thickness to the erosion and sedimentation of the bed. For example: if a grid cell is eroded, particular fractions of sediment are removed from the top layer, but the top layer preserves its thickness and thus it takes the same volume of sediment, likely of different composition than the eroded sediment, from the layer below. If this layer is a top layer as well, the thickness is preserved and again the same volume of sediment is taken form a lower bed layer. This continues until the variable or “breathing” layer is reached. This layer adapts its thickness to the amount of erosion. If the thickness of the layer becomes too small, the variable layer is merged with an adjacent bottom layer and a new bottom layer is defined underneath the existing ones to ensure a constant number of bed layers. Reversely, if a grid cell is accreting, the thickness of the variable layer will be increased and with sufficient increase the variable layer will be split in two effectively creating a new bottom layer. The lowest existing bottom layer is then discarded to ensure a constant number of bed layers. The “breathing” layer can be the upper or bottom layer in which case the top layer or bottom layer class does not exist. The thickness of the different layer classes can be set separately (keyword: dzg1, dzg2 and dzg3) or at once (keyword: dzg).

Each grid cell in XBeach holds its own sediment distribution and the sediment transport formulations are used differentiate between fractions. Therefore the distribution of sediment may change over time and processes like armoring and sorting can be simulated. Due to the shifting of sediment between bed layers numerical mixing of sediment occurs. Choosing bed layer thicknesses that are in balance with the expected erosion and deposition during the simulation should keep the numerical mixing to a minimum. A bed layer thickness that is too large will result in relatively uniform behavior, while a bed layer thickness that is too small will result in a lot of shifting and thus numerical mixing (Figure A.1).

Figure 2.9	Visualization of the diffusion that occurs when XBeach calculates sediment compositions. After sedimentation of fine sediment on top of coarser material it is uniformly mixed over the whole layer. Subsequent erosion erodes both the fines as the coarser material. To avoid this phenomenon, layers should not be too thick (van der Zwaag, 2014).

## Ship-induced wave motions

A relatively new application field for XBeach is the generation and propagation of waves induced by sailing vessels. This functionality has been implemented recently (Zhou, 2013), and has currently been used in several studies (e.g. Zhou et al., 2014, De Jong et al., 2013), showing very good results.

For computing ship-induced waves the non-hydrostatic version of XBeach is used. A moving ship is represented as a pressure head that moves along a pre-defined track through the model domain. The ship is defined on a separate grid, where the ship draft is specified per grid point. Each computational time step the ship draft is interpolated from the ship grid to the global grid, where the ship volume is kept constant. Then the water pressure head in each global grid cell is updated based on the interpolated ship draft. By moving the pressure fields, the waves are generated and will propagate further through the global domain.

In Figure A.1 an example XBeach setup for ship waves is shown. The ship track is user defined and can, for instance, be obtained from the Automatic Identification System (AIS) for marine traffic. In this example, the model results were compared with measurements taken at Bath, The Netherlands. A filtered time series of the measured and computed water level is shown in Figure A.2. The time series was filtered to focus on the computation of the primary ship wave.

Figure 2.10	Example XBeach setup (left) and result (right) for a ship wave simulation in the Scheldt Estuary (The Netherlands). The ship track (red dashed line) is user-defined, and the measurement location is indicated (magenta dot).

Figure 2.11	Example XBeach result for ship-induced waves. Measurements are taken at Bath, in the Scheldt Estuary, The Netherlands (Schroevers et al., 2011).

In addition to the propagation of ship-induced waves, XBeach computes the forces and moments acting on the ship body. With this functionality, passing ship effects can be analyzed (e.g. Zhou, 2013).

# Boundary conditions

## Waves

XBeach allows users to include two different options for wave boundary conditions in the model. These wave boundary conditions can be applied only at the seaward boundary (keyword: wbctype). First of all, in Section 3.1.1 the method to specify wave spectra is discussed. Secondly, in Section 3.1.2 the method to apply non-spectra, such as stationary wave conditions or time-series is elaborated. In Section 3.1.4 the lateral boundary conditions for waves are discussed. There is currently not a possibility to force waves on the landward boundary of a model.

### Spectral conditions

The most-used wave boundary condition in XBeach is a spectral type. The input description of spectral wave boundary conditions can be found in Section 4.4.1.  XBeach allows the user to define these with three possibilities:

- Parameterized spectrum: With this option you define the boundary condition as parametric spectral input. The parameters (i.e. wave height, wave period, peak-enhancement factor, mean direction en directional spreading) can be specified. The option is especially handy when there is no nested model or measured spectrum. Here are two options:

  - Specify a single parametric spectrum (keyword wbctype = jons).

  - Specify a series of parametric spectra (keyword wbctype = jons_table).

- SWAN spectrum input: In this case the two-dimensional (frequency-direction) output by the spectral wave model SWAN (.sp2 files) can be specified. (keyword wbctype = swan). This option is especially convenient when nesting XBeach into a SWAN model.

- Formatted variance density spectrum: In this case a more general type spectrum can be specified.  (keyword wbctype =vardens). This option is often used when a measured spectrum is available.

### Non-spectral conditions

XBeach also allows the user to define non-spectral wave boundary conditions. This is a variation of both wave conditions without wave groups and time series. The input description of non-spectral wave boundary conditions can be found in Section 4.4.2. XBeach allows the user to define these with two possibilities:

- Stationary wave boundary condition. This means that a uniform and constant wave energy is specified, based on the given values of Hrms, Tm01, direction and power of the directional distribution function. The station boundary condition will not contain wave groups. Here there are two options:

  - Specify a single sea state (keyword wbctype = stat)

  - Specify a series of sea states (keyword wbctype = stat_table)

- Time series of waves. The user can also specify the variation in time of the wave energy. There are three options:

  - First-order time series of waves (keyword wbctype = ts_1). XBeach will calculate the bound long wave based on the theory of Longuet-Higgins and Stewart (1964).

  - Second-order time series of waves (keyword wbctype = ts_2). The bound long wave is specified by the user via a long wave elevation.

  - It is also possible to specify a variation in time of the horizontal velocity, vertical velocity and the free surface elevation (keyword: wbctype = ts_nonh). Last two terms are optional in this boundary conditions type.

### Special conditions

Besides clear spectral or non-spectral wave boundary conditions, there are also three special boundary condition types implemented in XBeach.

- Bichromatic (two short-wave components) waves (keyword wbctype = bichrom). In this case, XBeach will be forced with regular wave groups as the two short-wave components force one difference (infragravity) wave period. The user needs to specify not only variables of the stationary situation but also a wave period for the long wave. This wave period will be used to calculate the long wave based on the theory of Longuet-Higgins and Stewart (1964). The bichromatic boundary condition is the most simplified form of a wave spectrum.

- No wave boundary conditions (keyword wbctype = off). This is a simple no wave action boundary condition. It still allows for a tidal record to be specified, however this trough the zs0file parameter.

- Reuse previous boundary conditions (keyword: wbctype = reuse). If the user does not wish to recalculate boundary condition files or specifically wants to reuse the boundary condition files of another XBeach simulation should be used. No further wave boundary condition data need be given. Obviously, the calculation grid should remain the same between runs, as the angles and number of grid points are embedded in the boundary condition files.

### Lateral boundary conditions

There are three options to set the lateral boundaries for the wave model:

- Neumann boundaries (keyword: lateralwave = neumann): here the longshore gradient is set to zero.

- Wave crest boundaries (keyword: lateralwave = wavecrest). here the gradient in the wave energy along the wave crest is set to zero.

For the stationary wave mode (keyword: wavemodel = stationary) this is the only option. It allows a correct representation of the wave propagation near the lateral boundaries, without the usual shadow zones in e.g. SWAN. By neglecting the longshore gradients, the model automatically computes a consistent 1D solution.

For the surfbeat mode (keyword: wavemodel = surfbeat), Neumann leads to shadow zones, not so much in the wave height, but in the groupiness; the 'blobs' propagating in the mean wave direction turn into elongated, longshore uniform patches. To reduce this effect, the gradient along the wave crests of the wave energy can be set to zero, instead of the longshore gradient (keyword: lateralwave = wavecrest). This way the crests of the wave groups have approximately the right orientation, though the along-crest groupiness also disappears. In the wavecrest case, the wave refraction may be overestimated leading to somewhat too large longshore currents. The effects of both boundary conditions are shown in Figure A.1.

Figure 3.1	Effect of the lateral wave boundary conditions on root-mean square wave height patterns (top) and longshore velocity (bottom) for the Delilah test case.  In this figure the left panels are used for simulations with Neumann boundaries and the right panel with the wavecrest boundary.

## Shallow water equations

### Offshore boundary

Typically, an offshore or lateral boundary is an artificial boundary which has no physical meaning. On the offshore boundary wave and flow conditions are imposed. In the domain waves and currents will be generated which need to pass through the offshore boundary to the deep sea with minimal reflection. One way to do this is to impose a weakly reflective-type boundary condition (absorbing-generating), but there are also other possibilities implemented in XBeach (keyword: front). This method can be applied in 1D or 2D, is recommended and therefore the default value for XBeach.

In XBeach, there are two options with regard to the offshore absorbing-generating boundary condition. With the parameter setting front = abs1d a simple one-dimensional absorbing-generating boundary condition is activated. This option allows for a time-varying water level (surge and/or infragravity waves) to be specified at the boundary while allowing any waves propagating perpendicularly towards the boundary to be absorbed (i.e., passed through the boundary with a minimum of reflection. It is therefore only useful for 1D (flume like) simulations.

With option front = abs2d (default value) the formulation by Van Dongeren and Svendsen (1997) is activated which in turn is based on Verboom et al. (1981) and is based on the ‘Method of Characteristics’. This boundary condition allows for obliquely-incident and obliquely-reflected waves to pass through the boundary. It is possible to account for situations with boundary-perpendicular and boundary-parallel currents. In order to differentiate between the particle velocities, the keyword epsi must be set. This parameter control a simple Kalman-update filter which controls which part of the particle velocity is assumed to be part of the current and which part is wave-related. By default XBeach computes the value for epsi automatically using offshore boundary conditions (keyword: epsi = -1).

There are three other possibilities implemented besides the absorbing-generating boundary conditions:

- No flux wall (keyword: front = wall). This boundary condition type is a simple no flux boundary condition.

- Water level specification (keyword: front = wlevel). This boundary sets the water level at a prescribed value. This can be constant or time-varying. With this option the outgoing long waves are not absorbed.

- Boundary condition for the non-hydrostatic option (keyword: front = nonh_1d). The user needs to provide a file containing time series for the velocity at the boundary.

- Radiation boundary condition (keyword: front = waveflume). This boundary uses a continuity relation at the front boundary. This means that no net water can come into the model domain. The wave flume boundary condition is especially useful in lab experiments with a large set-up (e.g. coral reefs).

### Lateral boundaries

Lateral boundaries are the boundaries perpendicular to the coastline. Usually these are artificial, because the model domain is limited but the physical coast will continue. At these boundaries (keywords: left & right) we need to prescribe information about the area beyond the numerical model domain in such a way that the boundary condition does not influence the results in an adverse way. One way to do this is to prescribe a so-called “no-gradient” or Neumann boundaries (XBeach default), which state that there is locally no change in surface elevation and velocity, but there are also other possibilities implemented into XBeach. This method is recommended and is therefore the default value for XBeach. Each lateral boundary is a separate condition, so it is possible to mix different type of lateral boundary per side.

Neumann boundary conditions are activated where the longshore water level gradient is prescribed. The alongshore gradient is prescribed by the difference in specified water levels at the offshore corner points, divided by the alongshore length of the domain. This type of Neumann boundary condition has been shown to work quite well with (quasi-) stationary situations, where the coast can be assumed to be uniform alongshore outside the model domain. So far we have found that also in case of obliquely incident wave groups this kind of boundary conditions appears to give reasonable results when a shadow zone is taken into account. This means that regions where the boundary conditions are not fully enforced the results are not taken into account. Neumann boundaries can be individually defined (keyword: left = neumann).

There are three other possibilities implemented besides the absorbing-generating boundary conditions:

- Simple no-flux boundary conditions can also be applied (keyword: left = wall). Wall boundary conditions will result in a zero velocity at the lateral boundary.

- Velocity at the boundary will be calculated from NLSWE, but only include the advective terms (keywords: left = no_advec). The effect is that only terms that decrease the velocity will be taken into account. The result is an intermediate form between a full Neumann boundary and a wall boundary.

- Velocity at the boundary will simply be copied from the adjacent cell in the model domain (keyword: left = neumann_v).

### Time varying water level

XBeach can take in up to four time-vary tidal signals to be applied to the four boundaries (offshore-left, backshore-left, backshore-right, offshore-right).  A time-varying water level signal is read into XBeach by reading the specified file in zs0file. The input signal will be interpolated to the local time step of the simulation; therefore the signals only need to be long enough and temporally-fine enough to resolve the water level phenomenon of interest (i.e. tide variations, surge event).

There are now four options for handling the tidal and/or surge contribution to the boundaries:

- Uniform water level (keyword: tideloc = 0)

- One time-varying water level signal (keyword: tideloc = 1)

- Two time-varying water level signals, which requires point of application indication. (keyword: tideloc = 2)

- Four time-varying water level signals (keyword: tideloc = 4)

For the option with a uniform water level the value specified in the params.txt is applied in the complete model domain (keyword: zs0). For the option with one time-varying water level signal the specified water level is applied (keyword: zs0file = name_of_your_time_serie) to the offshore boundary and a fixed value is applied at the backshore boundary (keyword: zs0=value). For the option with two time-varying water level signals two water level signals are read from the zs0file. Note: one tidal record is applied to both sea corners and one tidal record to both land corners. This means there is no alongshore variation. An alongshore variation can be applied when applying four time-varying water level signals.

### River and point discharge

The effect of a river outflow or other discharges can be simulated with XBeach. Multiple discharge locations can be designated. At a discharge location the discharge orifice is defined as well as the discharge time series in m3/s. The discharge orifice always constitutes an uninterrupted series of full grid abreast cell borders. It is not possible to define a discharge over half a grid cell nor is it possible to define a single discharge through grid cell borders that are either not adjacent or are not abreast.

At each time step the model sets the discharge and velocities at the grid cell borders that constitute the discharge orifice, which can be computed given the size of the discharge orifice and discharge time series. The discharge is positive in positive x or y direction. An exception is made when discharges are defined at the domain border. In that case the discharge is positive towards the domain (influx).

When a discharge is defined with a zero size orifice the discharge is assumed to be in vertical direction where a positive discharge is into the domain (influx). In these cases the discharge is linked to the closest grid cell center and at each time step mass according to the discharge time series is added. No momentum is added in case of a vertical discharge.

## Sediment transport

The boundary conditions for sediment transport are Neumann boundaries everywhere, implying that the cross-boundary gradients in the advection-diffusion equation are set to zero, as well as the gradients of the bed load transports in that direction. Cross-shore profile changes due to cross-shore transport gradients are possible, allowing the boundary to smoothly follow the rest of the model. Still, it is good modeling practice to have the boundaries away from the area of interest.

## Cyclic boundary conditions

The cyclic boundary condition (keyword: cyclic = 1) treats two lateral boundary regions as if they are physically connected. This makes the model cyclic for everything, just treating it the same way as with MPI domain boundaries. Waves, flow and sediment transport that exists the domain at one side will be transported toward to other side. The only thing the user needs to make sure is that the two grid rows of bathymetric data on the lateral boundaries are identical to the ones on the other side. The advantage of cyclic boundary conditions is that there are no shadow zones (see Figure 3.2). Note: it only works with the MPI version of XBeach since it uses the same routine, however cyclic boundary conditions can also be applied with a single domain.

Figure 3.2		Effect of the lateral boundary conditions on root-mean square wave height patterns for the Delilah test case.  In this figure the upper panel is used for simulations with Neumann boundaries and the right panel with the cyclic boundary. One can clearly see the ‘bulbs’ of wave height (shadow zone) in the upper panel as a result of oblique wave attack.

# Input description

## General

Upon running the XBeach executable xbeach.exe, the file params.txt in the current working directory will be read. The params.txt file contains grid and bathymetry info, wave input, flow input, morphological input, etc. in the form of keyword/value pairs. Each keyword/value pair may contain an actual model parameter or refer to another file with additional information on the model setup. If a params.txt file cannot be found then XBeach will not run.

In the params.txt file there can be a single keyword/value pair per line. The keywords can be specified in any order. A keyword/value pair is separated by an equal sign (=). Each line containing an equal sign is interpreted as a keyword/value pair. Reversely, any lines without an equal sign are ignored and may be used for comments. Only a few keywords are required for the model to run, others have default values that are used in case the keyword is not mentioned in the params.txt file. The essential parameters for a simulation with a JONSWAP spectrum are listed below:

- A grid with values for x and y. This can both in XBeach format (separate x and y files; keyword: xfile and yfile) or Delft3D (one single xy file; keyword: xyfile). On top of that the user needs to specify the width of each domain (keyword: nx and ny)

A bathymetry file (keyword: depfile) that matches with the grid you specified at 1)

A simulation time (keyword: tstop) in seconds

A directional grid for short waves and rollers. The grid is determined by a minimum and maximum angle and width per bin (keywords: thetamin, thetamax and dtheta).

A wave boundary condition type (keyword: wbctype = jons). With a separate file containing the variables of the parametric (keyword: bcfile).

It is strongly recommended to specify as few parameters explicitly as possible and rely on the defaults for the other parameters. When running XBeach, a file called xbeach.log is created, which lists all the parameters set through the params.txt but also all parameters not set, for which the defaults are used. When the user starts the model, it generates a file named XBlog.txt. In this file all the different keyword available are determined. When no keyword is defined the default value will be applied.

This chapter describes the possibilities of the params.txt file and any auxiliary information files that are called from the params.txt file. The tables in this chapter contain a description of the keywords, the default values, its units and recommended value ranges, while the formats for additional input files are described in the relevant sections. Keyword marked with an astrix (*) are essential for XBeach to run. Keywords marked with a plus (+) are considered advanced expert options and should not be used for regular applications of XBeach.

In this chapter, any references to keywords refer to keywords that can be used in the params.txt file. Also any references to time indications are in seconds unless stated otherwise. A typical params.txt file for a 1D XBeach model is:

params.txt

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%% XBeach parameter settings input file                                %%%

%%%                                                                     %%%

%%% date:     01-Jan-2015 12:00                                         %%%

%%% function: xb_write_params                                           %%%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%% Grid parameters %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

depfile      = bed.dep

posdwn       = 0

nx           = 265

ny           = 0

alfa         = 0

vardx        = 1

xfile        = x.grd

yfile        = y.grd

thetamin     = -90

thetamax     = 90

dtheta       = 15

thetanaut    = 0

%%% Model time %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

tstop        = 3600

%%% Physical constants %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

rho          = 1025

%%% Tide boundary conditions %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

tideloc      = 2

zs0file      = tide.txt

%%% Wave boundary condition parameters %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

wbctype      = jons

bcfile       = filelist.txt

%%% Output variables %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

outputformat = netcdf

tint         = 3600

tstart       = 0

nglobalvar   = 3

zb

zs

H

## Physical processes

XBeach supports a variety of physical processes from generic, like waves and flow, to very specific, like ship motions and point discharge. Each process can be switched on or off. The commonly used processes are turned on by default. The table below lists the keywords used to switch on or off physical processes in XBeach.

Table A.1	Input parameters for physical processes supported by XBeach

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| avalanching | Turn on avalanching | 1 | 0 - 1 | - |  |
| flow | Turn on flow calculation | 1 | 0 - 1 | - |  |
| gwflow+ | Turn on groundwater flow | 0 | 0 - 1 | - |  |
| lwave | Turn on short wave forcing on NLSW equations and boundary conditions | 1 | 0 - 1 | - |  |
| morphology | Turn on morphology | 1 | 0 - 1 | - |  |
| nonh+ | Turn on non-hydrostatic pressure: 0 = NSWE, 1 = NSW + non-hydrostatic pressure compensation Stelling & Zijlema, 2003 | 0 | 0 - 1 | - |  |
| sedtrans | Turn on sediment transport | 1 | 0 - 1 | - |  |
| setbathy | Turn on time series of prescribed bathy input | 0 | 0 - 1 | - |  |
| ships+ | Turn on ship waves | 0 | 0 - 1 | - |  |
| single_dir+ | Turn on stationary model for refraction, surfbeat based on mean direction | 0 | 0 - 1 | - |  |
| snells+ | Turn on Snell's law for wave refraction | 0 | 0 - 1 | - |  |
| swave | Turn on short waves | 1 | 0 - 1 | - |  |
| swrunup+ | Turn on short wave runup | 0 | 0 - 1 | - |  |
| vegetation+ | Turn on interaction of waves and flow with vegetation | 0 | 0 - 1 | - |  |

## Grid and bathymetry

XBeach’ spatial grid size is defined by the keywords nx and ny. Here nx are the number of grid points in the cross-shore direction and ny the number in the alongshore direction. The size of the computational grid will be nx+1 by ny+1 cells large. The initial bathymetry is provided using a separate file that is referred to by the depfile keyword, which has to have a size of [nx+1, ny+1]. This file contains an initial bed level for each grid cell where each line corresponds to a transect in x-direction (cross-shore). The values are positive down by default (e.g. a value of ‘10’ is 10 meters of water depth), but this can be changed using the posdwn keyword.

Three main types of XBeach grids are supported: fast 1D, 1D and 2DH. Fast 1D grids have a single alongshore grid cell and thus a value ny=0 and thus a single row (ny+1=1) in the depfile. The 1D grids have 3 alongshore grid cells and thus a value ny=2 and three rows in the depfile. The 2DH grids have more than 3 alongshore grid cells, a value ny>2 and as many rows in the depfile. In general, the bathymetry file has the following space-separated format:

bed.dep

<z 1,1> <z 2,1> <z 3,1> ... <z nx,1> <z nx+1,1>

<z 1,2> <z 2,2> <z 3,2> ... <z nx,2> <z nx+1,2>

<z 1,3> <z 2,3> <z 3,3> ... <z nx,3> <z nx+1,3>

...

<z 1,ny> <z 2,ny> <z 3,ny> ... <z nx,ny> <z nx+1,ny>

<z 1,ny+1> <z 2,ny+1> <z 3,ny+1> ... <z nx,ny+1> <z nx+1,ny+1>

XBeach spatial grids can be equidistant or non-equidistant. In the former case the grid size is defined by the keywords dx and dy. In the latter case the keyword vardx should be set to 1 and x- and y-coordinates of the grid cells should be provided through the files referenced by the xfile and yfile keywords. These files take exactly the same format as the depfile file where all coordinates along the x-direction are in one row and each row represents a cell in y-direction. XBeach grids are defined in a coordinate system of choice and can be either rectangular or curvilinear grids. This section can be found in 2.1.

Delft3D grids created with tools like RFGRID are also supported. To use Delft3D grids, choose gridform=delft3d and provide a grid file via the keyword xyfile. The format of Delft3D grids is not described here, but can be found in the Delft3D manual (Deltares, 2011). Also forced updating of bathymetries is supported as described in Section 5B.11.

Apart for the spatial grid, XBeach also uses a directional grid for short waves and rollers. The grid is determined by a minimum and maximum angle and a directional bin size using the keywords thetamin, thetamax and dtheta respectively. The thetamin and thetamax angles are either defined according to the Cartesian convention (angle w.r.t. the computational x-axis) or according to the nautical convention (angle w.r.t. deg. N, so from W is 270 deg. N). The convention is chosen using the keyword thetanaut (thetanaut=0 for Cartesian and thetanaut=1 for Nautical)

Examples of typical input for a non-equidistant, fast 1D XBeach model, together with the params.txt example at the start of this chapter, are:

depfile = bed.dep

-20.00 -20.00 -19.90 -19.80 -19.70 ... 14 14 15 15 15

xfile = x.grd

0.00 10.00 20.00 30.00 40.00 ... 1992.00 1994.00 1996.00 1998.00 2000.00

yfile = y.grd

0.00 0.00 0.00 0.00 0.00 0.00 0.00 ... 0.00 0.00 0.00 0.00 0.00 0.00 0.00

All keywords related to grid and bathymetry input are listed in the following table:

Table A.1	Input parameters for grid and bathymetry supported by XBeach

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| alfa | Angle of computational x-axis relative to “East” | 0.0 | 0.0 - 360.0 | deg |  |
| depfile | Name of the input bathymetry file |  |  | <file> |  |
| dtheta | Directional resolution | 10.0 | 0.1 - 20.0 | deg |  |
| dtheta_s | Directional resolution in case of stationary refraction | 10.0 | 0.1 - 20.0 | deg |  |
| dx | Regular grid spacing in x-direction | -1.0 | 0.0 - 1000000000.0 | m |  |
| dy | Regular grid spacing in y-direction | -1.0 | 0.0 - 1000000000.0 | m |  |
| gridform | Grid definition format | xbeach | xbeach, delft3d |  |  |
| nx | Number of computational cell corners in x-direction | 50 | 2 - 10000 | - |  |
| ny | Number of computational cell corners in y-direction | 2 | 0 - 10000 | - |  |
| posdwn | Bathymetry is specified positive down (1) or positive up (-1) | 1.0 | -1.0 - 1.0 | - |  |
| thetamax | Higher directional limit (angle w.r.t computational x-axis) | 90.0 | -180.0 - 180.0 | deg |  |
| thetamin | Lower directional limit (angle w.r.t computational x-axis) | -90.0 | -180.0 - 180.0 | deg |  |
| thetanaut | specify thetamin and thetamax in cartesian (0) or nautical (1) convention | 0 | 0 - 1 | - |  |
| vardx | Switch for variable grid spacing | 0 | 0 - 1 | - |  |
| xfile* | Name of the file containing x-coordinates of the calculation grid |  |  | <file> |  |
| xori | X-coordinate of origin of axis | 0.0 | -100000000.0 - 1000000000.0 | m |  |
| xyfile | Name of the file containing Delft3D xy-coordinates of the calculation grid |  |  | <file> |  |
| yfile* | Name of the file containing y-coordinates of the calculation grid |  |  | <file> |  |
| yori | Y-coordinate of origin of axis | 0.0 | -1000000000.0 - 1000000000.0 | m |  |

## Waves input

An XBeach model is generally forced by waves on its offshore boundary. These waves are described by the wave boundary conditions discussed in this section. The details of the wave motions within the model are described by the wave numerics in terms of the wave action balance (see Section 2.3.1), wave dissipation model (see Section 2.3.2) and wave roller model (see Section 2.3.6)

XBeach supports a variety of wave boundary condition types that are divided in two main groups: stationary and spectral boundary conditions. The wbctype keyword can be used to select one particular type of wave boundary conditions. 3.1 gives an overview of all types of wave boundary conditions available for XBeach. Figure A.1 can be used to help determine what type of wave boundary conditions is appropriate for your case. Each wave boundary condition type is explained in the following subsections. Note that most spectral wave boundary conditions can vary both in space and time using a FILELIST and/or LOCLIST construction as described in Section 4.4.4.

Table A.1	Overview of wave boundary conditions supported by XBeach

| wbctype | Type | description |
| --- | --- | --- |
| off | Special | no wave boundary condition |
| stat | Non-spectral | stationary wave boundary condition (sea state) |
| bichrom | Special | bichromatic (two wave component) waves |
| ts_1 | Non-spectral | first-order time series of waves (generated outside XBeach), this option specifies free surface elevation and short wave energy |
| ts_2 | Non-spectral | second-order time series of waves (generated outside XBeach), this option specifies free surface elevation and short wave energy |
| jons | Spectral | wave groups generated using a parametric (JONSWAP) spectrum |
| swan | Spectral | wave groups generated using a SWAN 2D output file |
| vardens | Spectral | wave groups generated using a formatted file |
| ts_nonh | Non-spectral | boundary conditions initially meant for the non-hydrostatic wave type, this option specifies free surface elevation and velocity |
| reuse | Special | reuse of wave conditions |
| stat_table | Non-spectral | a sequence of stationary conditions (sea states) |
| jons_table | Spectral | a sequence of time-varying wave groups |

Figure 4.1	Decision tree for selecting the appropriate type of wave boundary conditions

### Spectral wave boundary conditions

Spectral wave boundary conditions are enabled using wbctype values jons, swan, vardens or jons_table. The conditions are defined in separate files referenced from the params.txt file using the bcfile keyword. A spectral wave boundary condition describes a spectrum shape that XBeach uses to generate a (random) wave time series. The length and resolution of the generated time series is determined by the keywords rt and dtbc respectively. XBeach will reuse the generated time series until the simulation is completed. The resolution of the time series should be enough to accurately represent the bound long wave, but need not be as small as the time step used in XBeach.

An overview of all keywords relevant for spectral wave boundary conditions is given in the table below. The necessary file formats for each type of spectral wave boundary condition is explained in the following subsections.

Table A.1	Input parameters for spectral wave boundary conditions supported by XBeach

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| Tm01switch+ | Switch to enable Tm01 rather than Tm-10 | 0 | 0 - 1 | - |  |
| bcfile | Name of spectrum file |  |  | <file> |  |
| correctHm0+ | Switch to enable Hm0 correction | 1 | 0 - 1 | - |  |
| dtbc+ | Time step used to describe time series of wave energy and long wave flux at offshore boundary (not affected by morfac) | 1.0 | 0.1 - 2.0 | s |  |
| dthetaS_XB+ | The (counter-clockwise) angle in the degrees needed to rotate from the x-axis in SWAN to the x-axis pointing East | 0.0 | -360.0 - 360.0 | deg |  |
| fcutoff+ | Low-freq cutoff frequency for wbctype = jons, swan or vardens boundary conditions | 0.0 | 0.0 - 40.0 | Hz |  |
| wbctype | Wave boundary condition type | bichrom | stat, bichrom, ts_1, ts_2, jons, swan, vardens, reuse, ts_nonh, off, stat_table, jons_table |  |  |
| nonhspectrum+ | Spectrum format for wave action balance of non-hydrostatic waves | 0 | 0 - 1 | - |  |
| nspectrumloc+ | Number of input spectrum locations | 1 | 1 – ny+1 | - |  |
| nspr+ | Switch to enable long wave direction forced into centers of short wave bins | 0 | 0 - 1 | - |  |
| oldnyq+ | Switch to enable old nyquist switch | 0 | 0 - 1 | - |  |
| random+ | Switch to enable random seed for wbctype = jons, swan or vardens boundary conditions | 1 | 0 - 1 | - |  |
| rt | Duration of wave spectrum at offshore boundary, in morphological time | 3600 | 1200-7200 | s |  |
| sprdthr+ | Threshold ratio to maximum value of S above which spectrum densities are read in | 0.08 | 0.0 - 1.0 | - |  |
| trepfac+ | Compute mean wave period over energy band: for wbctype jons, swan or vardens; converges to Tm01 for trepfac = 0.0 and | 0.01 | 0.0 - 1.0 | - |  |
| wbcversion+ | Version of wave boundary conditions | 3 | 1 - 3 | - |  |

#### JONSWAP wave spectra

JONSWAP spectrum input is enabled using wbctype = jons. A JONSWAP wave spectrum is parametrically defined in a file that is referenced using the bcfile keyword. This file contains a single parameter per line in arbitrary order. The parameters that can be defined are listed in Table A.1. All variables are optional. If no value is given, the default value as specified in the table is used. It is advised not to specify the keyword dfj and allow XBeach to calculate the default value.

A typical JONSWAP definition file looks as follows:

jonswap.txt

Hm0		= 0.8

Tp		= 8

mainang	= 285.

gammajsp	= 3.3

s		= 10.

fnyq		= 0.3

For the definitions see the table below.

It is possible to use an alternative file format for time-varying JONSWAP spectra. To enable this option use the wbctype value jons_table. In this case, each line in the spectrum definition file contains a parametric definition of a spectrum, like in a regular JONSWAP definition file, plus the duration for which that spectrum is used during the simulation. XBeach does not reuse time-varying spectrum files. Therefore the total duration of all spectra should at least match the duration of the simulation. The name of the file can be chosen freely, but the file format is fixed as follows and all parameters should be present in all lines:

jonswap.txt

<Hm0> <Tp> <mainang> <gammajsp> <s> <duration> <dbtc>

Note that we refer to the keywords used in a regular JONSWAP definition file in this example, with three differences: 1) the peak period rather than the peak frequency is defined 2) the duration is added (similar to rt in params.txt) 3) the time resolution is added (similar to dtbc in params.txt). The duration and boundary condition time step in this file overrules rt and dtbf in params.txt.

As an example, the JONSWAP spectrum definition file presented above would look as follows if the significant wave height should be increased with 0.2 m every hour:

jonswap.txt

0.8 8. 285. 3.3 10. 3600. 1

1.0 8. 285. 3.3 10. 3600. 1

1.2 8. 285. 3.3 10. 3600. 1

A more generic way of providing time-varying spectral wave boundary conditions is using a FILELIST construction as described in Section 4.4.4. This approach is compatible with all spectral wave boundary condition types as well as spatially varying boundary conditions as described in the same section.

The parameter s in the JONSWAP spectrum definition is related to the directional spreading (in deg.) through the following relation . Here σ is the directional spreading in radians and s the JONSWAP spreading parameter.

Figure 4.2	Effect a variation in s for the direction spreading of wave energy

Table A.1	Overview of available keywords in JONSWAP definition file

| keyword | description | default | minimum | maximum |
| --- | --- | --- | --- | --- |
| Hm0 | Hm0 of the wave spectrum, significant wave height [m] | 0.0 | 0.0 | 5.0 |
| fp | Peak frequency of the wave spectrum [s-1] | 0.08 | 0.0625 | 0.4 |
| gammajsp | Peak enhancement factor in the JONSWAP expression [-] | 3.3 | 1.0 | 5.0 |
| s | Directional spreading coefficient, cos2s law [-] | 10. | 1.0 | 1000. |
| mainang | Main wave angle (nautical convention) [°] | 270. | 180. | 360. |
| fnyq | Highest frequency used to create JONSWAP spectrum [s-1] | 0.3 | 0.2 | 1.0 |
| dfj | Step size frequency used to create JONSWAP spectrum [s-1] | fnyq/200 | fnyq/1000 | fnyq/20 |

#### SWAN wave spectra

XBeach can read standard SWAN 2D variance density or energy density output files (+.sp2 files) as specified in the SWAN v40.51 manual. This option is enabled using wbctype = swan in params.txt and a reference to the spectrum file via the keyword bcfile. XBeach assumes the directional information in the SWAN file is according to the nautical convention. If the file uses the Cartesian convention for directions, the user must specify the angle in degrees to rotate the x-axis in SWAN to the x-axis in XBeach (by the Cartesian convention). This value is specified in params.txt using the keyword dthetaS_XB.

Note that time-varying and spatially varying SWAN spectra can be provided using the FILELIST and LOCLIST constructions as described in Section 4.4.4.

An example of a 2D SWAN spectrum is given below:

swan.txt

SWAN   1                                Swan standard spectral file

$ Data produced by SWAN version 40.51

$ Project:'projname'     ;   run number:'runnum'

LOCATIONS                               locations in x-y-space

1                                       number of locations

22222.22        0.00

RFREQ                                   relative frequencies in Hz

23                                      number of frequencies

0.0545

0.0622

0.0710

0.0810

0.0924

0.1055

0.1204

0.1375

0.1569

0.1791

0.2045

0.2334

0.2664

0.3040

0.3470

0.3961

0.4522

0.5161

0.5891

0.6724

0.7675

0.8761

1.0000

CDIR                                    spectral Cartesian directions in degr

12                                    number of directions

30.0000

60.0000

90.0000

120.0000

150.0000

180.0000

210.0000

240.0000

270.0000

300.0000

330.0000

360.0000

QUANT

1                                       number of quantities in table

VaDens                                  variance densities in m2/Hz/degr

m2/Hz/degr                              unit

-0.9900E+02                             exception value

FACTOR

0.675611E-06

51   242   574   956   1288  1482  1481   1286   957    579    244    51

129  610   1443  2402  3238  3725  3724   3234   2406   1454   613    128

273  1287  3054  5084  6846  7872  7869   6837   5091   3076   1295   271

665  3152  7463  12402 16712 19229 19221  16690  12419  7518   3172   662

1302 6159  14608 24275 32688 37618 37603  32644  24309  14716  6198   1296

2328 10989 26020 43341 58358 67109 67080  58281  43401  26213  11058  2317

3365 15922 37712 62733 84492 97150 97110  84380  62820  37991  16021  3349

3426 16230 38440 63939 86109 99010 98969  85995  64027  38724  16331  3410

2027 9612  22730 37790 50909 58529 58505  50841  37843  22898  9672   2018

672  3178  7538  12535 16892 19440 19432  16870  12552  7594   3198   669

101  479   1135  1890  2542  2924  2923   2539   1892   1144   482    101

2    11    26    43    57    66    66     57     43     26     11     2

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

0    0     0     0     0     0     0      0      0      0      0      0

#### Variance density spectra

2D spectral information that is not in SWAN format can be provided using a formatted variance density spectrum file and wbctype = vardens. The spectrum file itself is again referenced using the keyword bcfile. The contents of the file must adhere to a specific format:

vardens.txt

<number of frequencies (n)>

<frequency 1>

<frequency 2>

<frequency 3>

...

<frequency n-1>

<frequency n>

<number of directions (m)>

<directions 1>

<directions 2>

<directions 3>

...

<directions m-1>

<directions m>

<variance density 1,1> <variance density 2,1> ... <variance density m,1>

<variance density 1,2> <variance density 2,2> ... <variance density m,2>

...

<variance density 1,n> <variance density 2,n> ... <variance density m,n>

Note that the directions must be defined according to the Cartesian convention and in the coordinate system used by XBeach. In this coordinate system 0° corresponds to waves travelling in the direction of the x-axis, while 90° corresponds to the direction of the y-axis. Also, the directions must be defined in increasing order. Time-varying and spatially varying variance density spectra can be provided using the FILELIST and LOCLIST constructions as described in Section 4.4.4.

An example of a formatted variance density file is given below:

vardens.txt

15

0.0418

0.0477

0.0545

0.0622

0.0710

0.0810

0.0924

0.1055

0.1204

0.1375

0.1569

0.1791

0.2045

0.2334

0.2664

13

-180.0000

-150.0000

-120.0000

-90.0000

-60.0000

-30.0000

0.0000

30.0000

60.0000

90.0000

120.0000

150.0000

180.0000

0     0     0     0     0     0     0     0     0     0     0     0

51   242   574   956  1288  1482  1481  1286   957   579   244    51

129   610  1443  2402  3238  3725  3724  3234  2406  1454   613   128

273  1287  3054  5084  6846  7872  7869  6837  5091  3076  1295   271

665  3152  7463 12402 16712 19229 19221 16690 12419  7518  3172   662

1302  6159 14608 24275 32688 37618 37603 32644 24309 14716  6198  1296

2328 10989 26020 43341 58358 67109 67080 58281 43401 26213 11058  2317

3365 15922 37712 62733 84492 97150 97110 84380 62820 37991 16021  3349

3426 16230 38440 63939 86109 99010 98969 85995 64027 38724 16331  3410

2027  9612 22730 37790 50909 58529 58505 50841 37843 22898  9672  2018

672  3178  7538 12535 16892 19440 19432 16870 12552  7594  3198   669

101   479  1135  1890  2542  2924  2923  2539  1892  1144   482   101

2    11    26    43    57    66    66    57    43    26    11     2

0     0     0     1     1     1     1     1     1     0     0     0

0     0     0     0     0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0     0     0     0     0

### Non-spectral wave boundary conditions

Stationary wave boundary conditions are enabled using wbctype values stat, ts_1, ts_2, ts_nonh or stat_table. The conditions are generally defined within the params.txt file directly using the keywords described in the table below. In addition, in case of wbctype values ts_1 or ts_2 the file bc/gen.ezs should be present that describes the infragravity wave forcing generated outside of XBeach. For the wbctype ts_nonh also a separate file is needed. For more information about these files. see Section 4.4.2.2.

#### Stationary wave boundary conditions

Only in case of wbctype = stat_table the time-varying stationary wave boundary conditions are fully described in an external file referenced by the bcfile keyword. The bcfile keyword is part of the spectral wave boundary condition input and also the referenced file is designed for time-varying spectral input in the form of JONSWAP spectra (see Section 4.4.1.1). In stationary mode only the relevant data from this file is used and irrelevant data like gamma and dfj are discarded.

Table A.1	Overview of available keywords for stationary boundary conditions

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| Hrms | Hrms wave height for wbctype = stat, bichrom, ts_1 or ts_2 | 1.0 | 0.0 - 10.0 | m |  |
| Tlong | Wave group period for case wbctype = bichrom | 80.0 | 20.0 - 300.0 | s |  |
| Trep | Representative wave period for wbctype = stat, bichrom, ts_1 or ts_2 | 10.0 | 1.0 - 20.0 | s |  |
| dir0 | Mean wave direction for wbctype = stat, bichrom, ts_1 or ts_2 (nautical convention) | 270.0 | 180.0 - 360.0 | deg |  |
| wbctype | Wave boundary condition type | bichrom | stat, bichrom, ts_1, ts_2, jons, swan, vardens, reuse, ts_nonh, off, stat_table, jons_table |  |  |
| lateralwave | Switch for lateral boundary at left | neumann | neumann, wavecrest, cyclic |  |  |
| m | Power in cos^m directional distribution for wbctype = stat, bichrom, ts_1 or ts_2 | 10 | 2 - 128 | - |  |
| nmax+ | Maximum ratio of cg/c for computing long wave boundary conditions | 0.8 | 0.5 - 1.0 | - |  |
| taper | Spin-up time of wave boundary conditions, in morphological time | 100.0 | 0.0 - 1000.0 | s |  |

#### Time series

The wave boundary condition types of ts_1 and ts_2 need a separate file containing short wave energy and free surface elevation (including long wave motions). The format of this file is as follows:

bc/gen.ezs

<time 1> <zs 1> <E 1>

<time 1> <zs 2> <E 2>

<time 2> <zs 3> <E 3>

...

The wave boundary condition type of ts_nonh also needs a separate file in order to run the simulation. This file, however, needs to contain free surface elevations and velocities (both in u and v).

Boun_u.bcf

< scalar/vector > < number of variables > < variables: t,U,Zs,W >

### Special types of wave boundary conditions

Two special types of wave boundary conditions are available that makes XBeach skip the generation of new wave time series. The first is wbctype = off which simply does not provide any wave forcing on the model and hence no wave action in the model.

The second is wbctype = reuse which makes XBeach reuse wave time series that were generated during a previous simulation. This can be a simulation using the same or a different model as long as the computational grids are identical. In order to reuse boundary conditions, all relevant files should be copied to the current working directory of the model (where the params.txt file is located). Relevant files are the ebcflist.bcf and qbcflist.bcf files and all files referenced therein. Generally, the referenced files have E_ and q_ prefixes. No further wave boundary condition data need be given in params.txt.

On top of that bichromatic waves are also supported. Currently the same input parameters as non-spectral waves (see 4.4.2) are required; however there are planes to elaborate the input parameters of the bichromatic wave in order to specify individual frequency.

### Temporally and/or spatially varying wave boundary conditions

Time-varying spectral wave boundary conditions can be defined by feeding in multiple spectrum definition files rather than a single definition file. In addition, the duration for which these spectra should occur needs to be defined.

To make use of this option, the user must specify a regular wbctype value for spectral wave boundary conditions (jons, swan or vardens), but instead of referencing a single spectrum definition file using the bcfile keyword, an extra file listing all spectrum definition files is now referenced.

The first word in this extra file must be the keyword FILELIST. In the following lines, each line contains the duration of this wave spectrum condition in seconds (similar to rt in params.txt), the required time step in this boundary condition file in seconds (similar to dtbf in params.txt) and the name of the spectral definition file used to generate these boundary conditions. The duration and boundary condition time step in this file overrules rt and dtbf in params.txt. XBeach does not reuse time-varying spectrum files. Therefore the total duration of all spectra should at least match the duration of the simulation.

A typical input file contains the following:

filelist.txt

FILELIST

1800 0.2 jonswap1.inp

1800 0.2 jonswap1.inp

1350 0.2 jonswap2.inp

1500 0.2 jonswap3.inp

1200 0.2 jonswap2.inp

3600 0.2 jonswap4.inp

Similar to time-varying spectral wave boundary conditions, also spatially varying wave boundary conditions can be defined using a similar construction. In order to apply spatially varying spectra on the offshore boundary, the user must specify set the keywords wbcversion =3 and nspectrumloc=ns in params.txt where ns is the number of locations in which a spectrum is defined. By default the number of defined spectra is one.

Similar to time-varying spectral wave boundary conditions, its spatially varying sibling uses an extra file listing all relevant spectrum definition files. The first word in this extra file must be the keyword LOCLIST. This line should be followed by one line per spectrum definition location containing the world x-coordinate and world y-coordinate of the location that the input spectrum should apply, and the name of the file containing spectral wave information.

A typical input file for a run with three JONSWAP spectra contains the following:

loclist.txt

LOCLIST

0. 0.    jonswap1.inp

0. 100.  jonswap2.inp

0. 200.  jonswap3.inp

Note that it is not possible to use a mix of JONSWAP, SWAN and variance density files in either a FILELIST or a LOCLIST construction. It is also not possible to vary dthetaS_XB between files in case of non-nautical SWAN spectra. However, it is possible to combine FILELIST and LOCLIST files by referencing FILELIST files from the LOCLIST file. In this case all FILELIST files should adhere to the same time discretization, so the duration and time step values should be constant over al FILELIST files as well as the number of wave spectra definitions.

The user is reminded that along the offshore boundary of the model, the wave energy, rather than the wave height, is interpolated linearly between input spectra without consideration of the physical aspects of the intermediate bathymetry. In cases with large gradients in wave energy, direction or period, the user should specify sufficient wave spectra for the model to accurately represent changes in offshore wave conditions.

## Flow, tide and surge input

An XBeach model needs flow boundary conditions on all boundaries of the model domain. Moreover, on each boundary  tidal elevations and/ or surges may be imposed. The flow boundary conditions and time-varying tide or surge input are discussed in this section. The details on how the flow is computed within the model are described in the sections on bed friction and viscosity parameters (see Section 2.4).

### Flow boundary conditions

Flow boundary conditions need to be specified on all sides of the domain. We will differentiate between the offshore, lateral and landward boundaries that are set using the keywords front, back and left/right, respectively. From Table A.1 to Table A.4 an overview is given of the available flow boundary condition types for each of these boundaries.

The keyword freewave can be used to switch from bound to free long waves, which can be useful when time series of the free long wave incident on the offshore boundary need to be specified. The file bc/gen.ezs can be used to describe the free long waves at the offshore boundary as discussed in Section 3.1.

Table A.1	Overview of available offshore flow boundary condition types

| front | description |
| --- | --- |
| abs1d | absorbing-generating (weakly-reflective) boundary in 1D |
| abs2d | absorbing-generating (weakly-reflective) boundary in 2D |
| wall | no flux wall |
| wlevel | water level specification (from file) |
| nonh_1d | boundary condition for non-hydrostatic option |
| waveflume | boundary condition for flume experiments based on a continuity relation |

Table A.2	Overview of available landward flow boundary condition types

| back | description |
| --- | --- |
| wall | no flux wall |
| abs1d | absorbing-generating (weakly-reflective) boundary in 1D |
| abs2d | absorbing-generating (weakly-reflective) boundary in 2D |
| wlevel | water level specification (from file) |

Table A.3	Overview of available lateral flow boundary condition types

| back | description |
| --- | --- |
| wall | no flux wall |
| neumann | Neumann boundary condition (constant water level gradient) |
| neumann_v | velocity is determined by the adjacent cell |
| no_advec | Neumann boundary condition, but only the advective terms are taken into account. Intermediate form between wall and neumann |

Table A.4	Preview of all keywords related to the flow boundary conditions

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| ARC+ | Switch for active reflection compensation at seaward boundary | 1 | 0 - 1 | - |  |
| back | Switch for boundary at bay side | abs_2d | wall, abs_1d, abs_2d, wlevel |  |  |
| epsi+ | Ratio of mean current to time varying current through offshore boundary | -1.0 | -1.0 - 0.2 | - |  |
| freewave+ | Switch for free wave propagation 0 = use cg (default); 1 = use sqrt(gh) in wbctype = ts_2 | 0 | 0 - 1 | - |  |
| front | Switch for seaward flow boundary | abs_2d | abs_1d, abs_2d, wall, wlevel, nonh_1d, waveflume |  |  |
| left | Switch for lateral boundary at ny+1 | neumann | neumann, wall, no_advec, neumann_v |  |  |
| nc+ | Smoothing distance for estimating umean (defined as nr of cells) | ny+1 | 1 - ny+1 | - |  |
| order+ | Switch for order of wave steering, 1 = first order wave steering (short wave energy only), 2 = second order wave steering (bound long wave corresponding to short wave forcing is added) | 2.0 | 1.0 - 2.0 | - |  |
| right | Switch for lateral boundary at 0 | neumann | neumann, wall, no_advec, neumann_v |  |  |
| tidetype+ | Switch for offshore boundary, velocity boundary or instant water level boundary | velocity | instant, velocity |  |  |

### Time-varying water level

Time-varying tidal (or surge) signals can be applied all four boundaries in a number of ways.

The number of tidal signals is determined by the keyword tideloc that can take the values 0, 1, 2 or 4. Specifying three tidal signals is not an option. Setting tideloc=0 disables the time-varying tide/surge option. In this case a constant and uniform water level is used specified by the keyword zs0. With tideloc =1 the specified tidal record is specified on all four corners of the domain and interpolated along the boundaries.

Using tideloc = 2, two tidal signals are specified and there are two options available: 1) the first signal is imposed on the offshore boundary and the second on the landward boundary or 2) the first signal is imposed on the left lateral boundary and the second on the right lateral boundary. The choice between the two options is made using the keyword paulrevere where a value 0 indicates the first option and a value 1 indicates the second option. Also in the case of two tidal signals the signals are spatially interpolated along the boundaries.

Using tideloc=4, four tide/surge signals are to be specified on each corner of the model domain and spatially interpolated along the boundaries. The first signal is imposed to the left offshore boundary seen from sea (x=1,y=1) and the others according to a clockwise rotation. Therefore the columns in the zs0file must follow the order of: (x=1,y=1), (x=1,y=N), (x=N,y=N), (x=N,y=1).

The length of the tidal signals is determined by the keyword tidelen. This is the number of water levels specified in the file referenced with the zs0file keyword. The tidal signal will be interpolated to the local time step of the XBeach simulation; therefore the resolution of the signals only needs to be enough to resolve the water level phenomenon of interest (i.e. tide variations, surge event). The tidal signals are not re-used, therefore the signal should be at least as long as the simulation time.

The zs0file file must adhere to the following format where the last three columns are optional depending on the value of tideloc and tlen represents the value of tidelen:

tide.txt

<time 1> <zs 1,1> [<zs 2,1> [<zs 3,1> <zs 4,1>]]

<time 2> <zs 1,2> [<zs 2,2> [<zs 3,2> <zs 4,2>]]

<time 3> <zs 1,3> [<zs 2,3> [<zs 3,3> <zs 4,3>]]

...

<time tlen> <zs 1,tlen> [<zs 2,tlen> [<zs 3,tlen> <zs 4,tlen>]]

In case of a single tidal signal, the signal is imposed on both offshore corners of the domain, while a constant water level defined by the keyword zs0 is imposed on the landward corners.

Table A.1	Overview of all keywords related to the tide boundary conditions

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| paulrevere | Specifies tide on sea and land or two sea points if tideloc = 2 | land | land, sea |  |  |
| tideloc | Number of corner points on which a tide time series is specified | 0 | 0 - 4 | - |  |
| zs0 | Initial water level | 0.0 | -5.0 - 5.0 | m |  |
| zs0file | Name of tide boundary condition series |  |  | <file> |  |

## Water level (dam break)

Water levels can be imposed on the model boundaries as explained in Section 4.5.2 Time-varying  after which the shallow water equations force the water body in the model domain. Specific applications may require the initialization of the entire water body in the model domain at the start of the simulation. For example, an initial significant gradient in the water level that “collapses” at the start of the simulation may simulate a dam break. The initialization of the water level in the model domain is governed by the keywords listed in the table below.

The keyword zsinitfile references an external file describing the initial water levels in the entire model domain. The file should have the same format as the bathymetry input files described in Section 4.3 (Grid and bathymetry).

Table A.1	Overview of all keywords related to the water levels

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| hotstartflow+ | Switch for hotstart flow conditions with pressure gradient balanced by wind and bed stress | 0 | 0 - 1 | - |  |
| zs0 | Initial water level | 0.0 | -5.0 - 5.0 | m |  |
| zsinitfile | Name of initial water level file |  |  | <file> |  |

## Wind input

Spatially-uniform winds can parametrically defined using the keywords windv and width that represent the wind velocity and direction (nautical convention) respectively. Time-varying winds can be defined in an external file referenced by the windfile keyword. The file should adhere to the format indicated below. The total length of the time series is automatically determined and should be at least as long as the simulation time.

wind.txt

<time 1> <windv 1> <windth 1>

<time 2> <windv 2> <windth 2>

<time 3> <windv 3> <windth 3>

...

The table below gives an overview of all keywords related to the wind:

Table A.1	Overview of all keywords related to the wind input

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| Cd+ | Wind drag coefficient | 0.002 | 0.0001 - 0.01 | - |  |
| rhoa+ | Air density | 1.25 | 1.0 - 2.0 | kgm^-3 |  |
| windfile | Name of file with non-stationary wind data |  |  | <file> |  |
| windth | Nautical wind direction, in case of stationary wind | 270.0 | -360.0 - 360.0 | deg |  |
| windv | Wind velocity, in case of stationary wind | 0.0 | 0.0 - 200.0 | ms^-1 |  |

## Sediment input

The sediment input determines the (initial) composition of the bed and the detail in which processes related to sediment sorting are resolved. This is different from how the sediment transport processes are handled in the model itself and that are described in 2.7 and 2.8

The simplest situation is an XBeach simulation with uniform sediment. In this case it is sufficient to specify the uniform grain size using the keyword D50 indicating the median grain size. The effects of a specific sediment distribution can be parametrically defined by additionally specifying values for D15 and D90 and optionally the bed composition can be fine-tuned by specifying the porosity and sediment density using the keywords por and rhos respectively. In this case no sorting of sediment will be simulated.

If the effect of different sediment fractions, sorting and armoring are of importance, multiple sediment fractions can be defined. The number of sediment fraction is determined by the keyword ngd. For each sediment fraction a value for D50, and optionally D15 and D90, should be defined separated by a space. Moreover, when using multiple sediment fractions, multiple bed layers are needed as well. The number of bed layers can be defined using the keyword nd.

Three types of bed layers are distinguished: 1) the top layer 2) the variable or “breathing” layer and 3) the bottom layers. At least one of each type of bed layer is needed, which makes that at least three bed layers are required (see Section 2.8.3). Each bed layer has a thickness. Choosing bed layer thicknesses that are in balance with the expected erosion and deposition during the simulation should keep the numerical mixing to a minimum. A bed layer thickness that is too large will result in relatively uniform behavior, while a bed layer thickness that is too small will result in a lot of shifting and thus numerical mixing. The bed layer thicknesses are determined by the three keywords dzg1, dzg2 and dzg3 for the top, variable and bottom layers respectively.

Apart from the discretization of the grain size distribution and the vertical structure of the bed, the initial bed composition needs to be defined. The bed composition is defined using external files that are not explicitly referenced from params.txt, but are assumed to be located in the working directory of the model (next to params.txt). There is one file for each sediment fraction specified by ngd. The file corresponding to the first sediment fraction is named gdist1.inp, the second gdist2.inp, et cetera.

The bed composition files hold information on how much sediment of a specific fraction is in each grid cell and bed layer at the start of the simulation. The values are a volumetric fraction that implies that they should add up to unity over all fractions. For example, if a specific grid cell is filled with the first sediment fraction only, the value corresponding to this grid cell will be one in the gdist1.inp file and zero in all others. Alternatively, if we defined five sediment fractions and a specific grid cell is filled equally with all fractions, the value corresponding to this grid cell will be 1/5 = 0.2 in all files. The gidst<N>.inp files are formatted comparable to the bathymetry files (see 4.3 Grid and bathymetry), but now holds values over the three dimensions x (nx+1), y (ny+1) and the bed layers (nd). The file format is as follows:

gdist1.inp

<p 1,1,1> <p 1,2,1> <p 1,3,1> ... <p 1,nx,1> <p 1,nx+1,1>

<p 1,1,2> <p 1,2,2> <p 1,3,2> ... <p 1,nx,2> <p 1,nx+1,2>

<p 1,1,3> <p 1,2,3> <p 1,3,3> ... <p 1,nx,3> <p 1,nx+1,3>

...

<p 1,1,ny> <p 1,2,ny> <p 1,3,ny> ... <p 1,nx,ny> <p 1,nx+1,ny>

<p 1,1,ny+1> <p 1,2,ny+1> <p 1,3,ny+1> ... <p 1,nx,ny+1> <p 1,nx+1,ny+1>

...

<p 2,1,1> <p 2,2,1> <p 2,3,1> ... <p 2,nx,1> <p 2,nx+1,1>

<p 2,1,2> <p 2,2,2> <p 2,3,2> ... <p 2,nx,2> <p 2,nx+1,2>

<p 2,1,3> <p 2,2,3> <p 2,3,3> ... <p 2,nx,3> <p 2,nx+1,3>

...

<p 2,1,ny> <p 2,2,ny> <p 2,3,ny> ... <p 2,nx,ny> <p 2,nx+1,ny>

<p 2,1,ny+1> <p 2,2,ny+1> <p 2,3,ny+1> ... <p 2,nx,ny+1> <p 2,nx+1,ny+1>

...

<p nd,1,1> <p nd,2,1> <p nd,3,1> ... <p nd,nx,1> <p nd,nx+1,1>

<p nd,1,2> <p nd,2,2> <p nd,3,2> ... <p nd,nx,2> <p nd,nx+1,2>

<p nd,1,3> <p nd,2,3> <p nd,3,3> ... <p nd,nx,3> <p nd,nx+1,3>

...

<p nd,1,ny> <p nd,2,ny> <p nd,3,ny> ... <p nd,nx,ny> <p nd,nx+1,ny>

<p nd,1,ny+1> <p nd,2,ny+1> ... <p nd,nx,ny+1> <p nd,nx+1,ny+1>

The table below gives an overview of all keywords related to working with multiple sediment fractions and bed layers:

Table A.1	Overview of all keywords related to the sediment input

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| D15 | D15 grain size per grain type | 0.00015 | 0.0001-0.0008 | m |  |
| D50 | D50 grain size per grain type | 0.0002 | 0.0001-0.0008 | m |  |
| D90 | D90 grain size per grain type | 0.0003 | 0.0001-0.0015 | m |  |
| dzg+ | Thickness of top sediment class layers | 0.1 | 0.01 - 1.0 | m |  |
| dzg2+ | Nominal thickness of variable sediment class layer | 0.1 | 0.01 - 1.0 | m |  |
| dzg3+ | Thickness of bottom sediment class layers | 0.1 | 0.01 - 1.0 | m |  |
| nd+ | Number of computational layers in the bed | 3 | 3 - 1000 | - |  |
| ngd | Number of sediment classes | 1 | 1 - 20 | - |  |
| por | Porosity | 0.4 | 0.3 - 0.5 | - |  |
| rhos | Solid sediment density (no pores) | 2650.0 | 2400.0 - 2800.0 | kgm^-3 |  |
| sedcal+ | Sediment transport calibration coefficient per grain type | 1 | None | - |  |
| ucrcal+ | Critical velocity calibration coefficient per grain type | 1 | None | - |  |

## Vegetation input

Short wave dissipation, long wave dissipation and flow interaction due to vegetation is supported. The user can define multiple vegetation species. The number of vegetation species is set by the keyword nveg. Furthermore, two files should be created and specified in the params.txt-file: a vegetation characteristics file (keyword veggiefile) and a vegetation location file (keyword veggiemapfile).

The veggiefile is a text file listing the names of the vegetation characteristics files that should be created for every individual vegetation species that should be accounted for. These property files contain the vegetation parameters nsec, ah, Cd, bv and N that represent the number of vertical sections, height of vegetation section relative to the bed , the drag coefficient, stem diameter and vegetation density per vegetation section, respectively. An example of a set of files describing two different vegetation species is given below.

veggiefile.txt

seagrass.txt

mangrove.txt

seagrass.txt

ah = 0.2

Cd = 1.0

bv = 0.02

N = 1200

mangrove.txt

nsec = 3

ah = 0.5 0.8 1.3

Cd = 2.0 1.0 2.0

bv = 0.05 0.15 0.1

N = 1000 50 500

The nsec keyword in the species property file allows the user to define multiple height segments of the species with different properties. The height per vegetation section is defined relative to the bed level. For all properties, the values are given from bottom to top. A definition sketch is given in Figure A.1.

Figure 4.3	Definition sketch of vegetation specification in XBeach (example for mangrove type vegetation schematized in three vertical sections).

Finally,  the veggiemapfile indicates in what grid cell which vegetation species can be found. The format of this file is similar to the bathymetry files described in Section 4.3 (Grid and bathymetry), but the values are integers referring to a species where 1 refers to the first listed species, 2 to the second, et cetera. A zero indicates no vegetation at that particular location.

In summary, the following files should be created when the effect of vegetation is modeled:

- 1 x Veggiemapfile: file similar to bathymetry file containing 0, 1 etc. (up to nveg)

- 1 x Veggiefile: list of file names of vegetation property files per species

- Nveg x vegetation property file(s): describing vegetation properties per species

Below the relevant keywords in the params.txt are given. In addition, the keyword vegetation should be set to 1.

Table A.1	Overview of all keywords related to the vegetation module

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| nveg | Number of vegetation species | -123 |  | - |  |
| veggiefile | Name of vegetation species list file |  |  | <file> |  |
| veggiemapfile | Name of vegetation species map file |  |  | <file> |  |

## Discharge input

Discharge of water at the model boundaries or directly in the model domain is defined along specific grid sections. The keywords ndischarge and ntdischarge define the number of discharge sections and the length of the discharge time series respectively. The disch_loc_file keyword references a file that defines the discharge sections. Each line in this file corresponds to a grid section and each line contains four numbers being the start and end coordinates of the section. The file is formatted as follows, where ndisch refers to the keyword ndischarge:

disch_loc.txt

<x_start 1> <y_start 1> <x_end 1> <y_end 1>

<x_start 2> <y_start 2> <x_end 2> <y_end 2>

<x_start 3> <y_start 3> <x_end 3> <y_end 3>

...

<x_start ndisch> <y_start ndisch> <x_end ndisch> <y_end ndisch>

The world coordinates specified in this file must be chosen such that they are close to the desired grid cell borders, since the grid cell borders are eventually used as discharge section. Discharge sections can be located along grid cell borders that are either oriented cross-shore or alongshore, but not a combination of the two. In a regular grid this implies that either the start or end x-coordinates are equal, or the start and end y-coordinates are equal. Alternatively, both are equal. In this case a vertical discharge from above is assumed, rather than a horizontal discharge. Vertical discharges only add mass and no momentum to the water body.

The keyword disch_time series_file references a file defining the time series imposed on the discharge locations. The file lists the timings in the first column and a discharge value in m3/s for each discharge section as follows, where ntdisch refers to the keyword ntdischarge:

disch_time series.txt

<t 1> <Q 1,1> <Q 2,1> ... <Q ndisch,1>

<t 2> <Q 1,2> <Q 2,2> ... <Q ndisch,2>

<t 3> <Q 1,3> <Q 2,3> ... <Q ndisch,3>

...

<t ntdisch> <Q 1,ntdisch> <Q 2,ntdisch> ... <Q ndisch,ntdisch>

Discharges defined at the domain borders are positive in direction towards the domain (influx). Discharges defined in the domain itself are positive in direction of the positive x or y direction. Vertical discharges are positive into the domain (influx).

Figure 4.4	Possible discharge orifices. A discharge orifice is defined as line in between two points (red). The resulting discharge orifice constitutes out of full abreast grid cells (green). The discharge direction is in positive grid direction (s or n), except at the domain border where the discharge is an influx in the domain (green arrows).

The table below gives an overview of all keywords related to discharges:

Table A.1	Overview of all keywords related to the discharge input

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| disch_loc_file+ | Name of discharge locations file |  |  | <file> |  |
| disch_time series_file+ | Name of discharge time series file |  |  | <file> |  |
| ndischarge+ | Number of discharge locations |  | 0 - 100 | - |  |
| ntdischarge+ | Length of discharge time series |  | 0 - 100 | - |  |

## Drifters input

Drifters can be deployed during the model simulation by specifying the number of drifters using the keyword ndrifter and the location, start and end time of the drifter deployment in a separate file referenced by the drifterfile keyword. The file format is as follows:

drifter.txt

<x 1> <y 1> <t_start 1> <t_end 1>

<x 2> <y 2> <t_start 2> <t_end 2>

<x 3> <y 3> <t_start 3> <t_end 3>

...

<x ndrifter> <y ndrifter> <t_start ndrifter> <t_end ndrifter>

The table below gives an overview of all keywords related to drifters:

Table A.1	Overview of all keywords related to drifters

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| drifterfile | Name of drifter data file |  |  | <file> |  |
| ndrifter | Number of drifters | ndrifter | 0 - 50 | - |  |

## Ship-induced wave motions

Ship waves can be simulated by defining the ships' geometries and trajectories in a collection of files. The user can define multiple ships. The number of ships is set by the keyword nship. In the file referenced by the keyword shipfile each ship is given a name. The properties of each ship are summarized in another textfile with the name of the ship (shipname.txt). This properties file defines the parameters for the discretization of the ships geometry. The ship grid is determined by the keywords dx, dy, nx, and ny, and the ship geometry itself is given in a separate file, referenced bythe keyword shipgeom. This file contains the ship draft per ship grid point, and should have a size of nx+1 by ny+1. The center of gravity of the ship is also defined in the ship properties file using the keywords xCG, yCG and zCG. The ships trajectory is defined in a file referenced from the ship properties file by the keyword shiptrack. Each row in this file contains a time, x- and y-coordinate indicating the ships  position as function of time.

To avoid numerical problems, the full ship track should be within the model domain (i.e. a vessel cannot sail through the model boundary). Furthermore, it is advised to start the ship track with a very low velocity and gradually increase the sailing speed. When not taking into account such spin up period, the impact of the ship on the water level can be too abrupt, resulting in unrealistic wave patterns. In addition, it is advised to maintain a relatively deep edge of a few grid cells at both model boundaries (i.e. front and back side).

Another way to avoid numerical issues at the initialization of a ship simulation is to use the 'flying' option, which can be specified in the ship file (flying = 1). In case the option flying is enabled, also a z-coordinate is defined in the shiptrack-file indicating the vertical position of the ship. In this way, the vessel can ‘land’ on the water with its correct sailing speed, thereby avoiding unwanted disturbance to the water level. Also, the ship can ‘fly out’ of the model domain before reaching the back boundary. By using this method the spin-up time can be reduced considerably and both inflow and outflow boundaries are unaffected by the ship

The two keywords compute_force and compute_motion enable the computation of forces on the ship and the ships motions due to wave forcing respectively; the latter has not been implemented yet. Forces on a ship in motion may be unreliable due to near-field effects; forces on a ship at rest are much more reliable. An example of ship definition files is:

shipfile.txt

containership.txt

oiltanker.txt

containership.txt

dx = 10

dy = 10

nx = 30

ny = 10

shipgeom = container_geom.dep

xCG = 120

yCG = 50

zCG = 30

shiptrack = container_track.txt

flying = 1

compute_force = 1

compute_motion = 1

pannamax_geom.txt

<z 0,0> <z 1,0> <z 2,0> <z 3,0> ... <z nx,0> <z nx+1,0>

<z 0,1> <z 1,1> <z 2,1> <z 3,1> ... <z nx,1> <z nx+1,1>

...

<z 0,ny> <z 1,ny> <z 2,ny> <z 3,ny> ... <z nx,ny> <z nx+1,ny>

<z 0,ny+1> <z 1,ny+1> <z 2,ny+1> <z 3,ny+1> ... <z nx,ny+1> <z nx+1,ny+1>

pannamax_track.txt

<t 1> <x 1> <y 1> <z 1>

<t 2> <x 2> <y 2> <z 2>

<t 3> <x 3> <y 3> <z 3>

...

oiltanker.txt

dx = 2

dy = 2

nx = 20

ny = 4

shipgeom = tanker_geom.dep

xCG = 20

yCG = 40

zCG = 1.5

shiptrack = tanker_track.txt

flying = 0

tanker_track.txt

<t 1> <x 1> <y 1>

<t 2> <x 2> <y 2>

<t 3> <x 3> <y 3>

...

Table A.1	Overview of all keywords related to the ship module

| Keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| nship* | Number of ships | -123 |  | - |  |
| shipfile | Name of ship data file |  |  | <file> |  |

## Output selection

Output selection determines what data computed by XBeach is written to a file in terms of location and time and in what format. The output types, output times and output formats supported by XBeach are explained in more detail in the following subsections. The table below gives an overview of all keywords related to model output:

Table A.1	Overview of all keywords related to the output definitions

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| globalvars+ | Mnems of global output variables, not per se the same size as nglobalvar (invalid variables, defaults) | 'abc' |  | - |  |
| meanvars+ | Mnems of mean output variables (by variables) | 'abc' |  | - |  |
| ncfilename+ | Xbeach netcdf output file name |  |  | <file> |  |
| ncross+ | Number of output cross sections | 0 | 0 - 50 | - |  |
| nglobalvar | Number of global output variables (as specified by user) | -1 | -1 - 20 | - |  |
| nmeanvar | Number of mean, min, max, var output variables | 0 | 0 - 15 | - |  |
| npoints | Number of output point locations | 0 | 0 - 50 | - |  |
| npointvar | Number of point output variables | 0 | 0 - 50 | - |  |
| nrugauge | Number of output runup gauge locations | 0 | 0 - 50 | - |  |
| nrugdepth+ | Number of depths to compute runup in runup gauge | 1 | 1 - 10 | - |  |
| outputformat+ | Output file format | fortran | fortran, netcdf, debug |  |  |
| pointvars+ | Mnems of point output variables (by variables) | 'abc' |  | - |  |
| rugdepth+ | Minimum depth for determination of last wet point in runup gauge | 0 | 0 - 0.1 | m |  |
| timings+ | Switch enable progress output to screen | 1 | 0 - 1 | - |  |
| tintc+ | Interval time of cross section output | 1.0 | 0.01 - 100000.0 | s |  |
| tintg | Interval time of global output | 1.0 | 0.01 - 100000.0 | s |  |
| tintm | Interval time of mean, var, max, min output | tstop- tstart | 1 – [tstop – start] | s |  |
| tintp | Interval time of point and runup gauge output | 1.0 | 0.01 - 100000.0 | s |  |
| tscross+ | Name of file containing timings of cross section output | None | None - None | - |  |
| tsglobal+ | Name of file containing timings of global output | None | None - None | - |  |
| tsmean+ | Name of file containing timings of mean, max, min and var output | None | None - None | - |  |
| tspoints+ | Name of file containing timings of point output | None | None - None | - |  |
| tstart | Start time of output, in morphological time | 1.0 | 0.0 - 1000000.0 | s |  |

### Output types

XBeach supports four different types of output: 1) instantaneous spatial output 2) time-averaged spatial output 3) fixed point output or 4) runup gauge output. In principle any variable in XBeach can be outputted as long as it is part of the spaceparams structure defined in spaceparams.tmpl in the XBeach source code. An overview of all currently supported parameters in this file is presented in Table A.1.

The amount of output variables used for each type is determined by the keywords nglobalvar, nmeanvar, npoints and nrugauge. Each of these keywords takes a number indicating the number of parameters or locations that should be written to file. If any of the keywords is set to zero, the output type is effectively disabled. If nglovalvar is set to -1 then a standard set of output variables is used, being H, zs, zs0, zb, hh, u, v, ue, ve, urms, Fc, Fy, ccg, ceqsg, ceqbg, Susg, Svsg, E, R, D and DR. If nglobalvar is not set it defaults to -1. The lines in the params.txt file immediately following these keywords determine what parameters or locations are used, as will be explained in more detail in the following subsections.

Table A.1	Possible output parameters

| Name | Explanation | Unit |
| --- | --- | --- |
| As | asymmetry of short waves | [-] |
| bi | incoming bound long wave | [m] |
| BR | maximum wave surface slope used in roller dissipation formulation | [-] |
| c | wave celerity | [m/s] |
| ccbg | depth-averaged bed concentration for each sediment fraction | [m3/m3] |
| ccg | depth-averaged suspended concentration for each sediment fraction | [m3/m3] |
| cctot | Sediment concentration integrated over bed load and suspended and for all sediment grains | [m3/m3] |
| ceqbg | depth-averaged bed equilibrium concentration for each sediment class | [m3/m3] |
| ceqsg | depth-averaged suspended equilibrium concentration for each sediment class | [m3/m3] |
| cf | Friction coefficient flow | [-] |
| cg | group velocity | [m/s] |
| cgx | group velocity, x-component | [m/s] |
| cgy | group velocity, y-component | [m/s] |
| costh | cos of wave angles relative to grid direction | [-] |
| ctheta | wave celerity theta-direction (refraction) | [rad/s] |
| cx | wave celerity, x-component | [m/s] |
| cy | wave celerity, y-component | [m/s] |
| D | dissipation | [W/m2] |
| D50 | D50 grain diameters for all sediment classes | [m] |
| D50top | Friction coefficient flow | [-] |
| D90 | D90 grain diameters for all sediment classes | [m] |
| D90top | Friction coefficient flow | [-] |
| Dc | diffusion coefficient | [m2/s] |
| dcbdx | bed concentration gradient x-dir. | [kg/m3/m] |
| dcbdy | bed concentration gradient y-dir. | [kg/m3/m] |
| dcsdx | suspended concentration gradient x-dir. | [kg/m3/m] |
| dcsdy | suspended concentration gradient y-dir. | [kg/m3/m] |
| depo_ex | explicit bed deposition rate per fraction | [m/s] |
| depo_im | implicit bed deposition rate per fraction | [m/s] |
| Df | dissipation rate due to bed friction | [W/m^2] |
| dinfil | Infiltration layer depth used in quasi-vertical flow model for groundwater | [m] |
| dnc | grid distance in n-direction, centered around c-point | [m] |
| dnu | grid distance in n-direction, centered around u-point | [m] |
| dnv | grid distance in n-direction, centered around v-point | [m] |
| dnz | grid distance in n-direction, centered around z-point (=eta-point) | [m] |
| Dp | dissipation rate in the swash due to transformation of kinetic wave energy to potential wave energy | [W/m^2] |
| DR | roller energy dissipation | [W/m2] |
| dsc | grid distance in s-direction, centered around c-point | [m] |
| dsdnui | inverse of grid cell surface, centered around u-point | [1/m2] |
| dsdnvi | inverse of grid cell surface, centered around v-point | [1/m2] |
| dsdnzi | inverse of grid cell surface, centered around z-point | [1/m2] |
| dsu | grid distance in s-direction, centered around u-point | [m] |
| dsv | grid distance in s-direction, centered around v-point | [m] |
| dsz | grid distance in s-direction, centered around z-point (=eta-point) | [m] |
| dzav | total bed level change due to avalanching | [m] |
| dzbdt | rate of change bed level | [m/s] |
| dzbdx | bed level gradient in x-direction | [-] |
| dzbdy | bed level gradient in y-direction | [-] |
| dzbed | bed level gradient | [-] |
| dzsdt | rate of change water level | [m/s] |
| dzsdx | water surface gradient in x-direction | [m/s] |
| dzsdy | water surface gradient in y-direction | [m/s] |
| E | wave energy | [Nm/m2] |
| ee | directionally distributed wave energy | [J/m2/rad] |
| ero | bed erosion rate per fraction | [m/s] |
| Fx | wave force, x-component | [N/m2] |
| Fy | wave force, y-component | [N/m2] |
| gw0back | boundary condition back boundary for groundwater head | [m] |
| gwbottom | level of the bottom of the aquifer | [m] |
| gwhead | groundwater head (differs from gwlevel) | [m] |
| gwheight | vertical size of aquifer through which groundwater can flow | [m] |
| gwlevel | groundwater table | [m] |
| gwu | groundwater flow in x-direction | [m/s] |
| gwv | groundwater flow in y-direction | [m/s] |
| gww | groundwater flow in z-direction (interaction between surface and ground water) | [m/s] |
| H | Hrms wave height based on instantaneous wave energy | [m] |
| hh | water depth | [m] |
| hold | water depth previous time step | [m] |
| hu | water depth in u-points | [m] |
| hum | water depth in u-points | [m] |
| hv | water depth in v-points | [m] |
| hvm | water depth in v-points | [m] |
| idrift | Drifter x-coordinate in grid space | [-] |
| jdrift | Drifter y-coordinate in grid space | [-] |
| k | wave number | [rad/m] |
| kb | near bed turbulence intensity due to depth induces breaking | [m^2/s^2] |
| kturb | depth averaged turbulence intensity due to long wave breaking | [m^2/s^2] |
| L1 | wave length (used in dispersion relation) | [m] |
| maxzs | maximum elevation in simulation | [m] |
| minzs | minimum elevation in simulation | [m] |
| n | ratio group velocity/wave celerity | [-] |
| nd | number of bed layers (can be different for each computational cell) | [-] |
| ndist | cum. distance from right boundary along n-direction | [m] |
| nuh | horizontal viscosity coefficient | [m2/s] |
| pdisch | Discharge locations | [-] |
| ph | pressure head due to ship | [m] |
| pntdisch | Point discharge locations (no momentum) | [-] |
| pres | normalized dynamic pressure | [m^2/s^2] |
| Qb | fraction breaking waves | [-] |
| qdisch | Discharges | [m^2/s] |
| qx | discharge in u-points, x-component | [m2/s] |
| qy | discharge in u-points, y-component | [m2/s] |
| R | roller energy | [Nm/m2] |
| rolthick | long wave roller thickness | [m] |
| rr | directionally distributed roller energy | [J/m2/rad] |
| sdist | cum. distance from offshore boundary along s-direction | [m] |
| sedero | cum. sedimentation/erosion | [m] |
| sigm | mean frequency | [rad/s] |
| sigt | relative frequency | [rad/s] |
| sinth | sin of wave angles relative to grid direction | [-] |
| Sk | skewness of short waves | [-] |
| structdepth | Depth of structure in relation to instantaneous bed level | [m] |
| Subg | bed sediment transport for each sediment class (excluding pores), x-component | [m2/s] |
| Susg | suspended sediment transport for each sediment class (excluding pores), x-component | [m2/s] |
| Sutot | Sediment transport integrated over bed load and suspended and for all sediment grains, x-component | [m2/s] |
| Svbg | bed sediment transport for each sediment class (excluding pores), y-component | [m2/s] |
| Svsg | suspended sediment transport for each sediment class (excluding pores), y-component | [m2/s] |
| Svtot | Sediment transport integrated over bed load and suspended and for all sediment grains, y-component | [m2/s] |
| Sxx | radiation stress, x-component | [N/m] |
| Sxy | radiation stress, y-component | [N/m] |
| Syy | radiation stress, y-component | [N/m] |
| taubx | bed shear stress, x-component | [N/m^2] |
| tauby | bed shear stress, y-component | [N/m^2] |
| Tbore | wave period interval associated with breaking induced turbulence | [s] |
| tdisch | Discharge time series | [-] |
| tdrifter | Drifter retrieval time | [s] |
| theta | wave angles | [rad] |
| theta | wave angles directional distribution w.r.t. comp. x-axis | [rad] |
| theta0 | mean incident wave angle | [rad] |
| thetamax | minimum angle of computational wave grid (cart. in rad) | [rad] |
| thetamean | mean wave angle | [rad] |
| thetamin | minimum angle of computational wave grid (cart. in rad) | [rad] |
| tideinpt | input time of input tidal signal | [s] |
| tideinpz | input tidal signal | [m] |
| tidelen | length of tide time series | [-] |
| tm | mean wave direction | [rad] |
| Tsg | sediment response time for each sediment class | [s] |
| u | GLM velocity in cell center, x-component | [m/s] |
| ua | time averaged flow velocity due to wave asymmetry | [m/s] |
| ucrcal | calibration factor for u critical for each sediment class | [-] |
| ue | Eulerian velocity in cell center, x-component | [m/s] |
| ueu | Eulerian velocity in u-points, x-component | [m/s] |
| ui | incident bound wave velocity in, x-component | [m/s] |
| umean | long-term mean velocity at bnds in u-points, x-component | [m/s] |
| umwci | velocity (time-averaged) for wci, x-component | [m/s] |
| ur | reflected velocity at bnds in u-points | [m/s] |
| urepb | representative flow velocity for sediment advection and diffusion, x-component | [m/s] |
| ureps | representative flow velocity for sediment advection and diffusion, x-component | [m/s] |
| urms | orbital velocity | [m/s] |
| usd | return flow due to roller after breaker delay | [m/s] |
| ust | Stokes drift | [m/s] |
| ustr | return flow due to roller | [m/s] |
| uu | GLM velocity in u-points, x-component | [m/s] |
| uv | GLM velocity in v-points, x-component | [m/s] |
| uwf | Stokes drift, x-component | [m/s] |
| v | GLM velocity in cell center, y-component | [m/s] |
| vardx | 0 = uniform grid size, 1 = variable grid size | [-] |
| ve | Eulerian velocity in cell center, y-component | [m/s] |
| vev | Eulerian velocity in u-points, y-component | [m/s] |
| vi | incident bound wave velocity in, y-component | [m/s] |
| vmag | velocity magnitude in cell center | [m/s] |
| vmageu | Eulerian velocity magnitude u-points | [m/s] |
| vmagev | Eulerian velocity magnitude v-points | [m/s] |
| vmagu | GLM velocity magnitude u-points | [m/s] |
| vmagv | GLM velocity magnitude v-points | [m/s] |
| vmean | long-term mean velocity at bnds in u-points, y-component | [m/s] |
| vmwci | velocity (time-averaged) for wci, y-component | [m/s] |
| vrepb | representative flow velocity for sediment advection and diffusion, y-component | [m/s] |
| vreps | representative flow velocity for sediment advection and diffusion, y-component | [m/s] |
| vu | GLM velocity in u-points, y-component | [m/s] |
| vv | GLM velocity in v-points, y-component | [m/s] |
| vwf | Stokes drift, y-component | [m/s] |
| wb | vertical velocity at the bottom | [m/s] |
| wetu | mask wet/dry u-points | [-] |
| wetv | mask wet/dry v-points | [-] |
| wetz | mask wet/dry eta-points | [-] |
| wi | Vertical velocity at boundary due to (short) waves | [m/s] |
| winddirts | input wind direction | [deg nautical] |
| windinpt | input time of input wind signal | [s] |
| windlen | length of tide time series | [-] |
| windnv | wind velocity in N direction in v point at current time step | [m/s] |
| windsu | wind velocity in S direction in u point at current time step | [m/s] |
| windvelts | input wind velocity | [m/s] |
| windxts | time series of input wind velocity (not S direction), x-component | [m/s] |
| windyts | time series of input wind velocity (not N direction), y-component | [m/s] |
| wm | mean abs frequency | [rad/s] |
| ws | vertical velocity at the free surface | [m/s] |
| zb | bed level | [m] |
| zb0 | initial bed level | [m] |
| zi | Surface elevation at boundary due to (short) waves | [m] |
| zs | water level | [m] |
| zswci | water level (time-averaged) for wci | [m] |

#### Instantaneous spatial output

Instantaneous spatial output describes the instantaneous state of variables across the entire model domain at various points in time. To make use of this option the user must specify the output time (see Section 4.13.2 under the keywod tintg) and the number of output variables required using the nglobalvar keyword in params.txt, immediately followed by the names of the requested variables on a separate line each. The output of three instantaneous grids can look as follows:

params.txt

nglobalvar = 3

zs

zb

H

#### Time-averaged spatial output

Time-averaged spatial output describes the time-averaged state of variables across the entire model domain at various points in time. The user can define the averaging period in params.txt. To make use of this option the user must specify the output time (see Section 4.13.2 under tintm) and the number of output variables required using the nmeanvar keyword in params.txt, immediately followed by the names of the requested variables on a separate line each. The output of two time-averaged grids may look as follows:

params.txt

nmeanvar = 2

u

v

#### Fixed point output

Fixed point output allows the user to select one or more locations for which a time series of data is stored. This output describes a time-series of one or more variables at one point in the model domain. To make use of this option, the user must specify the number of output locations using the npoints keyword in params.txt, immediately followed by one line per output location describing the location coordinates given as the x-coordinate and y-coordinate and in world coordinates. XBeach will link the output location to the nearest computational point.

The user can specify the number and selection of output variables for all points (and runup gauges, discussed in the following section) using the npointvar keyword in params.txt to specify the number of output variables, immediately followed by the names of the requested variables on a separate line each. Fixed point output significantly reduces the amount of data written to file in each time step and is therefore particularly suitable for high temporal resolution output.

An example with two output locations is given below. The first point is located on the offshore boundary (x = 0.0) and somewhere in the middle of the model domain in y-direction (y = 800.0). The second point is located on the lateral boundary (y = 1600.0) and somewhere in the middle of the domain in x-direction (x = 2000.0). Both locations have four output variables: H, zs, zb and D.

params.txt

npoints = 2

0. 800.

2000. 1600.

npointvar = 4

H

zs

zb

D

#### Runup gauge output

Runup gauge output describes a time-series of a number of variables at the (moving) waterline. In this case XBeach scans in an x-directional transect defined by the user for the location of the waterline. Output information is recorded for this moving point. This is particularly useful to keep track of runup levels in cross-shore transects.

The definition of runup gauges is similar to the definition of fixed point output. The user needs to specify the number of runup gauges using the nrugauge keyword in params.txt, immediately followed by one line per runup gauge location describing the coordinates of the initial location of the runup gauge. XBeach will subsequently link the initial runup gauge location to the nearest computational cross-shore transect rather than just the nearest computational point. It is also possible for the user to set a minimum depth for determination of the last wet point in the runup gauge (keyword: rugdepth).

Runup gauges share their selection of output variables with regular point output. However, in the case of runup gauges, XBeach will automatically also include the variables xw, yw and zs to the point output variables, if these variables were not specified using the npointvar keyword in params.txt. Note that the user should refer to the pointvars.idx output file to check order of output variables for points and runup gauges.

An example of a runup gauge input is given below. The runup gauge is initially located on the offshore boundary (x = 0.0) and somewhere in the middle of the model domain in y-direction (y = 800.0). The runup gauge will display standard output variables (xw, yw and zs, as well as any output variables specified by the npointvar keyword).

params.txt

nrugauge = 1

0. 800.

### Output times

The user may determine the output times for regular spatial output variables, time averaged spatial variables and point location variables individually. Runup gauge output and fixed point output are given at the same moments in time. For all three types of output the user may choose to either state a fixed interval time at which output is given or supply an external file containing times at which output should be given or a combination of both.

#### Output at fixed intervals

The user should define a point in time after the start of the simulation at which the first output is generated for fixed interval output. The user can do this by using the tstart keyword in params.txt. All output that is being generated at fixed intervals uses tstart as their base. The interval for instantaneous spatial output is given by the tintg keyword. The keywords for the interval of time-averaged spatial output and point output are tintm and tintp respectively, where tintp is used both for fixed point and runup gauge output. Note that tintg, tintm and tintp supersede the older tint parameter that is valid for all types of output. The default value of tintg is one second. If tintp or tintm is not stated, but output is declared (npoints, nrugauge or nmeanvar is stated larger than zero), XBeach assumes the same output interval as tintg. An example of the definition of fixed intervals is given below.

params.txt

tstart = 100.

tintg = 100.

tintp = 2.

tintm = 3600.

In the case of instantaneous spatial output and point output, the first output is given at tstart. In the case of time-averaged spatial variables, the first output is given at tstart+tintm. This output represents the average condition over the interval between tstart and tstart+tintm.

#### Output times defined by external file

The user is given the option to have output at a set of points in time that are not separated by regular intervals. In this case the user must supply an additional file for each output type. The user specifies the name of the output time series file for instantaneous spatial output using the tsglobal keyword. The keywords for time series files for time-averaged spatial output and point output are tsmean and tspoint respectively, where tspoint is again used for both fixed point and runup gauge output. All time series files must contain on the first line the number of output times followed by every output time on a new line. An example of such irregular output time definition is given below.

params.txt

tsglobal= time series1.txt

tspoints = time series2.txt

tsmean= time series3.txt

time series1.txt

18

0.05

0.15

0.2

0.8

12.0

12.5

19.124

30.

60.

90.

120.

150.

160.

170.

177.

178.

179.

180.

In the case of instantaneous spatial output and point output, the first output is given at the first stated point in time. In the case of time-averaged spatial variables, the first output is given at the second stated point in time. This output represents the average condition over the interval between first and second stated point in time. Subsequent averaging is done over every interval.

#### Combinations of fixed internal and external files

The user is allowed to define certain types of output using fixed intervals and others using external files. The use of an external file supersedes the use of fixed intervals. Note that tstart will only apply to output of fixed interval type. An example of mixing fixed and varying output time intervals is given below.

param.txt

tstart = 100.

tintg = 100.

tspoints = time series2.txt

tintm = 3600.

### Output format

XBeach supports two types of output: 1) Fortran binary and 2) netCDF. The output format used is determined by the keyword outputformat. The use of netCDF output is more convenient since all output (and input) is stored in a single, easy accessible file. Also the netCDF file format is compatible with many programming languages (e.g. Matlab, Python) as well as many visualization tools (e.g. QuickPlot, Morphan). It should be noted that the support for output types in netCDF could be limited for recent functionalities of the XBeach model.

#### Fortran binary

Output files in Fortran binary format are bare matrix dumps of XBeach’ computational matrices. At each output time, one such matrix block is added to the output file. These files can generally be read by binary read functions, like fread in Matlab and the struct package in Python.

Output files written in Fortran binary format are given the name <variable>.dat, for example zs.dat, for instantaneous spatial output. The only exception is that files containing information about the wave height of the short waves are called hrms.dat instead of H.dat to maintain backward compatibility. Time-averaged spatial output is stored similarly, but the file names have a suffix indicating the type of averaging <variable>_mean.dat. For time-averaged spatial output also the variance, minimum and maximum values are stored using the suffixes _var, _min and _max respectively.

All data corresponding to fixed point locations will be stored in files called point<NNN>.dat. <NNN> represents a number between 001 and 999 corresponding to the order in which the points are declared in params.txt. The data files are plain text and contain one row for each output time step. The first position on each row is the time at which the output is given. The subsequent positions in the row are the instantaneous values of the variables at the given point. The order of the variables is equal to the order in which they are defined for that point in params.txt. Data corresponding to runup gauge locations are stored in the same format as fixed point output, but the files are named rugau<NNN>.dat.

An extra file called dims.dat is always written at the start of the simulation in Fortran binary output mode. This file contains the dimensions of the XBeach model. It simply states the following dimensions in order: nt (number of output time steps), nx (number of grid cells in x-direction), ny (number of grid cells in y-direction), ngd (number of sediment fractions), nd (number of bed layers), ntp (number of point output time steps), ntm (number of time-averaged output time steps). Subsequently, the irregular time series are stored, if applicable: tsglobal (irregular output times), tspoints (irregular point output times) and tsmean (irregular time-averaged output times). Similarly, a file xy.dat is written containing the x- and y- coordinates of the full computational grid.

#### netCDF

All data in netCDF output is stored in a single output file. By default this file is named xboutput.nc, but this name can be chosen freely using the keyword ncfilename. The netCDF file holds all output data, dimensions and input data in a single file. It should be noted that netCDF files can hold a multiple time axes. The temporal unit can be specified in the params.txt file using the keyword tunits. This unit does not affect calculations and is only used for output. An example of the layout of the netcdf file is given below:

xboutput.nc (structure only, no real contents)

netcdf xboutput {

dimensions:

x = 565 ;

y = 101 ;

wave_angle = 9 ;

bed_layers = 3 ;

sediment_classes = 1 ;

inout = 2 ;

globaltime = 2 ;

tidetime = 435 ;

tidecorners = 2 ;

windtime = 2 ;

variables:

double x(x) ;

x:units = "m" ;

x:long_name = "local x coordinate" ;

double y(y) ;

y:units = "m" ;

y:long_name = "local y coordinate" ;

double globaltime(globaltime) ;

globaltime:units = "s" ;

double H(globaltime, y, x) ;

H:units = "m" ;

H:long_name = "wave height" ;

double zs(globaltime, y, x) ;

zs:units = "m" ;

zs:long_name = "water level" ;

double zb(globaltime, y, x) ;

zb:units = "m" ;

zb:long_name = "bed level" ;

double ue(globaltime, y, x) ;

ue:units = "m/s" ;

## Time parameters

In all XBeach simulations the hydrodynamic simulation starts at time 0. Model output can be postponed until the time specified by the keyword tstart (see 4.13.2). The time step of the simulation is determined based on a given maximum Courant number using the keyword CFL. The table below gives an overview of all keywords related to time management:

Table A.1	Overview of all keywords related to time management

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| CFL | Maximum Courant-Friedrichs-Lewy number | 0.7 | 0.1 - 0.9 | - |  |
| tstop | Stop time of simulation, in morphological time | 2000.0 | 1.0 - 1000000.0 | s |  |
| tunits+ | Time units in udunits format (seconds since 1970-01-01 00:00:00.00 +1:00) | 's' |  | - |  |

# Bibliography

Andrews, D. G., & Mcintyre, M. E. (1978). An exact theory of nonlinear waves on a Lagrangian-mean flow. Journal of Fluid Mechanics, 89, 609. doi:10.1017/S0022112078002773

Baldock, T. E., Holmes, P., Bunker, S., & van Weert, P. (1998). Cross-shore hydrodynamics within an unsaturated surfzone. Coastal Engineering, 34, 173–196.

Battjes, J. A. (1975). Modelling of turbulence in the surfzone. Symposium on Modelling Techniques, San Francisco, 1050–1061.

Daly, C., Roelvink, J. A., van Dongeren, A. R., & McCall, R. T. (2010). Short wave breaking effects on law frequency waves. Proceedings 30th International Conference on Coastal Engineering, San Diego, 1–13.

Daly, C., Roelvink, J. A., van Dongeren, A. R., van Thiel de Vries, J. S. M., & McCall, R. T. (2012). Validation of an advective-deterministic approach to short wave breaking in a surf-beat model. Coastal Engineering, 60, 69–83. doi:10.1016/j.coastaleng.2011.08.001

Darcy, H. (1856). Les fontaines publiques de la ville de dijon. Tech. Rep., Dalmont, Paris.

De Jong, M. P. C., Roelvink, J. A., & Breederveld, C. (2013). Numerical modelling of passing-ship effects in complex geometries and on shallow water. Pianc Smart Rivers 2013.

De Vet, P. L. M. (2014). Modelling sediment transport and morphology during overwash and breaching events. MSc thesis, Delft University of Technology, Delft.

Deigaard, R. (1993). A note on the three-dimensional shear stress distribution in a surf zone. Coastal Engineering, 20, 157–171. doi:10.1016/0378-3839(93)90059-H

Deltares. (2011). Delft3D-FLOW: user manual.

Den Adel, H. (1987). Heranalyse doorlatendheidsmetingen door middel van de forchheimer relatie. Technical Report M 1795/H 195, CO 272550/56, Grondmechanica Delft, Waterloopkundig Laboratorium. Dutch.

Galappatti, R., & Vreugdenhill, C. B. (1985). A depth integrated model for suspended transport. Journal for Hydraulic Research, 23(4), 359–377.

Guza, R. T., & Thornton, E. B. (1985). Velocity moments in the nearshore. Coastal Engineering, 111(2), 235–256.

Halford, K. (2000). Simulation and interpretation of borehole flowmeter results under laminar and turbulent flow conditions. Proceedings of the Seventh International Symposium on Logging for Minerals and Geotechnical Applications, Golden, Colorado, The Minerals and Geotechnical Logging Society, 157–168.

Hallermeier, R. J. (1981). Terminal settling velocity of commonly occurring sand grains. Sedimentology, 28, 859–865.

Harbaugh, A. W. (2005). MODFLOW-2005 , The USGS Modular Ground-Water Model. U.S. Geological Survey Techniques and Methods, 253.

Hirsch, C. (2007). Numerical computation of internal and external flows. New York John Wiley & Sons.

Holthuijsen, L. H., Booij, N., & Herbers, T. H. C. (1989). A prediction model for stationary, short-crested waves in shallow water with ambient currents. Coastal Engineering, 13(1), 23–54. doi:10.1016/0378-3839(89)90031-8

Janssen, T. T., & Battjes, J. A. (2007). A note on wave energy dissipation over steep beaches. Coastal Engineering, 54, 711–716. doi:10.1016/j.coastaleng.2007.05.006

Komar, P. D., & Miller, M. C. (1975). On the comparison between the threshold of sediment motion under waves under unidirectional currents with a discussion of the practical evaluation of the threshold. Journal of Sedimentary Research, 362–367.

Lam, D. C. L., & Simpson, R. B. (1976). Centered differencing and the box scheme for diffusion convection problems. Journal of Computational Physics, 22, 486–500.

Lee, K. H., Mizutani, N., Hur, D. S., & Kamiya, A. (2007). The effect of groundwater on topographic changes in a gravel beach. Ocean Engineering, 34, 605–615. doi:10.1016/j.oceaneng.2005.10.026

Li, L., & Barry, D. A. (2000). Wave-induced beach groundwater flow. Advances in Water Resources, 23, 325–337. doi:10.1016/S0309-1708(99)00032-9

Longuet-Higgins, M. S., & Stewart, R. W. (1962). Radiation stress and mass transport in gravity waves, with application to “surf beats.” Journal of Fluid Mechanics, 13, 481–504.

Longuet-Higgins, M. S., & Stewart, R. W. (1964). Radiation stress in water waves: a physical discussion with applications. Deep-Sea Research, 529–562.

Longuet-Higgins, M. S., & Turner, J. S. (1974). An “entraining plume” model of a spilling breaker. Journal of Fluid Mechanics, 63(01), 1–20.

Lowe, R. J., Falter, J. L., Koseff, J. R., Monismith, S. G., & Atkinson, M. J. (2007). Spectral wave flow attenuation within submerged canopies: Implications for wave energy dissipation. Journal of Geophysical Research: Oceans, 112, 1–14. doi:10.1029/2006JC003605.

MacCormack, R. W. (1969). The effect of viscocity in hypervecloity impact cratering. AIAA Hyper Velocity Impact Conference, 69–354.

McCall, R. T., Masselink, G., Poate, T. G., Roelvink, J. a., Almeida, L. P., Davidson, M., & Russell, P. E. (2014). Modelling storm hydrodynamics on gravel beaches with XBeach-G. Coastal Engineering, 91, 231–250. doi:10.1016/j.coastaleng.2014.06.007

Mendez, F. J., & Losada, I. J. (2004). An empirical model to estimate the propagation of random breaking and nonbreaking waves over vegetation fields. Coastal Engineering, 51, 103–118. doi:10.1016/j.coastaleng.2003.11.003.

Nairn, R. B., Roelvink, J. A., & Southgate, H. N. (1990). Transition zone width and implications for modeling surfzone hydrodynamics. Proceedings 22th International Conference on Coastal Engineering, 68–81. doi:10.9753/icce.v22.

Nederhoff, C. M., Lodder, Q. J., Boers, M., Den Bieman, J. P., & Miller, J. K. (2015). Modeling the effects of hard structures on dune erosion and overwash - a case study of the impact of Hurricane Sandy on the New Jersey coast. Proceedings Coastal Sediments, San Diego, CA.

Phan, L., van Thiel de Vries, J. S. M., & Stive, M. J. F. (2014). Coastal Mangrove Squeeze in the Mekong Delta. Journal of Coastal Research.

Phillips, O. M. (1977). The dynamics of the upper ocean. Cambridge University Press, 366.

Quataert, E., Storlazzi, C. S., van Rooijen, A. A., Cheriton, O., & van Dongeren, A. R.  The influence of coral reefs and climate change on wave-driven flooding of tropical coastlines. Geophysical Research Letters (in Press).

Raubenheimer, B., Guza, R. T., & Elgar, S. (1999). Tidal water table fluctuations in a sandy ocean beach. Water Resources Research, 35(8), 2313. doi:10.1029/1999WR900105

Reniers, A. J. H. M., MacMahan, J. H., Thornton, E. B., & Stanton, T. P. (2007). Modeling of very low frequency motions during RIPEX. Journal of Geophysical Research: Oceans, 112(February), 1–14. doi:10.1029/2005JC003122

Reniers, A. J. H. M., Roelvink, J. A., & Thornton, E. B. (2004). Morphodynamic modeling of an embayed beach under wave group forcing. Journal of Geophysical Research, 109, 1–22. doi:10.1029/2002JC001586

Rienecker, M. M., & Fenton, J. D. (1981). A Fourier approximation method for steady water waves. Journal of Fluid Mechanics, 104, 119. doi:10.1017/S0022112081002851

Roelvink, J. A. (1993a). Dissipation in random wave group incident on a beach. Coastal Engineering, 19, 127–150.

Roelvink, J. A. (1993b). Surf beat and its effect on cross-shore profiles. PhD thesis, Delft Unversity of Technology, Delft.

Roelvink, J. A. (2006). Coastal morphodynamic evolution techniques. Coastal Engineering, 53, 277–287. doi:10.1016/j.coastaleng.2005.10.015

Roelvink, J. A., & Stive, M. J. F. (1989). Bar-generating cross-shore flow mechanisms on a beach. Journal of Geophysical Research, 94, 4785–4800.

Rowe, P. N. (1987). A convenient empirical equation for estimation of the richardson-zaki exponent. Chemical Engineering Science, 42(11), 2795 – 2796.

Ruessink, B. G., Miles, J. R., Feddersen, F., Guza, R. T., & Elgar, S. (2001). Modeling the alongshore current on barred beaches. Journal of Geophysical Research, 106(22), 451–463.

Ruessink, B. G., Ramaekers, G., & van Rijn, L. C. (2012). On the parameterization of the free-stream non-linear wave orbital motion in nearshore morphodynamic models. Coastal Engineering, 65, 56–63. doi:10.1016/j.coastaleng.2012.03.006

Schroevers, M., Huisman, B. J., van der Wal, A., & Terwindt, J. (2011). Measuring ship induced waves and currents on a tidal flat in the Western Scheldt Estuary. Current, Waves and Turbulence Measurements (CWTM), 2011 IEEE/OES 10th, 123–129.

Shields, A. (1936). Anwendung der Aehnlichkeitsmechanik under der Turbulenzforschung auf die Geschiebebewegung. Preussischen Versuchsanstalt Fur Wasserbau and Schiffbau, 26, 524–526.

Smagorinsky, J. (1963). General circulation experiments wiht the primitive equations I. The basic experiment. Monthly Weather Review, 91, 99–164. doi:10.1126/science.27.693.594

Smit, P. B., Stelling, G. S., Roelvink, J. A., van Thiel de Vries, J. S. M., McCall, R. T., van Dongeren, A. R., Jacobs, R. (2010). XBeach: Non-hydrostatic model. Validation, verification and model description.

Smit, P. B., Janssen, T. T., Holthuijsen, L. H., & Smith, J. (2014). Non-hydrostatic modeling of surf zone wave dynamics. Coastal Engineering, 83, 36–48. doi:10.1016/j.coastaleng.2013.09.005

Soulsby, R. L. (1997). Dynamics of Marine Sands. London: Thomas Telford Publications.

Stelling, G. S., & Zijlema, M. (2003). An accurate and efficient finite-difference algorithm for non-hydrostatic free-surface flow with application to wave propagation. International Journal for Numerical Methods in Fluids, 43(1), 1–23.

Stelling, G. S., & Duinmeijer. (2003). A staggered conservative scheme for every Froude number in rapidly varied shallow water flows. International Journal for Numerical Methods in Fluids, 43(12), 1329–1354.

Stive, M. J. F., & De Vriend, H. J. (1994). Shear stresses and mean flow in shoaling and breaking waves. Proceedings 24th International Conference on Coastal Engineering, 594–608. doi:10.9753/icce.v24.

Suzuki, T., Zijlema, M., Burger, B., Meijer, M. C., & Narayan, S. (2012). Wave dissipation by vegetation with layer schematization in SWAN. Coastal Engineering, 59(1), 64–71. doi:10.1016/j.coastaleng.2011.07.006

Svendsen, I. A. (1984a). Mass flux and undertow in a surf zone. Coastal Engineering, 8, 347–365.

Svendsen, I. A. (1984b). Wave heights and set-up in a surf zone. Coastal Engineering, 8, 303–329. doi:10.1016/0378-3839(84)90028-0

Talmon, A. M., van Mierlo, M. C., & Struiksma, N. (1995). Laboratory measurements of the direction of sediment transport on transverse alluvial-bed slope. Journal of Hydraulic Research, 33(4), 495–517.

Van Bendegom, L. (1947). Enige beschouwingen over riviermorphologie en rivierverbetering.

Van der Zwaag, J. (2014). Modelling sediment sorting near the large scale nourishment ’ The Sand Motor ’. MSc thesis, Delft University of Technology, Delft.

Van Dongeren, A. R., & Svendsen, I. A. (1997). Absorbing-generating boundary condition for shallow water models. Journal of Waterway, Port, Coastal and Ocean Engeering, (123), 303–313.

Van Dongeren, A. R., Lowe, R. J., Pomeroy, A., Trang, D. M., Roelvink, J. A., Symonds, G., & Ranasinghe, R. (2013). Numerical modeling of low-frequency wave dynamics over a fringing coral reef. Coastal Engineering, 73, 178–190.

Van Rhee, C. (2010). Sediment entrainment at high flow velocity. Journal of Hydraulic Engineering, 136, 572–582.

Van Rijn, L. C. (1985). Sediment transport, part III: bed forms and alluvial roughness. Journal of Hydraulic Engineering, 110(12), 1733–1754.

Van Rijn, L. C. (2007). Unified View of Sediment Transport by Currents and Waves: part I and II. Journal of Hydraulic Engineering, (June), 649–667.

Van Rooijen, A. A., Van Thiel de Vries, J. S. M., McCall, R. T., van Dongeren, A. R., Roelvink, J. A., & Reniers, A. J. H. M. (2015). Modeling of wave attenuation by vegetation with XBeach. E-Proceedings of the 36th IAHR World Congress 28 June – 3 July, 2015, The Hague, The Netherlands.

Van Thiel de Vries, J. S. M. (2009). Dune erosion during storm surges. PhD thesis, Delft Unversity of Technology, Delft.

Verboom, G. K., Stelling, G. S., & Officer, M. J. (1981). Boundary conditions for the shallow water equations. Engineering Applications of Computational Hydraulics, 230–262.

Walstra, D. J. R., Roelvink, J. A., & Groeneweg, J. (2000). Calculation of wave-driven currents in a 3D mean flow model. In Proceedings 27th International Conference on Coastal Engineering (pp. 1050–1063).

Walstra, D. J. R., van Rijn, L. C., Van Ormondt, M., Briere, C., & Talmon, A. M. (2007). The Effects of Bed Slope and Wave Skewness on Sediment Transport and Morphology (pp. 137–150).

Zhou, M., Roelvink, J. A., Verheij, H. J., & Ligteringen, H. (2013). Study of Passing Ship Effects along a Bank by Delft3D-FLOW and XBeach. International Workshop on Nautical Traffic Models 2013, Delft, The Netherlands, July 5-7, 2013. Delft University of Technology.

Zhou, M., Roelvink, J. A., Zou, Z., & van Wijhe, H. J. (2014). Effects of Passing Ship With a Drift Angle on a Moored Ship. ASME 2014 33rd International Conference on Ocean, Offshore and Arctic Engineering.

Zijlema, M., & Stelling, G. S. (2005). Further experiences with computing non-hydrostatic free-surface flows involving water waves. INTERNATIONAL JOURNAL FOR NUMERICAL METHODS IN FLUIDS, (48), 169–197.

Zijlema, M., & Stelling, G. S. (2008). Efficient computation of surf zone waves using the nonlinear shallow water equations with non-hydrostatic pressure. Coastal Engineering, 55(10), 780–790.

Zijlema, M., Stelling, G. S., & Smit, P. B. (2011). SWASH: An operational public domain code for simulating wave fields and rapidly varied flows in coastal waters. Coastal Engineering, 58(10), 992–1012. doi:10.1016/j.coastaleng.2011.05.015

###### Hands on exercises

The hands-on exercises can be downloaded via subversion. Subversion is a well-known version management system that allows you to always have the most recent source code at hand. It also allows developers to commit changes to the source code, without interfering with other developers. In order to use Subversion, you will need a Subversion client. A well-known client for Windows is Tortoise. If you have registered, you can download the source code via the following URL: https://svn.oss.deltares.nl/repos/xbeach/Courses/DSD_2014/Examples – Basic. For the tools like Quickplot and Quickin of the Delft3D environment is needed.

Dune erosion at Delfland, Netherlands (1D)

The first case we will run is a relative simple 1D case. It concerns a profile along the Dutch coast and the hydraulic boundary conditions are based on the 1953 storm surge that caused substantial flooding in the Netherlands.

You can work on the following assignments:

- Go to the folder “Examples\DelflandStorm” and double click the file “run_model.bat”. The simulation will start. The model will run for a few minutes, but in the meantime you can already work on question 2 to 5.

- Open params.txt in which you specify the model input files and settings. Check the number of grid-points in x-direction (keyword: nx) and y-direction (keyword: ny). Check the filenames in which you specify the wave conditions (keyword: bcfile) and the storm surge level (SSL) (keyword: zs0file).

- Do the wave conditions change during the simulation? What is/are the wave height(s) and wave period(s) applied in the simulation?

- Does the storm surge level change during the simulation? What is the maximum surge height in the simulation. Surge height is defined with respect to the mean sea level (MSL)?

- What is the simulation time (keyword: tstop)? Do we apply a morphological acceleration factor (keyword: morfac)? What variables are stored as output and with what time interval? How much hydrodynamic time is simulated?

- Probably the simulation has finished. When you start the model, it generates a file named XBlog.txt. Open this file and check what is stored in the file. What was the total simulation time?

- To check out the simulation results we make use of the Quickplot tool (A brief tutorial is attached to this document). You can start Quickplot via the Delft3D environment we installed (Start  Programs  Deltares  Delft3D  Delft3D). In the Delft 3D menu choose Utilities  Quickplot. Choose Files of type “NetCDF files and GRIB files” and open “xboutput.nc” in the simulation folder.

- Use the Quickplot tutorial and try to make an animation in which you plot short wave height (H), water level (including long wave variations, zs) and bed level (zb) as function of time.

- Plot the offshore water level as function of time. Also open the file “tide.tek” (Tekal data files format), which contains the imposed surge level. Did the model correctly simulate the imposed surge level?

- Copy all model files to a new folder named “superfast”. Edit params.txt and set ny=0 (instead of ny=2), and run the model. What is the simulation time compare to the original simulation?

- Compare simulation results for the “superfast” and “default” simulation. Are these the same? What option will you use in the future?

Nourishment scenarios near Kijkduin, Holland (1D)

This case concerns the exploration of a nourishment strategy near Kijkduin along the Holland coast in the Netherlands. At this location a mega nourishment of 21 Mm3 named the Sand Engine was constructed. In this case we will explore to what extent nourishments can reduce the (dune and beach) erosion during a storm event.

You can work on the following assignments:

- Go to the folder “Examples\Nourishment case” and double click on the file “runall.bat”. This batch file will run three simulations sequentially in which the profile configuration varies and corresponds with the undisturbed profile (folder reference), a shoreface nourishment (folder shoreface) and a beach nourishment (folder beach) respectively. Each model will run for a few minutes. While running you can already answer question 2 to 6.

- For the reference case open the params.txt in which you specify model input files and settings. Check the number of grid-points in x-direction (keyword: nx) and y-direction (keyword: ny). How many directional wave bins are defined and what is their width (keywords: thetamin, thetamax, dtheta).

- Do the wave conditions change during the simulation? What is/are the wave height(s) and wave period(s) applied in the simulation?

- Does the storm surge level change during the simulation? What is the maximum surge height in the simulation. Surge height is defined with respect to the mean sea level (MSL)?

- What is the simulation time (keyword: tstop)? Do we apply a morphological acceleration factor (keyword: morfac)? What variables are stored as output and with what time interval? How much hydrodynamic time is simulated?

- Probably the simulation has finished. When you start the model, it generates a file named XBlog.txt. Open this file and check what is stored in the file. What was the total simulation time?

- Inspect the initial bathymetries of each simulation with QUICKPLOT. Choose Files of type “NetCDF files and GRIB files” and open “xboutput.nc” in the simulation folder).

  - At what cross-shore position were the shoreface nourishment and beach nourishment placed?

  - What is the (average) thickness of the nourishments?

  - Is the volume of the nourishments comparable?

  - Plot the reference profile with markers; does the grid resolution vary in cross-shore direction?

- Use the Quickplot tutorial and try to make an animation in which you plot short wave height (H), water level (including long wave variations, zs) and bed level (zb) as function of time.

- Plot the offshore water level as function of time. Also open the file “tide.tek” (Tekal data files format), which contains the imposed surge level. Did the model correctly simulate the imposed surge level?

- Inspect the final bathymetries of each simulation.

  - What is the dune face retreat in the three simulations you have carried out?

  - Where does the eroded sediment form the dunes deposit?

  - What nourishment type is most effective in reducing the impact of a storm and do you have an explanation for this?

- In the folder “banquette” you find a final simulation in which a special beach nourishment type is evaluated named a banquette. This beach nourishment has a highly elevated flat area that connects to the dune foot on which beach restaurants can be build.

  - Run the model and compare in Quickplot the banquette design with the beach nourishment design we have evaluated before. Do you expect more or less erosion?

  - Check your hypothesis by comparing the final profile of the banquette simulation to the other simulations.

  - What would be your approach to further reduce beach and dune erosion?

Overwash at Santa Rosa Island , USA (2DH)

This case concerns overwash at Santa Rosa island in the Gulf of Mexico during hurricane Ivan in 2004.

You can work on the following assignments.

- For the reference case open the params.txt in which you specify model input files and settings. Check the number of grid-points in x-direction (keyword: nx) and y-direction (keyword: ny). How many directional wave bins are defined and what is their width (keywords: thetamin, thetamax, dtheta).

- In this simulation the grid is specified in Delft3D format. Open Quickin in the Delft 3D menu (Grid  Quickin) and use the brief tutorial to read in the grid and bathymetry. Does the grid resolution vary in cross-shore direction? And in longshore direction? What are the minimum dx and dy? Why can the grid be coarse offshore?

- How many wave conditions do we apply in this simulation? What is the offshore mean wave direction? Does the surge level change in the simulation?

- What is the simulation time (hydrodynamic and morphologic)?

- Inspect the model results and make an animation of the short wave height (H) and the water levels (including long wave, zs). Describe what is happening.

- For the water levels set the color limits manual between -0.5 and 3.5.

- Make an animation of cumulative sedimentation/erosion. Describe what is happening.

- For the sedimentation/erosion set the color limits manual between -3 and 3

- Look at the mean flow field. Plot the flow field in colored vectors. Where are the flow velocities highest and what is the direction of the flow (cross-shore or longshore)? Is there (also) a longshore current present and what is its intensity?

If you have time left feel free to:

- Narrow or broaden the imposed spectrum by changing the parameter directional spreading (s) in ‘jonswap.inp’ (you could for example set s = 100 and s = 2 respectively). Make animations of the instantaneous short wave height to see what is happening to the size of the wave groups.

- Design a nourishment in Quickin to reduce the impact of the storm on Santa Rosa Island. Change the depth file in params.txt to make a simulation with the updated bathymetry.

Yanchep perched beach and natural breakwater (2DH)

This case is an example of a beach 60km north of Perth most commonly known as Yanchep lagoon. Many beaches in WA like Yanchep are fronted by shallow reef and here we are investigating the effects of the reef on the morphodynamics.

You can work on the following assignments:

- Go to the folder “Examples\YanchepBeach” and double click the file “run_model.bat”. The simulation will start (and will run about 15 minutes).

- Meanwhile, inspect the bathymetry file and the structure file (using Quickin). What is the depth in the lagoon? Is the reef enclosing the lagoon below or above the model initial water level? What is the wave height at the boundary condition?

- Use  Quickplot and try to make an animation in which you plot short wave height (H), water level (including long wave variations) (zs) and Eulerian velocities (ue and ve) as function of time. What happens in the lagoon?

- Use  Quickplot and try to make an animation of cumulative sedimentation/erosion. What happens in the lagoon?

- How is the lagoon affected by the mean water level? Increase or decrease the mean water level condition (‘tide.tx’), run the model again (maybe for a shorter time by reducing keyword: tstop). How are the circulation and sediment transport affected?

- What would happen if the lagoon was open at the southern end? Open the structure file (keyword: ne_layer=’reef.dep’) with the Quickin tool and modify it to allow the southern end of the lagoon to be eroded. Modify the param.txt file to use this new structure file and run the model. Alternatively, remove the reef from the bathymetry and rerun the model without the structure file, by setting the keyword struct=0.

If you still have time;

- Reefs are very rough what happens in the model when the friction is increased? Reduce the Chezy roughness and increase the value of fw. Rerun the model what do you observe?

- Is wave/current interaction (keyword: wci=1) switched on? Rerun the model with the wave/current switch on/off. Compare the output with model you ran previously. How much effect do you see on the morphology?

###### Advanced model coefficients

In 4.1 the main input parameters and files required by XBeach to start a simulation are explained. It explained how the user can switch on and off specific processes and how the user can define the model initial and boundary conditions. XBeach offers, however, many more parameters to fine-tune the simulation of different processes. These parameters are listed in the following subsections grouped by process. Most parameters are not relevant for the average XBeach user. Parameters marked with a plus (+) are considered advanced options that are recommended to stay untouched unless you know what you are doing.

Wave numerics

The parameters listed in the table below involve the numerical aspects of the wave action balance that solves the wave propagation in the model. The keyword scheme can be used to set the numerical scheme. By default a higher-order upwind scheme is used to minimize numerical dissipation.

Table B.1	Overview of available keyword related to wave numerics

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| maxerror+ | Maximum wave height error in wave stationary iteration | 5e-05 | 1e-05 - 0.001 | m |  |
| maxiter+ | Maximum number of iterations in wave stationary | 500 | 2 - 1000 | - |  |
| scheme+ | Numerical scheme for wave propagation | upwind_2 | upwind_1, lax_wendroff, upwind_2 |  |  |
| wavint | Interval between wave module calls (only in stationary wave mode) | 60.0 | 1.0 - 3600.0 | s |  |

Wave dissipation

The parameters listed in the table below involve the wave dissipation process. For instationary model runs use either break=roelvink1, roelvink2 or roelvink_daly. Note that the standard value gamma=0.55 and n=10 was calibrated for option break=roelvink1. For break=roelvink2 the wave dissipation is proportional to H3/h instead of H2; this affects the calibration. For stationary runs the break=baldock option is suitable. The break=roelvink_daly option is a model in which waves start and stop breaking. Reducing gammax will reduce wave heights in very shallow water, probably 2 is a reasonable value.

Table B.2	Overview of available keyword related to the wave dissipation model

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| alpha+ | Wave dissipation coefficient in Roelvink formulation | 1.0 | 0.5 - 2.0 | - |  |
| break | Type of breaker formulation | roelvink2 | roelvink1, baldock, roelvink2, roelvink_daly, janssen |  |  |
| breakerdelay+ | Switch to enable breaker delay model | 1 | 0 - 1 | - |  |
| delta+ | Fraction of wave height to add to water depth | 0.0 | 0.0 - 1.0 | - |  |
| facrun+ | Calibration coefficient for short wave runup | 1.0 | 0.0 - 2.0 | - |  |
| facsd+ | Fraction of the local wave length to use for shoaling delay depth | 1.0 | 0.0 - 2.0 | - |  |
| fw+ | Bed friction factor | 0.0 | 0.0 - 1.0 | - |  |
| fwcutoff | Depth greater than which the bed friction factor is not applied | 1000.0 | 0.0 - 1000.0 | - |  |
| gamma | Breaker parameter in Baldock or Roelvink formulation | 0.55 | 0.4 - 0.9 | - |  |
| gamma2 | End of breaking parameter in Roelvink Daly formulation | 0.3 | 0.0 - 0.5 | - |  |
| gammax+ | Maximum ratio wave height to water depth | 2.0 | 0.4 - 5.0 | - |  |
| n+ | Power in Roelvink dissipation model | 10.0 | 5.0 - 20.0 | - |  |
| shoaldelay+ | Switch to enable shoaling delay | 0 | 0 - 1 | - |  |

Rollers

The parameters listed in the table below involve the wave roller model. Using the roller model will give a shoreward shift in wave-induced setup, return flow and alongshore current. This shift becomes greater for lower beta values.

Table B.3	Overview of available keyword related to the roller model

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| beta+ | Breaker slope coefficient in roller model | 0.1 | 0.05 - 0.3 | - |  |
| rfb+ | Switch to feedback maximum wave surface slope in roller energy balance, otherwise rfb = Beta | 0 | 0 - 1 | - |  |
| roller+ | Switch to enable roller model | 1 | 0 - 1 | - |  |

Wave-current interaction

The parameters listed in the table below involve the process of wave-current interaction. With the switch wci one can turn off or on the wave-current interaction, the wave current interaction will result in a feedback of currents on the wave propagation. On top of that, hwci limits the computation of wave-current interaction in very shallow water where the procedure may not converge.

Table B.4	Overview of available keyword related to the wave-current interaction (wci)

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| cats+ | Current averaging time scale for wci, in terms of mean wave periods | 4.0 | 1.0 - 50.0 | Trep |  |
| hwci+ | Minimum depth until which wave-current interaction is used | 0.1 | 0.001 - 1.0 | m |  |
| wci | Turns on wave-current interaction | 0 | 0 - 1 | - |  |

Bed friction and viscosity

The parameters listed in the table below involve the settings for bed friction and viscosity influencing the flow in XBeach. The bed friction is influenced by the dimensionless friction coefficient cf or other formulation like the dimensional Chézy or Manning. The bed friction formulation applied needs to be determined with the keyword bedfriction. It is possible both to define one value (keyword: bedfriccoef) or to apply, spatially varying values for the bed friction. A spatial varying friction can be provided through an external file referenced via the keyword bedfricfile. The file has the same format as the bathymetry file explained in Section 4.3 (Grid and bathymetry).

The horizontal viscosity is composed of an overall background viscosity nuh and a viscosity depending on the roller dissipation tuned by nuhfac. In the alongshore direction the viscosity may be multiplied by a factor nuhv to account for additional advective mixing. It is also possible to use a user-defined value for the horizontal viscosity (keyword smag = 0)

Table B.5	Overview of available keyword related to the bed friction and viscosity

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| bedfriccoef | Bed friction coefficient | 0.01 | 3.5e-05 - 0.9 | - |  |
| bedfricfile | Bed friction file |  |  | <file> |  |
| bedfriction | Bed friction formulation | chezy | chezy, cf, white-colebrook, manning, white-colebrook-grainsize |  |  |
| nuh | Horizontal background viscosity | 0.1 | 0.0 - 1.0 | m^2s^-1 |  |
| nuhfac+ | Viscosity switch for roller induced turbulent horizontal viscosity | 1.0 | 0.0 - 1.0 | - |  |
| nuhv+ | Longshore viscosity enhancement factor, following Svendsen | 1.0 | 1.0 - 20.0 | - |  |
| smag+ | Switch for smagorinsky sub grid model for viscosity | 1 | 0 - 1 | - |  |

Flow numerics

The parameters listed in the table below involve the numerical aspects of the shallow water equations that solve the water motions in the model. Especially in very shallow water some processes need to be limited to avoid unrealistic behavior. For example hmin prevents very strong return flows or high concentrations and the eps determines whether points are dry or wet and can be taken quite small.

Table B.6	Overview of available keyword related to flow numerics

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| eps | Threshold water depth above which cells are considered wet | 0.005 | 0.001 - 0.1 | m |  |
| eps_sd | Threshold velocity difference to determine conservation of energy head versus momentum | 0.5 | 0.0 - 1.0 | m/s |  |
| hmin | Threshold water depth above which Stokes drift is included | 0.2 | 0.001 - 1.0 | m |  |
| oldhu+ | Switch to enable old hu calculation | 0 | 0 - 1 | - |  |
| secorder+ | Use second order corrections to advection/non-linear terms based on MacCormack scheme | 0 | 0 - 1 | - |  |
| umin | Threshold velocity for upwind velocity detection and for vmag2 in equilibrium sediment concentration | 0.0 | 0.0 - 0.2 | m/s |  |

Sediment transport

The parameters listed in the table below involve the process of sediment transport. The keywords facAs and facSk determine the effect of the wave form on the sediment transport, this is especially important in the nearshore. The facua is an alias setting in which both parameters can be varied at once. The wave form model itself is selected using the keyword waveform. Processes like short- and long-wave stirring and turbulence can be switched on or off using the keywords sws, lws and lwt. Several options for calibrating the sediment transport formulations are available as well as keywords to incorporate the bed slope effect.

Table B.7	Overview of available keyword related to the sediment transport model

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| BRfac+ | Calibration factor surface slope | 1.0 | 0.0 - 1.0 | - |  |
| Tbfac+ | Calibration factor for bore interval Tbore: Tbore = Tbfac+Tbore | 1.0 | 0.0 - 1.0 | - |  |
| Tsmin+ | Minimum adaptation time scale in advection diffusion equation sediment | 0.5 | 0.01 - 10.0 | s |  |
| bdslpeffdir | Modify the direction of the sediment transport based on the bed slope | none | none, talmon |  |  |
| bdslpeffdirfac | Calibration factor in the modification of the direction | 1.0 | 0.0 - 2.0 | - |  |
| bdslpeffini | Modify the critical shields parameter based on the bed slope | none | none, total, bed |  |  |
| bdslpeffmag | Modify the magnitude of the sediment transport based on the bed slope, uses facsl | roelvink_total | none, roelvink_total, roelvink_bed, soulsby_total, soulsby_bed |  |  |
| bed+ | Calibration factor for bed transports | 1 | 0 - 1 | - |  |
| betad+ | Dissipation parameter long wave breaking turbulence | 1.0 | 0.0 - 10.0 | - |  |
| bulk+ | Switch to compute bulk transport rather than bed and suspended load separately | 0 | 0 - 1 | - |  |
| dilatancy | Switch to reduce critical shields number due dilatancy | 0 | 0 - 1 | - |  |
| facAs+ | Calibration factor time averaged flows due to wave asymmetry | 0.1 | 0.0 - 1.0 | - |  |
| facDc+ | Option to control sediment diffusion coefficient | 1.0 | 0.0 - 1.0 | - |  |
| facSk+ | Calibration factor time averaged flows due to wave skewness | 0.1 | 0.0 - 1.0 | - |  |
| facsl+ | Factor bedslope effect | 1.6 | 0.0 - 1.6 | - |  |
| facua+ | Calibration factor time averaged flows due to wave skewness and asymmetry | 0.1 | 0.0 - 1.0 | - |  |
| fallvelred | Switch to reduce fall velocity for high concentrations | 0 | 0 - 1 | - |  |
| form | Equilibrium sediment concentration formulation | vanthiel_vanrijn | soulsby_vanrijn, vanthiel_vanrijn |  |  |
| jetfac+ | Option to mimic turbulence production near revetments | 0.0 | 0.0 - 1.0 | - |  |
| lws+ | Switch to enable long wave stirring | 1 | 0 - 1 | - |  |
| lwt+ | Switch to enable long wave turbulence | 0 | 0 - 1 | - |  |
| pormax | Max porosity used in the expression of Van Rhee | 0.5 | 0.3 - 0.6 | - |  |
| reposeangle | Angle of internal friction | 30.0 | 0.0 - 45.0 | deg |  |
| rheeA | A parameter in the Van Rhee expression | 0.75 | 0.75 - 2.0 | - |  |
| smax+ | Maximum Shields parameter for equilibrium sediment concentration acc. Diane Foster | -1.0 | -1.0 - 3.0 | - |  |
| sus+ | Calibration factor for suspensions transports | 1 | 0 - 1 | - |  |
| sws+ | Switch to enable short wave and roller stirring and undertow | 1 | 0 - 1 | - |  |
| tsfac+ | Coefficient determining Ts = tsfac + h/ws in sediment source term | 0.1 | 0.01 - 1.0 | - |  |
| turb+ | Switch to include short wave turbulence | bore_averaged | none, wave_averaged, bore_averaged |  |  |
| turbadv+ | Switch to activate turbulence advection model for short and or long wave turbulence | none | none, lagrangian, eulerian |  |  |
| waveform | Wave shape model | vanthiel | ruessink_vanrijn, vanthiel |  |  |
| z0+ | Zero flow velocity level in Soulsby and van Rijn (1997) sediment concentration | 0.006 | 0.0001 - 0.05 | m |  |

Sediment transport numerics

The parameters listed in the table below involve the numerical aspects of sediment transport that are all considered advanced options. For example the maximum allowed sediment concentration can be varied with the keyword cmax. It is however not recommended varying these settings.

Table B.8	Overview of available keyword related to sediment transport numerics

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| cmax+ | Maximum allowed sediment concentration | 0.1 | 0.0 - 1.0 | - |  |
| sourcesink+ | Switch to enable source-sink terms to calculate bed level change rather than suspended transport gradients | 0 | 0 - 1 | - |  |
| thetanum+ | Coefficient determining whether upwind (1) or central scheme (0.5) is used. | 1.0 | 0.5 - 1.0 | - |  |

Quasi-3D sediment transport

The parameters listed in the table below involve the tuning of quasi-3D sediment transport, if enabled. The most important setting is the kmax in which the user specifies the number of layers used in the quasi 3D sediment model.

Table B.9	Overview of available keyword related to the quasi 3D sediment transport

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| kmax+ | Number of sigma layers in Quasi-3D model; kmax = 1 is without vertical structure of flow and suspensions | 1 | 1 - 1000 | - |  |
| sigfac+ | Dsig scales with log(sigfac) | 1.3 | 0.0 - 10.0 | - |  |
| vicmol+ | Molecular viscosity | 1e-06 | 0.0 - 0.001 | - |  |
| vonkar+ | Von Karman constant | 0.4 | 0.01 - 1.0 | - |  |

Morphology

The parameters listed in the table below involve the morphological processes. The dryslp and wetslp keyword define the critical avalanching slope above and below water respectively. If the bed exceeds the relevant critical slope it collapses and slides downward (avalanching). To reduce the impact of these landslides the maximum bed level change due to avalanching is limited by the dzmax value. Which of the two slopes is applied to a grid cell is determined by the hswitch keyword.

The keyword morfac enables the user to decouple the hydrodynamical and the morphological time. This is suitable for situations where the morphological process is much slower than the hydrodynamic process. The factor defined by the morfac keyword is applied to all morphological change. A morfac=10 therefore results in 10 times more erosion and deposition in a given time step than usual. The simulation time is however then shortened with the same factor to obtain an approximate result more quickly. The user can prevent the simulation time to be adapted to the morfac value by setting morfacopt to zero. The keywords morstart and morstop let the user enable the morphological processes in XBeach only for a particular period during the (hydrodynamic) simulation. These options can be useful if a spin-up time is needed for the hydrodynamics.

The struct and ne_layer keywords enable the user to specify non-erodible structures in the model. To switch on non-erodible structures use struct=1. The location of the structures is specified in an external file referenced by the ne_layer keyword. The file has the same format as the bathymetry file explained in Section 4.3 (Grid and bathymetry). The values of the file define the thickness of the erodible layer on top of the non-erodible layer. A ne_layer file with only zeros therefore defines a fully non-erodible bathymetry and a file with only tens means a erodible layer of 10 meters. Only at the grid cells where the value in the ne_layer file is larger than zero erosion can occur. Non-erodible layers are infinitely deep and thus no erosion underneath these layers can occur.

Table B.10	Overview of available keyword related to morphology

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| dryslp | Critical avalanching slope above water (dz/dx and dz/dy) | 1.0 | 0.1 - 2.0 | - |  |
| dzmax+ | Maximum bed level change due to avalanching | 0.05 | 0.0 - 1.0 | m/s/m |  |
| hswitch+ | Water depth at which is switched from wetslp to dryslp | 0.1 | 0.01 - 1.0 | m |  |
| morfac | Morphological acceleration factor | 1.0 | 0.0 - 1000.0 | - |  |
| morfacopt+ | Switch to adjusting output times for morfac | 1 | 0 - 1 | - |  |
| morstart | Start time morphology, in morphological time | 120.0 | 0.0 - 10000000.0 | s |  |
| morstop | Stop time morphology, in morphological time | 2000.0 | 0.0 - 10000000.0 | s |  |
| ne_layer | Name of file containing depth of hard structure |  |  | <file> |  |
| struct | Switch for enabling hard structures | 0 | 0 - 1 | - |  |
| wetslp | Critical avalanching slope under water (dz/dx and dz/dy) | 0.3 | 0.1 - 1.0 | - |  |

Bed update

The parameters listed in the table below involve the settings for the bed update process especially in the case multiple sediment fractions and bed layers are involved. The frac_dz, split and merge keywords determine the fraction of the variable bed layer thickness at which the layer is split or merged respectively with the surrounding bottom layers. The variable layer is chosen using the nd_var keyword.

Pre-defined bed evolution can be used with the keywords setbathy that is used to turn user-imposed bed level change on, nsetbathy that determines the number of imposed bed level conditions and setbathyfile that references a file that specifies the time series of imposed bed levels. If the setbathy option is used, XBeach will automatically interpolate the bed level at each computational time step from the imposed bed level time series.

The setbathyfile file must contain a time series (of length nsetbathy) of the bed level at every grid point in the XBeach model. The format of the setbathyfile file is such that each imposed bed level starts with the time at which the bed level should be applied (in seconds relative to the start of the simulation), followed on a new line by the bed level in each grid cell in an identical manner to the normal initial bathymetry file (e.g., one line per transect in the x-direction). The following imposed bed level condition starts on a new line with the time at which the condition is imposed and the bed levels at that time, etc. An example of a setbathyfile is given below.

setbathyfile.dep (generalized)

<t 0>

<z 1,1> <z 2,1> <z 3,1> ... <z nx,1> <z nx+1,1>

<z 1,2> <z 2,2> <z 3,2> ... <z nx,2> <z nx+1,2>

<z 1,3> <z 2,3> <z 3,3> ... <z nx,3> <z nx+1,3>

...

<z 1,ny> <z 2,ny> <z 3,ny> ... <z nx,ny> <z nx+1,ny>

<z 1,ny+1> <z 2,ny+1> <z 3,ny+1> ... <z nx,ny+1> <z nx+1,ny+1>

<t 1>

<z 1,1> <z 2,1> <z 3,1> ... <z nx,1> <z nx+1,1>

<z 1,2> <z 2,2> <z 3,2> ... <z nx,2> <z nx+1,2>

<z 1,3> <z 2,3> <z 3,3> ... <z nx,3> <z nx+1,3>

...

<z 1,ny> <z 2,ny> <z 3,ny> ... <z nx,ny> <z nx+1,ny>

<z 1,ny+1> <z 2,ny+1> <z 3,ny+1> ... <z nx,ny+1> <z nx+1,ny+1>

...

...

<t nsetbathy>

<z 1,1> <z 2,1> <z 3,1> ... <z nx,1> <z nx+1,1>

<z 1,2> <z 2,2> <z 3,2> ... <z nx,2> <z nx+1,2>

<z 1,3> <z 2,3> <z 3,3> ... <z nx,3> <z nx+1,3>

...

<z 1,ny> <z 2,ny> <z 3,ny> ... <z nx,ny> <z nx+1,ny>

<z 1,ny+1> <z 2,ny+1> <z 3,ny+1> ... <z nx,ny+1> <z nx+1,ny+1>

setbathyfile.dep (short 1D example, nx = 20, ny = 0, nsetbathy = 4)

0

-20 -18 -16 -14 -12 -10 -8 -6 -5 -4 -3 -2 -1 -0.5 0 0.5 1 2 3 4 5

1200

-20 -18 -16 -14 -12 -10 -8 -6 -5 -4 -3 -1.8 -0.75 -0.25 0 0.2 0.7 2 3 4 5

2400

-20 -18 -16 -14 -12 -10 -8 -6 -5 -4 -2.5 -1.6 -0.5 -0.3 0 0.1 0.6 1.5 3 4 5

3600

-20 -18 -16 -14 -12 -10 -8 -6 -5 -4 -2 -1.4 -0.3 0 0.05 0.1 0.6 1 2.2 3.5 5

Note that if the setbathy option is used, the initial bed level is derived from an interpolation of the setbathyfile file time series, not from the depfile file. It is strongly advised to turn of the computation of morphological updating (keyword: morphology = 0) if the setbathy option is used, as the computed morphological change will be overridden by the imposed morphological change.

Table B.11	Overview of available keyword related to the bed update module

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| frac_dz* | Relative thickness to split time step for bed updating | 0.7 | 0.5 - 0.98 | - |  |
| merge* | Merge threshold for variable sediment layer (ratio to nominal thickness) | 0.01 | 0.005 - 0.1 | - |  |
| nd_var* | Index of layer with variable thickness | 2 | 2 - nd | - |  |
| nsetbathy* | Number of prescribed bed updates | 1 | 1 - 1000 | - |  |
| setbathyfile* | Name of prescribed bed update file |  |  | <file> |  |
| split* | Split threshold for variable sediment layer (ratio to nominal thickness) | 1.01 | 1.005 - 1.1 | - |  |

Groundwater flow

The parameters listed in the table below involve the process of groundwater flow. The vertical permeability coefficient in the vertical can be set differently than that of the horizontal using the keywords kz and kz respectively. The initial bed level of the aquifer is read from an external file referenced by the aquiferbotfile keyword and the initial groundwater head can be set to either a uniform value using the gw0 keyword or to spatially varying values using an external file referenced by the gw0file keyword. Both files have the same format as the bathymetry file explained in Section 4.3 (Grid and bathymetry).

Table B.12	Overview of available keyword related to the groundwater module

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| aquiferbot+ | Level of uniform aquifer bottom | -10.0 | -100.0 - 100.0 | m |  |
| aquiferbotfile+ | Name of the aquifer bottom file |  |  | <file> |  |
| dwetlayer+ | Thickness of the top soil layer interacting more freely with the surface water | 0.1 | 0.01 - 1.0 | m |  |
| gw0+ | Level initial groundwater level | 0.0 | -5.0 - 5.0 | m |  |
| gw0file+ | Name of initial groundwater level file |  |  | <file> |  |
| gwReturb+ | Reynolds number for start of turbulent flow in case of gwscheme = turbulent | 100.0 | 1.0 - 600.0 | - |  |
| gwfastsolve+ | Reduce full 2D non-hydrostatic solution to quasi-explicit in longshore direction | 0 | 0 - 1 | - |  |
| gwheadmodel+ | Model to use for vertical groundwater head | parabolic | parabolic, exponential |  |  |
| gwhorinfil+ | Switch to include horizontal infiltration from surface water to groundwater | 0 | 0 - 1 | - |  |
| gwnonh+ | Switch to turn on or off non-hydrostatic pressure for groundwater | 0 | 0 - 1 | - |  |
| gwscheme+ | Scheme for momentum equation | laminar | laminar, turbulent |  |  |
| kx+ | Darcy-flow permeability coefficient in x-direction | 0.0001 | 1e-05 - 0.1 | ms^-1 |  |
| ky+ | Darcy-flow permeability coefficient in y-direction | 0.0001 | 1e-05 - 0.1 | ms^-1 |  |
| kz+ | Darcy-flow permeability coefficient in z-direction | 0.0001 | 1e-05 - 0.1 | ms^-1 |  |

Non-hydrostatic correction

The parameters listed in the table below involve the settings for the non-hydrostatic option (keyword: wavemodel = nonh). These are all considered advanced options and it is thus recommended not to change these

Table B.13	Overview of available keyword related to the non-hydrostatic module

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| Topt+ | Absolute period to optimize coefficient | 10.0 | 1.0 - 20.0 | s |  |
| breakviscfac+ | Factor to increase viscosity during breaking | 1.5 | 1.0 - 3.0 | - |  |
| breakvisclen+ | Ratio between local depth and length scale in extra breaking viscosity | 1.0 | 0.75 - 3.0 | - |  |
| dispc+ | Coefficient in front of the vertical pressure gradient | 1.0 | 0.1 - 2.0 | ? |  |
| kdmin+ | Minimum value of kd (pi/dx > min(kd)) | 0.0 | 0.0 - 0.05 | - |  |
| maxbrsteep+ | Maximum wave steepness criterium | 0.6 | 0.3 - 0.8 | - |  |
| nhbreaker+ | Non-hydrostatic breaker model | 2 | 0 - 3 | - |  |
| reformsteep+ | Wave steepness criterium to reform after breaking | 0.25 + maxbrsteep | 0.0 - 0.95+ maxbrsteep | - |  |
| secbrsteep+ | Secondary maximum wave steepness criterium | 0.5 + maxbrsteep | 0.0 - 0.95+ maxbrsteep | - |  |
| solver+ | Solver used to solve the linear system | tridiag | sip, tridiag |  |  |
| solver_acc+ | Accuracy with respect to the right-hand side used in the following termination criterion: ¦¦b-Ax ¦¦ < acc+¦¦b¦¦ | 0.005 | 1e-05 - 0.1 | - |  |
| solver_maxit+ | Maximum number of iterations in the linear sip solver | 30 | 1 - 1000 | - |  |
| solver_urelax+ | Under relaxation parameter | 0.92 | 0.5 - 0.99 | - |  |

Physical constants

The parameters listed in the table below involve physical constants used by XBeach. The gravitational acceleration and density of water are universally used coefficient. The depthscale is a factor in order to set different cut-off values like eps and hswitch. A value of the depthscale lower than one means the cut-off values will increase.

Table B.14	Overview of available keyword related to physics

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| depthscale+ | Depthscale of (lab)test simulated, affects eps, hmin, hswitch and dzmax | 1.0 | 0.1 - 200.0 | - |  |
| g | Gravitational acceleration | 9.81 | 9.7 - 9.9 | ms^-2 |  |
| rho | Density of water | 1025.0 | 1000.0 - 1040.0 | kgm^-3 |  |

Coriolis force

The parameters listed in the table below involve the settings for incorporating the effect of Coriolis on the shallow water equations. The keywords are universally used coefficients.

Table B.15	Overview of available keyword related to the Coriolis force

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| lat+ | Latitude at model location for computing Coriolis | 0.0 | -90.0 - 90.0 | deg |  |
| wearth+ | Angular velocity of earth calculated as: 1/rotation_time (in hours) | 0.0417 | 0.0 - 1.0 | hour^-1 |  |

MPI

The parameters listed in the table below involve the settings for parallelization of XBeach. When running XBeach in parallel mode, the model domain is subdivided in sub models and each sub model is then computed on a separate core. This will increase the computational speed of the model. The sub models only exchange information over their boundaries when necessary. The MPI parameters determine how the model domain is subdivided. The keyword mpiboundary can be set to auto, x, y or man. In auto mode the model domain is subdivided such that the internal boundary is smallest. In x or y mode the model domain is subdivided in sub models extending to either the full alongshore or the full cross-shore extent of the model domain. In man mode the model domain is manually subdivided using the values specified with the mmpi and nmpi keywords. The number of sub models is not determined by XBeach itself, but by the MPI wrapper (e.g. MPICH2 or OpenMPI).

Table B.16	Overview of available keyword related to MPI

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| mmpi+ | Number of domains in cross-shore direction when manually specifying mpi domains | 2 | 1 - 100 | - |  |
| mpiboundary+ | Fix mpi boundaries along y-lines, x-lines, use manual defined domains or find shortest boundary automatically | auto | auto, x, y, man |  |  |
| nmpi+ | Number of domains in alongshore direction when manually specifying mpi domains | 4 | 1 - 100 | - |  |

Output projection

The parameters listed in the table below involve the projection of the model output. These settings do not influence the model results in anyway. The rotate keyword can be used to rotate the model output with an angle specified by the keyword alfa. The projection string can hold a string specifying the coordinate reference system used and is stored in the netCDF output file as metadata.

Table B.17	Overview of available keyword related to the output projection

| keyword | description | default | range | units | remark |
| --- | --- | --- | --- | --- | --- |
| projection+ | Projection string | ' ' |  | - |  |
| rotate | Rotate output as post processing with given angle | 1 | 0 - 1 | - |  |

###### Numerical implementation

Grid set-up

The new implementation utilizes a curvilinear, staggered grid where depths, water levels, wave action and sediment concentrations are given in the cell centers (denoted by subscript z) and velocities and sediment fluxes at the cell interfaces (denoted by subscript u or v). In Error! Reference source not found. the z, u, v and c (corner) points with the same numbering are shown. The grid directions are named s and n; grid distances are denoted by Δs and Δn, with subscripts referring to the point where they are defined. A finite-volume approach is utilized where mass, momentum and wave action are strictly conserved.  In the middle panel of Error! Reference source not found., the control volume for the mass balance is shown with the corresponding grid distances around the u- and v-points. The right panel explains the numbering of the fluxes Q and the volume V.

Figure 5.1		Location of staggered grid points (left panel); definition of grid distances (middle) and terms in volume balance (right)

Wave action balance

Surfbeat solver

The time-varying wave action balance solved in XBeach is as follows:

(C.1)

Where A is the wave energy or wave action, cg is the group velocity, cΘ the refraction speed in theta-space and SINK refers to effects of wave breaking, vegetation and bottom friction. Again, the advection terms are the only ones affected by the curvilinear scheme so we will discuss their treatment in detail. The control volume is the same as for the mass balance. In (C.1) the procedure to compute the wave energy fluxes across the cell boundaries is outlined. All variables should also have an index itheta referring to the directional grid, but for brevity these are omitted here.

The component of the group velocity normal to the cell boundary, at the cell boundary, is interpolated from the two adjacent cell center points. Depending on the direction of this component, the wave energy at the cell boundary is computed using linear extrapolation based on the two upwind points, taking into account their grid distances. This second order upwind discretization preserves the propagation of wave groups with little numerical diffusion.

(C.2)

The other three fluxes are computed in a similar way; for brevity we will not present all formulations.

The time integration is explicit and the same as in the original implementation. The advection in u- and v-direction is computed simply by adding the four fluxes and dividing by the cell area. This procedure guarantees conservation of wave energy.

(C.3)

The procedure for the roller energy balance is identical to that for the wave energy balance and will not be repeated here.

Stationary solver

In the stationary solver the wave energy and roller energy balances are solved line by line, from the seaward boundary landward. For each line the automatic time step is computed and the quasi-time-dependent balance according to (C.3) is solved until convergence or the maximum number of iterations is reached, after which the solver moves to the next line.

The iteration is controlled by the keywords maxiter  and maxerror.

Shallow water equations

Mass balance equation

The mass balance reads as follows:

(C.4)

This is discretized according to:

(C.5)

Here, Acell is the area of the cell around the cell center, zs is the surface elevation, uu is the u-velocity in the u-point, hu the water depth in the u-point and vv the v-velocity in the v-point. The indices i,j refer to the grid number in u resp. v direction; the index n refers to the time step.

Momentum balance equation

Second, we will outline the derivation of the u-momentum balance. The control volume is given in Figure C.2. It is centered around the u-point. We now consider the rate of change of the momentum in the local u-direction as follows:

(C.6)

Where V is the cell volume, u the velocity in local grid direction, Q the fluxes, the density, g acceleration of gravity, τbu the bed shear stress, τsu, wind shear stress and Fu wave force in u-direction. We consider that the outgoing fluxes carry the velocity inside the cell, u and that uin is determined at each inflow boundary by interpolation, reconstructing the component in the same direction as u.

The volume balance for the same volume reads:

(C.7)

By multiplying the volume balance by u, subtracting it from the momentum balance and dividing the result by V (Acell, hum) we arrive at the following equation:

(C.8)

Where Acell is the cell area and hum is the average depth of the cell around the u-point. The procedure for the second term (the others are straightforward) now boils down to integrating (only) the incoming fluxes over the interfaces and multiplying them with the difference between u in the cell and the component of velocity in the same direction at the upwind cell.

Figure 5.2 			Control volume u-momentum balance and definition of fluxes

In (C.9) and (C.10) the procedure for computing the u-momentum balance is outlined. The discharges in the u-points are computed by multiplying the velocity in the u- or v-point by the water depth at that point. These discharges are then interpolated to the borders of the control volume around the u-point. The difference Δa in grid orientation between the incoming cell and the u-point is computed and used to compute the component of the incoming velocity in the local u-direction, from the left and right side of the control volume.

(C.9)

The same is done for the top and bottom of the control volume, based on the discharges in v-direction:

(C.10)

Finally, the advective term in the momentum balance is given in (C.11).

(C.11)

Time integration scheme

The time integration of the mass and momentum balance equations is combined in an explicit leap-frog scheme, as depicted in Figure C.3. The velocities (in the '-' points) are updated using the momentum balance. The water levels are updated using the mass balance. The water level gradients influence the momentum balance and the velocities and derived discharges affect the mass balance. Because of the leap-frog scheme these influences are always computed at the half time step level, which makes the scheme second order accurate.

Figure 5.3			Leap-frog time integration scheme

Using this straightforward finite volume approach, complicated transformations of the equations are avoided and the solution scheme remains transparent. It is also completely compatible with the original rectilinear implementation and is even slightly more efficient.

Groundwater flow

In order to solve (C.12), the spatial and temporal domain of the groundwater system is split into the same spatial grid and time steps as the XBeach surface water model it is coupled to. At each time step in the numerical model, the depth average groundwater head is calculated in the center of the groundwater cells, and the fluxes (specific discharge, submarine exchange, infiltration and exfiltration) are calculated on the cell interfaces

At the start of the time step, every cell is evaluated whether the groundwater and surface water are connected:

(C.12)

In (C.12) ε is a numerical smoothing constant used to deal with numerical round off errors near the bed (equal to the keyword dwetlayer in the case of hydrostatic groundwater flow, and to the parameter eps in the case of non-hydrostatic groundwater flow), and i and j represent cross-shore and longshore coordinates in the numerical solution grid, respectively. Infiltration is calculated in cells where the groundwater and surface water are not connected and there exists surface water. As shown in (2.70) the infiltration rate is a function of the thickness of the wetting front, which is zero at the start of infiltration, and increases as a function of the infiltration rate. The equations for the infiltration rate and the thickness of the wetting front are approximated by first-order schemes, in which the wetting front is updated using a backward-Euler scheme, which ensures numerical stability:

(C.13)

In (C.13) the pressure term plpf  is the surface water pressure at the bed, which in the case of non-hydrostatic surface water flow is high-pass filtered at 4/Trep, the superscript n corresponds to the time step number and Δt is the size of the time step. The infiltration rate in the coupled relationship can be solved through substitution:

(C.14)

At the end of infiltration, i.e. when the groundwater and surface water become connected or there is no surface water left, the wetting front thickness is reset to zero. If the infiltration rate exceeds the Reynolds number for the start of turbulence, the local hydraulic conductivity is updated using the local Reynolds number:

(C.15)

XBeach iterates until a minimum threshold difference between iterations is found for Equation (C.14) and (C.15). Infiltration in one time step is limited to the amount of surface water available in the cell and to the amount of water required to raise the groundwater level to the level of the bed:

(C.16)

If during infiltration the groundwater level reaches the bed level, the fraction of the time step required to do so is estimated [see (C.17)] and the remaining fraction is used in the submarine exchange.

(C.17)

Exfiltration is calculated in cells where the groundwater and surface water are not connected and the groundwater level exceeds the bed level:

(C.18)

Horizontal infiltration and exfiltration (keyword: gwhorinfil = 1) is computed across the numerical vertical interface between the groundwater domain and the surface water domain in adjoining cells. The direction of exchange is determined by the head gradient, and the bed level slope direction:

(C.19)

After infiltration and exfiltration have been calculated, the groundwater level and surface water level are updated:

(C.20)

Whether the surface water and groundwater are connected or unconnected

The cell height at the center of the groundwater cells (ΔzH,i,j) is calculated from the groundwater level and the bottom of the aquifer in the center of the cell, whereas the cell heights at the horizontal cell interfaces are calculated using an upwind procedure:

(C.21)

In (C.21) zaquifer is the level of the bottom of the aquifer. As described in Section 3.3.6, the head applied on the top boundary of the groundwater domain (Hbc) depends on whether the groundwater and surface water are connected or unconnected:

(C.22)

In (C.22) the parameter κr is the relative numerical ‘connectedness’ of the groundwater and surface water head, determined by linear interpolation across the numerical smoothing constant ε.

In the case of hydrostatic groundwater flow, the groundwater head in each cell is set equal to the head applied on the top boundary of the groundwater domain (Hbc) and the horizontal groundwater flux is computed from the groundwater head gradient:

(C.23)

In (C.23) the superscripts x and y refer to the components of the variable in the cross-shore and longshore direction, respectively, and the subscripts u and v refer to variables approximated at the horizontal cell interfaces in the cross-shore and longshore direction, respectively.

In the case of non-hydrostatic groundwater flow, the horizontal specific discharge on each cell interface can be found through an approximation of the non-hydrostatic groundwater head gradient:

(C.24)

In  (C.24) the subscript H refers to variables approximated at the cell centers. The hydraulic conductivity may be different at each cell interface and is therefore computed at every interface where every K is calculated separately.

Continuity in the groundwater cell is found following:

(C.25)

In (C.25) the variable qz refers to the vertical groundwater discharge (e.g., submarine exchange if connected to the surface water, or groundwater level fluctuations if the groundwater is not connected to the surface water).

In the case of hydrostatic groundwater flow, the variable qz can be solved through the known variables qx and qy. However, in the case of non-hydrostatic groundwater flow, all variables in Equation (C.25) contain an unknown value for the groundwater pressure head, described in terms of a known head at the surface of the groundwater (Hbc) and the unknown curvature of the vertical groundwater head function (β). Since water is incompressible, the groundwater pressure must be solved for all cells simultaneously using matrix algebra:

(C.26)

In (C.26) A is a matrix containing coefficients for the horizontal and vertical specific discharge, x is a vector containing the unknown groundwater head curvature, and b contains the known forcing terms. For a one dimensional cross-shore case, A is reduced to a tridiagonal matrix. The vector of known forcing consists of the numerical gradients in the contribution of the head applied on the top boundary of the groundwater domain to the horizontal specific discharge.

In the one dimensional case, the solution to the tridiagonal matrix A can be computed using the efficient Thomas algorithm (Thomas 1949). In the two dimensional case, matrix A contains two additional diagonals that are not placed along the main diagonal, and vector b contains additional forcing terms from the alongshore contribution. The solution to the two dimensional case requires a more complex and less computationally efficient matrix solver. In this case the Strongly Implicit Procedure (Stone 1968) is used in a manner similar to Zijlema et al. (2011).

Since in both hydrostatic and non-hydrostatic groundwater flow some local velocities may exceed the critical Reynolds number for the start of turbulence (Recrit), the turbulent hydraulic conductivity (K) is updated using the local Reynolds number. The solution to (C.15) and the update of the turbulent hydraulic conductivity are iterated until a minimum threshold difference between iterations is found. Note that this approach is only used is the turbulent groundwater model is selected (keyword: gwscheme = turbulent).

The (iterated) solution for the specific vertical discharge is used to update the groundwater level and surface water level:

(C.27)

If the groundwater and surface water are connected, and the submarine exchange from the surface water to the groundwater estimated in (C.27) is greater than the amount of surface water available in the cell, continuity is enforced by lowering the groundwater level to compensate for the lack of permeating water:

(C.28)

Sediment transport

The advection-diffusion equation for suspended sediment is the basis for the sediment transport computations in XBeach. The partial differential equation to solve is:

(C.29)

Here C is the depth-averaged concentration, Ceq the equilibrium concentration, Ts a typical timescale proportional to water depth divided by fall velocity. As is often done to increase robustness, we treat the erosion term explicitly but taking an implicit scheme for the sedimentation term:

(C.30)

This can be rewritten as:

(C.31)

The sediment transport gradient is discretized in a similar way as the mass balance:

(C.32)

The sediment transports in the u- points contain an advective term, a diffusive term and a bed slope term:

(C.33)

Here urep,s is a representative velocity for suspended transport, which contains contributions due to return flow, wave skewness and wave asymmetry; Dc is a horizontal diffusion coefficient and fslope a coefficient for the bed slope. In discretized form the expression for the suspended transport in the u-point is:

(C.34)

The concentrations in the u-points are computed with a -method, where  means a fully upwind approximation, and  a central scheme. In practice, we mostly use the upwind approximation for its robustness.

(C.35)

The erosion and deposition terms, which may also be used in the bed updating, are finally computed from:

(C.36)

The evaluation of the bed load transport takes place in the same way as in the previous versions of XBeach, except for the fact that the directions are taken in local grid direction, and will not be repeated here.

Bottom updating schemes

Two alternative formulations are available for the bed updating: one where the bottom changes are computed based on the gradients of suspended and bed load transport, and one where the changes due to suspended transport are accounted for through the erosion and deposition terms [see (C.37)].

(C.37)

In both cases MF is the morphological factor used to accelerate morphological changes. In the first case, the sediment in the bottom is conserved in all cases, but changes in the amount of sediment in the water are not considered; one can also say that the sediment in suspension is added to the bottom sediment. In the second case, the storage of sediment in the water is accounted for, but will be distorted in cases of high MF. Since under most circumstances the real effect of the storage in the water phase is small we prefer the first formulation which guarantees mass conservation in the bottom. Both formulations are calculated using an explicit scheme, see also (C.38).

(C.38)

Avalanching

XBeach implements avalanching as described in Section 2.8.2. It first calculates bed level change due to avalanching in the cross-shore dimension. It then calculates the slopes in alongshore direction and bottom change due to avalanching in this direction. To avoid disrupted sediment balance XBeach does not calculate bottom change due to avalanching at the boundary grid cells. Consequently XBeach cannot calculate avalanching at the lateral boundary.

Bed composition

The bed is discretized into layers with mass M(i,j) in which i refers to the layer number and j to the sediment class. The mass fraction per sediment class p, layer thickness Δ and bed level zb are defined by:

(C.39)

with porosity np and sediment density ρ. The level z0 is the lowest point of the array of bed layers.

Due to bed load transport, sediment is exchanged between the top layer and the four horizontally neighboring top layers. Exchange with the water column and the top layer is due to erosion rate E and deposition rate D. A mixed Eulerian/Lagrangian framework is proposed. Within the set of layers, one layer is defined as the variable layer. This is the only layer that has a variable total mass. All other layers have a constant total mass, which implies for a constant porosity a constant thickness. Above the variable layer, the layers move with the bed level (Lagrangian): upwards in case of aggradation and downwards in case of degradation. This vertical movement gives an advective flux with advection velocity equal to the bed level change A=dz/dt. The variable layer is the transition to the lower layers, which are passive. The number of layers below the variable layer has thus no influence on the computation time. Note that diffusive processes within the bed are not considered yet. These could lead to fluxes between the layers below the variable layer.

The mass balance for the top layer can now be defined by:

(C.40)

in which Sb is the bed-load transport (e.g. Meyer-Peter-Muller), based on the sediment properties of the specific class. The dimensions of the grid cell are defined by dx and dy. The subscripts W, E, S and N refer to West, East, South and North indicating the four vertical faces of the bed cell. The horizontal faces are indicated with bot for the bottom of the cell and ceil for the ceiling of the cell.  As the fraction p is not defined at the faces but in the cell centers, the upstream fraction is required. For the bed load fluxes, the velocity direction used. For the vertical advection term, the upstream value is based on the bed level variation: in case of aggradation the value in the top layer is used and in case of degradation the value of the second layer is used. If the top layer is the variable layer, there is no advective flux: δ=0 otherwise δ=1.

The mass balance for the layers in between the top layer and the variable layer is:

(C.41)

and for the variable layer, it reads:

(C.42)

In order to avoid a variable layer which is too thin a or too thick, the variable layer is merged or split. If the thickness is smaller than the critical value Δmerge, the variable layer is merged with the lower layer. To keep the same number of cells, a cell is added at the bottom of the array, implying that z0 = z0 – Δ. Similarly, the variable is split into two layers if the critical value Δsplit is exceeded. Then, the array is shifted upwards: z0 = z0 + Δ.

As the bed level update is explicit, the time step is limited. A conservative estimate can be made by assuming that no more mass can be eroded than available in the top layer:

(C.43)

The transport rate depends on the direction of the transport. The transport rates and erosion rates should be based on the formulation for the smallest fraction: j=1. Note that the fraction p falls out. This time step restriction is less severe than the one for shallow water flow. Only in case of very top layers and/or the use of a morphological factor, this time step restriction might be relevant.

Boundary conditions

At the start of the XBeach simulation, XBeach checks whether non-stationary varying wave boundary conditions are to be used. If this is the case, it next checks whether the wave spectrum of the wave boundary conditions is to change over time, or remain constant. If the wave spectrum is to remain constant, XBeach will only read from one input file to generate wave boundary conditions. If the wave spectrum is to vary in time, XBeach reads from multiple files.

Whether or not the wave spectrum of the boundary conditions changes over time, the XBeach module requires a record length during which the current wave spectral parameters are applied. For the duration of the record length, boundary conditions are calculated at every boundary condition file time step. These time steps are not required to be the same as the time steps in the XBeach main program; XBeach will interpolate where necessary. The boundary condition time steps should therefore only be small enough to accurately describe the incoming bound long waves. The statistical data for the generation of the wave boundary conditions is read from user-specified files. The XBeach module tapers the beginning and end of the boundary condition file. This is done to ensure smooth transitions from one boundary condition file to the next.

The combination of a large record length and a small time step lead to large demands on the system memory. If the memory requirement is too large XBeach will shut down. The user must choose to either enlarge the boundary condition time step, or to reduce the record length. In case of the latter, several boundary condition files can be generated and read sequentially. It is unwise however to reduce the record length too much, as then the transitions between the boundary condition files may affect the model results.

Every time the XBeach wave boundary condition module is run, it outputs data to the local directory. Metadata about the wave boundary conditions are stored in list files: ebcflist.bcf and qbcflist.bcf. The main XBeach program uses the list files to know how and when to read and generate boundary condition files. The actual incoming short-wave energy and long-wave mass flux data is stored in other files. These files have E_ and q_ prefixes. The main XBeach program uses these files for the actual forcing along the offshore edge.

Non-hydrostatic

Global continuity equation

The global continuity equation, which describes the relation between the free surface and the depth averaged discharge, is given by

(C.44)

In which U is the depth averaged velocity vector in x, V in y, H the water depth. A simple semi-discretization of (C.44) using central differences for the space derivative and using the Hansen scheme for the coupling between velocity and free surface results in:

(C.45)

With , and the water depth is defined by a first order accurate upwind interpolation:

(C.46)

The resulting scheme is only first order accurate by virtue of the upwind interpolations and mass conservative. When first order computations are considered accurate enough  is set to . For higher order accuracy the first order prediction is corrected using a limited version of the McCormack scheme. The corrector step reads:

(C.47)

With  and  is given for positive flow as:

(C.48)

Here  denotes the minmod limiter. Similar expression can be constructed for negative flow. The expression for  and  are obtained in a similar manner. Note that the total flux at the cell boundaries thus reads

(C.49)

The predictor-corrector set is second order accurate in regions where the solution is smooth, and reduces locally to first order accuracy near discontinuities. Furthermore, the method remains mass conservative. Note that other flux limiters can be used instead of the minmod limiter. However, as the minmod limiter performed adequately, this has not been investigated. (for an overview of flux limiters see Hirsch, 2007)

Local continuity equation

The depth averaged local continuity equation is given by

(C.50)

This equation is discretized using central differences

(C.51)

Missing grid variables are approximated with upwind interpolation. Because there is no separate time evolution equation for the pressure the local continuity equation will be used to setup a discrete set of poison type equations in which the pressures are the only unknown quantities.

Horizontal Momentum

To obtain a conservative discretization of the momentum equation the approach from Stelling and Duinmeijer (2003) is followed. However, to improve the accuracy of the method the combined space-time discretization of the advection is done using a variant of the MacCormack (1969) is used. This scheme consists of a first order predictor step and a flux limited corrector step. The hydrostatic pressure is integrated using the midpoint rule and central differences, while the source terms and the turbulent stresses are integrated using an explicit Euler time integration. Formally the time integration is therefore first order accurate, but in regions where the turbulent stresses are negligible the scheme is of almost second order accuracy.

The depth averaged horizontal momentum equation for is given by

(C.52)

A first order accurate predictor step in time and space is then given as:

(C.53)

Here Pr represents a discretization of the dynamic pressure; T the effect of (turbulent) viscosity and S includes all other source terms. The discretization of the (turbulent) viscous terms is given by central differences:

(C.54)

Here  and  are obtained from the surrounding points by simple linear interpolation.

Due to the incompressible flow assumption the dynamic pressure does not have a separate time evolution equation, but instead it satisfies an elliptical equation in space. As such its effect cannot be calculated explicitly using values at the previous time level. However to improve the accuracy of the predictor step the effect of the dynamic pressure is included explicitly. To do this first the unknown pressure is decomposed as:

(C.55)

where the difference in pressure is generally small. In the predictor step the effect of the pressure is included explicitly using. In the corrector step the full Poisson equation is then solved for . The pressure term in the predictor step is thus given as:

(C.56)

Here represents the average pressure over the vertical which is approximated with, in which  is the pressure at the bottom. Furthermore  is given as.

Currently (C.54) is formulated with the depth integrated momentum as the primitive variable, and not the depth averaged velocity. To reformulate (C.54) in terms of we use the method by Stelling and Duinmeijer (2003). First note that  and  are approximated as  and . Now using (C.45)  is equivalent to:

(C.57)

Substituting (C.57) into (C.54) the full expressions (including those for ) become:

(C.58)

Where we again use a first order upwind interpolation for and. This is exactly the approximation used by Stelling and Duinmeijer (2003) and is fully momentum conservative.

The predictor step (C.58) is first order accurate in both space and time due to the use of upwind approximations for and Euler explicit time integration for the advective terms, and first order time integration for the source/viscous terms. This level of accuracy is acceptable near shore, where strong non-linearity (wave breaking, flooding and drying) will force the use of small steps in space and time anyway. However, in the region where waves only slowly change (e.g. shoaling/refraction on mild slopes), the first order approximations suffer from significant numerical damping. To improve the accuracy of the numerical model in these regions a corrector step is implemented after the predictor step.

The corrector step is given by:

(C.59)

Or, when formulated in terms of the depth averaged velocity

(C.60)

The values of  are obtained from slope limited expressions. For positive flow these read:

(C.61)

Where  again denotes the minmod limiter. Similar expressions can be constructed for, and.

The predictor-corrector set is second order accurate in regions where the solution is smooth, and reduces to first order accuracy near sharp gradients in the solutions to avoid unwanted oscillations. Furthermore, the method remains momentum conservative.

Vertical momentum equations

The depth averaged vertical momentum equation for is given by:

(C.62)

The vertical momentum equation is discretized in a similar manner to the horizontal momentum equations using the McCormack scheme. In terms of the depth averaged vertical velocity the predictor step is:

(C.63)

The pressures are defined on the cell faces and therefore do not have to be interpolated. Furthermore, we can exactly set the dynamic pressure at the free surface  to zero. The vertical velocities are defined on the cell faces and therefore the depth averaged velocity  needs to be expressed in terms of the bottom and surface velocities. Using a simple central approximation gives

(C.64)

At the bottom the kinematic boundary condition is used for the vertical velocity:

(C.65)

Horizontal interpolation of  and  is done using first order upwind similar to (C.46). The turbulent stresses are again approximated using a central scheme as:

(C.66)

Thus combining (C.64), (C.65) and (C.66) explicit expressions for  and  are obtained. The predicted values are again corrected using a variant of the McCormack scheme and including the pressure difference implicitly gives the corrector step:

(C.67)

Where  and  are obtained using relations similar to (C.61). Note that similar to (C.64) and again the kinematic boundary conditions is substituted for .

The discrete vertical momentum balance of (C.63) and (C.67) looks very different from the relations found in Zijlema and Stelling (2005), Zijlema and Stelling (2008)  and Smit (2008). This is mainly due to the application of the McCormack scheme for the advection. The discretization of the pressure term is numerically fully equivalent to either the Keller box scheme as used in Zijlema and Stelling (2005), Zijlema and Stelling (2008) or the Hermetian relation used in Smit (2008).

Currently, this formulation is only possible when the wave shape formulation of Van Thiel de Vries (2009) is applied, see Section 2.3.4.